|  |  | LLWEB-20205469 |  |  |
| --- | --- | --- | --- | --- |
|  |  | Released |  |  |
|  |  | 1.07 |  |  |
|  | Component Architecture Design Specification Protocol Conversion | August 6, 2026 |  |  |
|  |  | SW&D/SVCS/SVMB/SVMB3 |  |  |
|  |  | Approver | Checker | Author |
|  |  |  |  | Huy Huynh |

Table of Contents

1	Introduction	5

1.1	Scope	5

1.2	References	7

1.3	Abbreviations	8

1.4	Terminology	8

1.5	Environment	8

1.6	Limitations for development	8

2	Summary of Component Architecture	9

2.1	Features	9

2.1.1	Protocol Conversion Initialization Function	10

2.1.2	CAN to Ethernet Conversion Function	11

2.1.3	LIN to Ethernet Conversion Function	18

2.1.4	Port to Ethernet Conversion Function	22

2.1.5	Ethernet to X Conversion Function	28

2.2	API List	31

2.3	Sub-Components	32

2.4	Interfaces	33

2.4.1	R_ProtoConv_Init	33

2.4.2	R_ProtoConv_CantoEthTS	34

2.4.3	R_ProtoConv_CantoEth	35

2.4.4	R_ProtoConv_LintoEth	36

2.4.5	R_ProtoConv_PorttoEth	37

2.4.6	R_ProtoConv_TriggerXtoEthTx	38

2.4.7	R_ProtoConv_MainFunction	39

2.4.8	R_ProtoConv_EthtoX	41

2.4.9	R_ProtoConv_GetSendStatus	43

2.4.10	R_ProtoConv_UpdateSendStatus	44

2.4.11	R_ProtoConv_EnqueueCan	45

2.4.12	R_ProtoConv_EnqueueLin	46

2.4.13	R_ProtoConv_EnqueuePort	47

2.4.14	R_ProtoConv_EnqueueEth	48

2.4.15	R_ProtoConv_DequeueEth	49

2.4.16	R_ProtoConv_TriggerQueueDataXtoEth	51

2.5	Interfaces (User defined)	53

2.5.1	ProtoConv_EthTransmitUserDef	53

2.5.2	ProtoConv_CanTransmitTSUserDef	54

2.5.3	ProtoConv_CanNotifyTSUserDef	55

2.5.4	ProtoConv_CanTransmitUserDef	56

2.5.5	ProtoConv_CanNotifyUserDef	57

2.5.6	ProtoConv_LinTransmitUserDef	58

2.5.7	ProtoConv_LinNotifyUserDef	59

2.5.8	ProtoConv_PortTransmitUserDef	60

2.5.9	ProtoConv_GetTimeUserDef	61

2.5.10	ProtoConv_EnterCriticalUserDef	62

2.5.11	ProtoConv_ExitCriticalUserDef	63

2.6	Error handling	64

2.6.1	Interfaces	64

2.6.2	Interfaces (User defined)	65

2.7	Data Design	65

2.7.1	Data Type Definition	65

2.7.2	Public Definition	77

2.7.3	Error code definition	79

2.7.4	Enumeration	80

3	Sub-Component Design	82

3.1	Initialization Function Design	82

3.1.1	Features	82

3.1.2	Interfaces	82

3.1.3	Error handling	83

3.2	CAN/LIN/Port to Ethernet Conversion Function Design	83

3.2.1	Features	83

3.2.2	Interfaces	83

3.2.3	Error handling	102

3.3	Ethernet to CAN/LIN/Port Conversion Function Design	102

3.3.1	Features	102

3.3.2	Interfaces	103

3.3.3	Error handling	125

3.4	Utility Function	127

3.4.1	Features	127

3.4.2	Interfaces	128

3.4.3	Error handling	136

4	File Structure	137

4.1	Source File List	140

4.2	Header File List	141

5	Configuration and Calibration Design	142

6	State Transition	143

6.1	Summary of State	143

6.2	Send State	144

7	Control Flow Design	145

7.1	Sequence diagram	145

7.1.1	R_ProtoConv_Init	145

7.1.2	R_ProtoConv_CantoEthTS	146

7.1.3	R_ProtoConv_CantoEth	147

7.1.4	R_ProtoConv_LintoEth	148

7.1.5	R_ProtoConv_PorttoEth	149

7.1.6	R_ProtoConv_TriggerXtoEthTx	150

7.1.7	R_ProtoConv_MainFunction	151

7.1.8	R_ProtoConv_EthtoX	152

7.1.9	R_ProtoConv_GetSendStatus	153

7.1.10	R_ProtoConv_UpdateSendStatus	153

7.1.11	R_ProtoConv_EnqueueCan	154

7.1.12	R_ProtoConv_EnqueueLin	154

7.1.13	R_ProtoConv_EnqueuePort	155

7.1.14	R_ProtoConv_EnqueueEth	155

7.1.15	R_ProtoConv_DequeueEth	156

7.1.16	R_ProtoConv_TriggerQueueDataXtoEth	156

7.2	Error Handling Sequence	158

7.2.1	API error on Protocol Conversion	158

8	Data Flow Design	159

8.1	R_ProtoConv_Init	159

8.2	R_ProtoConv_CantoEthTS	161

8.3	R_ProtoConv_CantoEth	162

8.4	R_ProtoConv_LintoEth	163

8.5	R_ProtoConv_PorttoEth	164

8.6	R_ProtoConv_TriggerXtoEthTx	165

8.7	R_ProtoConv_Mainfunction	166

8.8	R_ProtoConv_EthtoX	167

8.9	R_ProtoConv_GetSendStatus	169

8.10	R_ProtoConv_UpdateSendStatus	170

8.11	R_ProtoConv_EnqueueCan	171

8.12	R_ProtoConv_EnqueueLin	172

8.13	R_ProtoConv_EnqueuePort	173

8.14	R_ProtoConv_EnqueueEth	174

8.15	R_ProtoConv_DequeueEth	175

8.16	R_ProtoConv_TriggerQueueDataXtoEth	176

9	Resource Usage	177

9.1	Hardware Function List	177

9.2	Code Size	177

9.3	Stack Size	177

9.4	Execution Time	177

9.5	Memory Map	177

9.5.1	RAM	177

9.5.2	ROM	177

10	Uncovered List	178

# Introduction

## Scope

This document explains the Component Architecture Design of Protocol Conversion.

To enable the design of each unit of Protocol Conversion, the following is defined.

- The functional specifications of each sub-component

- The interface specifications

- etc.

The peripheral modules related to Protocol Conversion are shown below.

Figure 1-1: Scope of targets and related applications in RH850 X2x Gateway ProtoConv Software

Figure 1-2: Scope of targets and related applications in R-Car U5x Gateway ProtoConv Software

## References

| No. | Title (Document Name) | Version |
| --- | --- | --- |
|  | RH850/X2x MCAL Software Architectural Design Specification “RH850_X2x_Driver_AD_Arc.docx” | 15.00 |
|  | Protocol Conversion Requirement Specification https://3ds.eapp.renesas.com/3ddashboard | 1 |
|  | Complex Driver design and integration guideline “AUTOSAR_CP_EXP_CDDDesignAndIntegrationGuideline.pdf” | R22-11 |
|  | IEEE Standard for a Transport Protocol for Time-Sensitive Applications in Bridged Local Area Networks “IEEE Std 1722-2016.pdf” | IEEE Std 1722™-2016 |
|  |  | - |
|  | Network Working Group Request for Comments: 1071 | September 1988 |
|  | Common Requirement Specification https://3ds.eapp.renesas.com/3dspace/common/emxNavigator.jsp?isPopup=false&objectId=1366.1694.57332.26040 | 1 |
|  | Complex Driver design and integration guideline “AUTOSAR_CP_EXP_CDDDesignAndIntegrationGuideline.pdf” | R23-11 |

## Abbreviations

| Abbreviation | Description |
| --- | --- |
| N/A | Not Applicable |
| HW | HardWare |
| SW | SoftWare |
| CAN | Controller Area Network |
| LIN | Local Interconnect Network |
| AVTP | Audio/Video Transport Protocol |
| AVTPDU | Audio/Video Transport Protocol Data Unit |
| ACF | AVTP Control Format |
| MCU | Micro Controller Unit |
| PDU | Protocol Data Unit |
| IP | Internet Protocol |
| IPDUM | Interaction Layer Protocol Data Units Multiplexer |
| UDP | User Datagram Protocol |
| ID | Identifier |
| FIFO | First In – First Out |
| VLAN | Virtual Local Area Network |

## Terminology

| Terminology | Description |
| --- | --- |
| Software | Unit that integrates all software components |
| Software component | Unit that integrates all Software sub-component |
| Software sub-component | Unit consisting of one or more software units |
| Software unit | Function unit capable of stand-alone operation |
| Port | Represents a whole configurable port on an MCU device |
| Mix | Represents the type of message which includes CAN/LIN/Parallel Port messages |

## Environment

| No. | Item | Content |
| --- | --- | --- |
| (1) | Target SoC | X2x |
| (2) | Target SoC Core | RH850 G4KH RH850 G4MH |
| (3) | Compiler | GHS V8.1.4 v2024.1.4 |
| (4) | Evaluation Board | RH850 X2x System Evaluation Board Renesas Electronics |

## Limitations for development

N/A

# Summary of Component Architecture

[iso26-18_p6_7.4.5] a; [iso26-18_p6_7.4.4]

Figure 2-1 shows the software component architecture of Protocol Conversion.

Figure 2-1: Software Component Architecture of Protocol Conversion

## Features

[Gen5GW_ProtoConv_AD_C_01_001]

This software is implemented as the Complex Driver compliant with AUTOSAR.

[Covers: req-8552172-00527976]

[Gen5GW_ProtoConv_AD_C_01_002]

This software supports IEEE1722 AVTP Control Format (ACF) and IPDU-M Packing format.

[Covers: req-8552172-00902551]

[Covers: req-8552172-00903437]

[Covers: req-8552172-00909162]

[Gen5GW_ProtoConv_AD_C_01_003]

This software supports classic and extended formats of CAN/CAN FD ACF message.

[Covers: req-8552172-00903439]

[Covers: req-8552172-00904661]

This software supports classic and extended formats of Abbreviated CAN/CAN FD ACF message.

[Covers: req-8552172-00903440]

This software supports ACF LIN message.

[Covers: req-8552172-00903441]

This software supports ACF Parallel Port (PARALLEL) message.

[Covers: req-8552172-00903442]

[Gen5GW_ProtoConv_AD_C_01_004]

This software shall output VLAN-ID information passed by binding to the payload.

[Covers: req-8552172-00904662]

### Protocol Conversion Initialization Function

The Initialization function performs the initial settings for the CAN/LIN/Parallel Port/Mix to Ethernet Conversion and Ethernet to CAN/LIN/Parallel Port/Mix Conversion functions.

The mix includes CAN/LIN/Parallel Port. The CAN/LIN to Ethernet Routing Table and Ethernet to CAN/LIN Routing Table are configured as routing tables.

For IPDU-M format, CAN/LIN/Port will have the common fields in frames: PDU ID (4 bytes) + PDU length (4 bytes) + Data (64 bytes maximum for CAN, 8 bytes maximum for LIN, 256 bytes maximum for Port).

Initialize the buffers shown below.

Table 2-1: Buffer Configuration

| Buffer name | Description | Size range |
| --- | --- | --- |
| CAN Rx Buffer | CAN receive buffer for 1 CAN frame to 1 Ethernet frame Conversion function | 80 Bytes (Fixed) |
| CAN Tx Buffer | CAN transmit buffer for 1 Ethernet frame to 1 CAN frame Conversion function | 64 Bytes (Fixed) |
| LIN Rx Buffer | LIN receive buffer for 1 LIN frame to 1 Ethernet frame Conversion function | 20 Bytes (Fixed) |
| LIN Tx Buffer | LIN transmit buffer for 1 Ethernet frame to 1 LIN frame Conversion function | 8 Bytes (Fixed) |
| Port Rx Buffer | Port receive buffer for 1 Port frame to 1 Ethernet frame Conversion function | 36 ~ 288 Bytes |
| Port Tx Buffer | Port transmit buffer for 1 Ethernet frame to 1 Port frame Conversion function | 32 ~ 256 Bytes |
| Ethernet Tx Buffer | Ethernet transmit buffer for CAN/LIN/Port to Ethernet Conversion function | 152 ~ 1500 Bytes |
| Ethernet Rx Buffer | Ethernet receive buffer for Ethernet to CAN/LIN/Port Conversion function | 152 ~ 1500 Bytes |
| Mix Rx Buffer | Mix receive buffer for CAN/LIN/Port to Ethernet Conversion function | 80 ~ 1420 Bytes |

※Rationale for the fixed sizes:

CAN Rx Buffer (fixed value, using ACF CAN/CAN/FD format) 80 bytes, including:

- acf_msg_type: 7 bits

- acf_msg_length: 9 bits

- pad (padding length): 2 bits

- mtv (message_timestamp valid): 1 bit

- rtr (remote transmission request): 1 bit

- eff (extended frame format): 1 bit

- brs (bit rate switch): 1 bit

- fdf (CAN Flexible Data-rate (FD) format): 1 bit

- esi (error state indicator): 1 bit

- rsv (reserved): 3 bits

- can_bus_id: 5 bits

- message_timestamp: 8 octets

- rsv (reserved): 3 bits

- can_identifier: 29 bits

- can_msg_payload: 16 quadlets

CAN Tx Buffer (fixed value): 64 bytes

Since CAN Tx Buffer is used for 1 frame only, set 1 frame maximum size value (Data length = 64 bytes). ID and DLC are passed as other valuable, so not included in size.

LIN Rx Buffer (fixed value, using ACF LIN format): 20 bytes, including:

- acf_msg_type: 7 bits

- acf_msg_length: 9 bits

- pad (padding length): 2 bits

- mtv (message_timestamp valid): 1 bit

- lin_bus_id: 5 bits

- lin_identifier: 1 octet

- message_timestamp: 8 octets

- lin_msg_payload: 2 quadlets

LIN Tx Buffer (fixed value): 8 bytes

Since LIN Tx Buffer is used for 1 frame only, set 1 frame maximum size value (Data length = 8 bytes). ID and DLC are passed as other valuable, so not included in size.

※Rationale for the minimum sizes:

Port Rx Buffer (using ACF Parallel Port format): 36 bytes, including:

- acf_msg_type: 7 bits

- acf_msg_length: 9 bits

- reserved: 1 octet

- bit_width: 1 octet

- parallel_msg_payload: 8 quadlets

Port Tx Buffer (using IPDU-M format): 32 bytes

Since Port Tx Buffer is used for 1 frame only, set 1 frame maximum size value (Data length = 32 bytes). ID and DLC are passed as other valuable, so not included in size.

Ethernet Rx/Tx Buffer:

VLAN TAG 4 Byte/IPv6 40 Byte/UDP 8 Byte/EncapSeq 4 Byte/TSCF 24 Byte/IPDU-M CAN max 72 Byte = 152 Byte

Mix Rx Buffer = CAN Rx Buffer = 80 bytes

※Rationale for the maximum sizes:

Port Rx Buffer (using ACF Parallel Port format): including 36 Bytes * 8 ACF Parallel messages = 288 Bytes

Port Tx Buffer (using IPDU-M format): 256 bytes

Since Port Tx Buffer is used for 1 frame only, set 1 frame maximum size value (Data length = 32 bytes * 8 messages = 256 Bytes). ID and DLC are passed as other valuable, so not included in size.

Ethernet Rx/Tx Buffer:

Maximum Ethernet payload size: 1500 Byte

Mix Rx Buffer:

Maximum Ethernet Payload size (1500 Byte) - VLAN TAG (4 Byte) - IPv6 header (40 Byte) - UDP header (8 Byte) - EncapSeq (4 Byte) - TSCF header size (24 Byte) = 1420 Byte

[Gen5GW_ProtoConv_AD_C_02_001]

The user can specify whether CRAM or LRAM should be used as the buffer area. The user statically reserves buffer space and allocates buffer space to the section he/she wants to use.

[Covers: req-8552172-00905312]

[Gen5GW_ProtoConv_AD_C_02_002]

The buffer size can be specified by the user. The user allocates the buffer area statically and passes the first address and size information of the buffer area as arguments to the initialization API. If the specified buffer area is outside the range of CRAM and LRAM area, or if the buffer size is outside the range, an error is returned.

[Covers: req-8552172-00905312]

### CAN to Ethernet Conversion Function

This software receives CAN frames by the user application calling the API of this software. The composition of CAN frames received by this software are shown in Table 2-2 and Table 2-4.

Table 2-2: Structure CAN/CANFD frame in ACF format

| Field | Length | Description |
| --- | --- | --- |
| acf_msg_type | 7 bits | This field describes the type of message contained in the message payload. ACF_CAN: 0x01 (CAN/CAN FD message) ACF_CAN_BRIEF: 0x02 (Abbreviated CAN/CAN FD message) |
| acf_msg_length | 9 bits | It is an unsigned integer that indicates the number of quadlets of data contained in this message, including the quadlet containing the acf_msg_length field and all subsequent quadlets through and including the last quadlet of acf_msg_payload. |
| pad | 2 bits | It indicates the length of padding at the end of the message payload in octets. |
| mtv | 1 bit | It indicate whether the 8-octet message_timestamp field contains valid data. mtv = 1: message_timestamp field contains a valid timestamp mtv= 0: message_timestamp field is not valid. An AVTP Listener ignores the message_timestamp in any message where mtv is set to zero (0). Note: The Abbreviated CAN/CAN FD ACF message does not have message_timestamp field and mtv bit is set to 0. |
| rtr | 1 bit | It contains the CAN message remote transmission request bit. It is set to 0 due to remote frame not being supported. |
| eff | 1 bit | The eff field contains the CAN message extended frame format bit. - If the eff bit is set to zero (0) then the identifier field contains an 11-bit CAN identifier. - If the eff bit is set to one (1) then the identifier field contains a 29-bit CAN identifier. |
| brs | 1 bit | It contains the CAN message bit rate switch bit. It is set to 0 due to not being supported by ProtoConv. |
| fdf | 1 bit | The fdf field contains the CAN message Flexible Data-rate Format bit. - fdf = 0x00: this message is a CAN message and lengths for valid data in the can_msg_payload field are 0 through 8 octets. - fdf = 0x01: this message is a CAN FD message and the lengths for valid data in the can_msg_payload field are 0 through 8, 12, 16, 20, 24, 32, 48, and 64 octets. |
| esi | 1 bit | It contains the CAN message error state indicator bit. It is set to 0 due to not being supported by ProtoConv. |
| rsv | 1 bit | This is reserved field. |
| can_bus_id | 5 bits | The can_bus_id field provides an identifier for the CAN bus on which this message originated. The value of can_bus_id is application specific and is typically specified by the OEM of the vehicle. It has several potential uses, including the identification of the originating CAN bus when using Ethernet as a backbone to carry messages from a multiplicity of CAN buses in a vehicle. If not specified for a particular application, the value of can_bus_id shall be set to zero (0) by the Talker. |
| message_timestamp | 8 octets | The message_timestamp field contains the acquisition time in nanoseconds, if available, of the data contained in the control message. Acquisition time is the time of receipt for the base CAN message of this CAN ACF message or, if the Talker is originating this message, the timestamp is the time of creation. Note: The Abbreviated CAN/CAN FD ACF message does not have message_timestamp field and mtv bit is set to 0. |
| rsv | 3 bits | This is reserved field. |
| can_identifier | 29 bits | CAN message identifiers are either 11 or 29 bits in length. 11-bit CAN message identifiers are stored in bits 21 through 31 of the quadlet holding the can_identifier field and the remaining bits of the field are set to zero (0). Storage of the 11-bit CAN identifier is shown in Figure 2-4: . The letter v indicates a valid identifier bit. |
| can_msg_payload | 0-16 quadlets | The can_msg_payload field is a variable length field that contains from 0 to 16 quadlets of message data. This is sufficient to hold the maximum CAN frame of 64 octets. The overall size of the can_msg_payload field shall always be an integer multiple of quadlets. Table 2-3: Example CAN payloads provides a few examples of CAN payload calculations. |

Figure 2-2: CAN/CAN FD ACF message

Figure 2-3: Abbreviated CAN/CAN FD ACF message

Figure 2-4: Storage of an 11-bit CAN identifier

Table 2-3: Example CAN payloads

| Original CAN message length | Payload length | pad value | acf_msg_length value |
| --- | --- | --- | --- |
| 0 | 0 | 0 | 4 |
| 7 octets | 2 quadlets | 1 | 6 |
| 32 octets | 8 quadlets | 0 | 12 |
| 64 octets | 16 quadlets | 0 | 20 |

Table 2-4: IPDU-M CAN/CANFD frame structure

| Field | Description |
| --- | --- |
| PDU ID | PDU ID (32-bit value). |
| DLC | Data length |
| Data field | CAN Data field |

[Gen5GW_ProtoConv_AD_C_03_001]

Searches the CAN to Ethernet Routing Table by the CAN ID of the received CAN frame to obtain the destination information. The obtained destination information is stored in the header of the Ethernet frame.

[Covers: req-8552172-00905314]

[Gen5GW_ProtoConv_AD_C_03_005]

The received CAN frame is stored in the CAN Rx Buffer if the conversion type is Basic mode or the frame is emergency frame. Otherwise, it is stored in Mix Rx Buffer.

The CAN frame stored in the CAN Rx Buffer or Mix Rx Buffer is stored in the payload section of the Ethernet frame in the Ethernet Tx Buffer when the transmission conditions are met. The transmission conditions vary depending on the conversion method shown in Table 2-6.

[Covers: req-8552172-00905312]

This software handles the process up to passing the Ethernet frame to the user-defined Ethernet send function, and the actual sending process is executed by the user-defined Ethernet send function. For details on user-defined functions, see Section 2.5.1.

The CAN to Ethernet Routing Table is searched by the ID of the CAN frame to obtain the destination (destination MAC address, destination IP address, destination UDP port) of the Ethernet frame to be sent.

[Gen5GW_ProtoConv_AD_C_03_002]

The maximum number of entries in the CAN to Ethernet Routing Table shall be 256.

[Covers: req-8552172-00904669]

[Gen5GW_ProtoConv_AD_C_03_003]

If the CAN ID of the received CAN frame does not exist in the CAN to Ethernet Routing Table, it is discarded without conversion and an error is notified to the user application.

[Covers: req-8552172-00903610]

Because of concerns about packet loss when changing settings during routing, the CAN to Ethernet Routing Table should not be changed dynamically, but only at initialization.

Table 2-5 shows the structure of the Ethernet frame sent by this software. The Ethernet frame together with the destination MAC address, EtherType, and data size information is passed to the user-defined Ethernet send function.

Table 2-5: Ethernet frame structure

| Field | Description |
| --- | --- |
| IP/UDP Header (Optional) | IEEE Std 1722-2016 Annex J “J.3 IP AVTPDU Format” Compliant ・IP/UDP IPv4/IPv6 is supported. |
| AVB Format (Optional) | IEEE Std 1722-2016 “9. AVTP Control Format” Compliant ・Non–Time-Synchronous Control Stream PDU Format (NTSCF) ・Time-Synchronous Control Stream PDU Format (TSCF) |
| Payload | Payload is stored according to the format type. - CAN/CANFD ACF: the format shown in Figure 2-2: CAN/CAN FD ACF message - Abbreviated CAN/CANFD ACF: the format shown in Figure 2-3: Abbreviated CAN/CAN FD ACF message - IPDU-M: the format shown in Table 2-4: IPDU-M CAN/CANFD frame structure If the Ethernet frame is less than the minimum size, pad it with zero. |

[Gen5GW_ProtoConv_AD_C_03_004]

Table 2-6 shows the details of the conversion method. The software operates based on the conversion method selected at initialization.

Table 2-6: Ethernet Conversion Type

| Conversion Type | Description |
| --- | --- |
| Basic Type | Convert one CAN frame to one Ethernet frame sequentially. |
| Buffer Type | Convert one or more CAN frames into one Ethernet frame. A CAN frame is converted to an Ethernet frame when the following transmission conditions are met. - When even receiving one more CAN frame with the minimum size (CAN/CANFD frame with DLC=0) makes Mix Rx Buffer overflow. - When the number of the frames stored in the Mix Rx Buffer exceeds the set threshold value. - When time has elapsed for a conversion cycle. - When the trigger transmit API is called. If CAN frame is the emergency frame, store it in CAN Rx Buffer and send immediately. |

The time elapsed for the conversion cycle is realized by periodically executing the MainFunction API, which performs the transmission process when the time for the conversion cycle has elapsed.

[Covers: req-8552172-00905314]

[Covers: req-8552172-00904667]

Figure 2-5 shows the process of CAN to Ethernet Conversion in Basic type.

Figure 2-6 shows the process of CAN to Ethernet Conversion in Buffer type.

Figure 2-5: Process of CAN to Ethernet Conversion in Basic type (CAN/CANFD)

Figure 2-6: Process of CAN to Ethernet Conversion in Buffer type (CAN/CANFD)

### LIN to Ethernet Conversion Function

This software receives LIN frames by the user application calling the API of this software. The composition of LIN frames received by this software are shown in Figure 2-7 and Table 2-8.

Table 2-7: ACF LIN frame structure

| Field | Length | Description |
| --- | --- | --- |
| acf_msg_type | 7 bits | This field describes the type of message contained in the message payload. ACF_LIN: 0x03 (LIN message) |
| acf_msg_length | 9 bits | It is an unsigned integer that indicates the number of quadlets of data contained in this message, including the quadlet containing the acf_msg_length field and all subsequent quadlets through and including the last quadlet of acf_msg_payload. |
| pad | 2 bits | It indicates the length of padding at the end of the message payload in octets. |
| mtv | 1 bit | It indicate whether the 8-octet message_timestamp field contains valid data. mtv = 0 due to timestamp not being supported in LIN. |
| lin_bus_id | 5 bits | The lin_bus_id field provides an identifier for the LIN bus on which this message originated. The value of lin_bus_id is application specific and is typically specified by the OEM of the vehicle. It has several potential uses, including the identification of the originating LIN bus when using Ethernet as a backbone to carry messages from a multiplicity of LIN buses in a vehicle. If not specified for a particular application, the value of lin_bus_id shall be set to zero (0) by the Talker. |
| lin_identifier | 1 octet | It contains the LIN message identifier. |
| message_timestamp | 8 octets | This field is set to 0 due to timestamp not being supported in LIN. |
| lin_msg_payload | 0-2 quadlets | The lin_msg_payload field is a variable length field that contains from 0 to 8 octets of valid message data. |

Figure 2-7: LIN ACF message

Table 2-8: IPDU-M LIN frame structure

| Field | Description |
| --- | --- |
| PDU ID | PDU ID (32-bit value). |
| DLC | Data length |
| Data field | LIN Data field |

[Gen5GW_ProtoConv_AD_C_03_006]

Searches the LIN to Ethernet Routing Table by the LIN ID of the received LIN frame to obtain the destination information. The obtained destination information is stored in the header of the Ethernet frame.

[Covers: req-8552172-00905315]

[Gen5GW_ProtoConv_AD_C_03_010]

The received LIN frame is stored in the LIN Rx Buffer if the conversion type is Basic mode or the frame is emergency frame. Otherwise, it is stored in Mix Rx Buffer.

The LIN frame stored in the LIN Rx Buffer or Mix Rx Buffer is stored in the payload section of the Ethernet frame in the Ethernet Tx Buffer when the transmission conditions are met. The transmission conditions vary depending on the conversion method shown in Table 2-10.

[Covers: req-8552172-00905312]

This software handles the process up to passing the Ethernet frame to the user-defined Ethernet send function, and the actual sending process is executed by the user-defined Ethernet send function. For details on user-defined functions, see Section 2.5.1.

The LIN to Ethernet Routing Table is searched by the ID of the LIN frame to obtain the destination (destination MAC address, destination IP address, destination UDP port) of the Ethernet frame to be sent.

[Gen5GW_ProtoConv_AD_C_03_007]

The maximum number of entries in the LIN to Ethernet Routing Table shall be 256.

[Covers: req-8552172-00904670]

[Gen5GW_ProtoConv_AD_C_03_008]

If the LIN ID of the received LIN frame does not exist in the LIN to Ethernet Routing Table, it is discarded without conversion and an error is notified to the user application.

[Covers: req-8552172-00903611]

Because of concerns about packet loss when changing settings during routing, the LIN to Ethernet Routing Table should not be changed dynamically, but only at initialization.

Table 2-9 shows the structure of the Ethernet frame sent by this software. The Ethernet frame together with the destination MAC address, EtherType, and data size information is passed to the user-defined Ethernet send function.

Table 2-9: Ethernet frame structure

| Field | Description |
| --- | --- |
| IP/UDP Header (Optional) | IEEE Std 1722-2016 Annex J “J.3 IP AVTPDU Format” Compliant ・IP/UDP IPv4/IPv6 is supported. |
| AVB Format (Optional) | IEEE Std 1722-2016 “9. AVTP Control Format” Compliant ・Non–Time-Synchronous Control Stream PDU Format (NTSCF) ・Time-Synchronous Control Stream PDU Format (TSCF) |
| Payload | Payload is stored according to the format type. - LIN ACF: the format shown in Figure 2-7: LIN ACF message - IPDU-M: the format shown in Table 2-8: IPDU-M LIN frame structure If the Ethernet frame is less than the minimum size, pad it with zero. |

[Gen5GW_ProtoConv_AD_C_03_009]

Table 2-10 shows the details of the conversion method. The software operates based on the conversion method selected at initialization.

Table 2-10: Ethernet Conversion Type

| Conversion Type | Description |
| --- | --- |
| Basic Type | Convert one LIN frame to one Ethernet frame sequentially. |
| Buffer Type | Convert one or more LIN frames into one Ethernet frame. A LIN frame is converted to an Ethernet frame when the following transmission conditions are met. - When even receiving one more LIN frame with the minimum size (DLC=0) makes Mix Rx Buffer overflow. - When the number of the frames stored in the Mix Rx Buffer exceeds the set threshold value - When time has elapsed for a conversion cycle - When the trigger transmit API is called. If LIN frame is the emergency frame, store it in LIN Rx Buffer and send immediately. |

The time elapsed for the conversion cycle is realized by periodically executing the MainFunction API, which performs the transmission process when the time for the conversion cycle has elapsed.

[Covers: req-8552172-00905315]

[Covers: req-8552172-00904667]

Figure 2-8 shows the process of LIN to Ethernet Conversion in Basic type.

Figure 2-9 shows the process of LIN to Ethernet Conversion in Buffer type.

Figure 2-8: Process of LIN to Ethernet Conversion in Basic type

Figure 2-9: Process of LIN to Ethernet Conversion in Buffer type

### Port to Ethernet Conversion Function

This software receives Port frames by the user application calling the API of this software. The composition of Port frames received by this software are shown in Table 2-11 and Table 2-12.

Table 2-11: Parallel Port frame structure

| Field | Length | Description |
| --- | --- | --- |
| acf_msg_type | 7 bits | This field describes the type of message contained in the message payload. ACF_PARALLEL: 0x07 (Parallel port message) ProtoConv does only support Parallel port message, does not support Serial port message. |
| acf_msg_length | 9 bits | It is an unsigned integer that indicates the number of quadlets of data contained in this message, including the quadlet containing the acf_msg_length field and all subsequent quadlets through and including the last quadlet of acf_msg_payload. |
| reserved | 1 octet | This is reserved field. It is set to (0). |
| bit_width | 1 octet | It contains the number of valid bits in the parallel_msg_payload field starting at bit 0. When the bit_width field is zero (0), the number of valid bits in the parallel_msg_payload field is 256. parallel_msg_payload is zero-padded to the nearest quadlet boundary. |
| parallel_msg_payload | 0 to 8 quadlets | It contains the payload of the parallel message. Each bit position in the parallel_msg_payload carries the value of the corresponding pin on the parallel interface. Figure 2-11: Example Parallel message with bit_width = 67 shows an example payload when bit_width = 67. The letter v in the figure indicates that a given bit position carries valid data for the corresponding pin on parallel interface. In this case acf_msg_length has a value of four (4). |

Figure 2-10: Parallel Port ACF message

Figure 2-11: Example Parallel message with bit_width = 67

Table 2-12: IPDU-M Port frame structure

| Field | Description |
| --- | --- |
| PDU ID | PDU ID (32-bit value). |
| DLC | Data length |
| Data field | Port Data field |

[Gen5GW_ProtoConv_AD_C_03_011]

Use the defined index to obtain the destination information. The obtained destination information is stored in the header of the Ethernet frame.

[Covers: req-8552172-00905316]

[Gen5GW_ProtoConv_AD_C_03_012]

The received Port frame is stored in the Port Rx Buffer.

The Port frame stored in the Port Rx Buffer is stored in the payload section of the Ethernet frame in the Ethernet Tx Buffer when the transmission conditions are met. The transmission conditions vary depending on the conversion method shown in Table 2-14.

[Covers: req-8552172-00905312]

This software handles the process up to passing the Ethernet frame to the user-defined Ethernet send function, and the actual sending process is executed by the user-defined Ethernet send function. For details on user-defined functions, see Section 2.5.1.

Table 2-13: Ethernet frame structure

| Field | Description |
| --- | --- |
| IP/UDP Header (Optional) | IEEE Std 1722-2016 Annex J “J.3 IP AVTPDU Format” Compliant ・IP/UDP IPv4/IPv6 is supported. |
| AVB Format (Optional) | IEEE Std 1722-2016 “9. AVTP Control Format” Compliant ・Non–Time-Synchronous Control Stream PDU Format (NTSCF) ・Time-Synchronous Control Stream PDU Format (TSCF) |
| Payload | Payload is stored according to the format type. - Parallel Port ACF: the format shown in Figure 2-10: Parallel Port ACF message - IPDU-M: the format shown in Table 2-12: IPDU-M Port frame structure If the Ethernet frame is less than the minimum size, pad it with zero. |

[Gen5GW_ProtoConv_AD_C_03_013]

Table 2-14 shows the details of the conversion method. The software operates based on the conversion method selected at initialization.

Table 2-14: Ethernet Conversion Type

| Conversion Type | Description |
| --- | --- |
| Basic Type | Convert one sequence of IO data of multiple Port to one Ethernet frame sequentially. |
| Buffer Type | Convert one or more sequences of IO data of multiple Port to one Ethernet frame. A sequence of IO data of multiple Port is converted to an Ethernet frame when the following transmission conditions are met. - When even receiving one more sequence of IO data of multiple Port with the minimum size (PDU frame with DLC=0) makes Mix Rx Buffer. - When the number of the frames stored in the Mix Rx Buffer exceeds the set threshold value - When time has elapsed for a conversion cycle - When the trigger transmit API is called. If PORT frame is the emergency frame, store in PORT Rx Buffer and send immediately. |

The time elapsed for the conversion cycle is realized by periodically executing the MainFunction API, which performs the transmission process when the time for the conversion cycle has elapsed.

[Covers: req-8552172-00905316]

[Covers: req-8552172-00904667]

[Gen5GW_ProtoConv_AD_C_03_014]

Protocol Conversion Library supports 80 Port IO data, 4-bit data for each Port IO channel.

Supports up to 256 bytes for future expansion.

Since up to 32 Bytes (256 bits) can be included in one ACF message, the Protocol Conversion Library divides the data into multiple ACF messages and stores them according to the data size as in Table 2-15 and Figure 2-12.

[Covers: req-8552172-00904660]

Table 2-15: ACF Message numbers according to Port data size

| Data Length | Num of ACF Message |
| --- | --- |
| 1 ~ 32 | 1 |
| 33 ~ 64 | 2 |
| 65 ~ 96 | 3 |
| 97 ~ 128 | 4 |
| 129 ~ 160 | 5 |
| 161 ~192 | 6 |
| 193 ~ 224 | 7 |
| 225 ~ 256 | 8 |

Example:

Figure 2-12: ACF Parallel Port message with full of 80 Port IO Data

Figure 2-13 shows the process of Port to Ethernet Conversion in Basic type.

Figure 2-14 shows the process of Port to Ethernet Conversion in Buffer type.

Figure 2-13: Process of Port to Ethernet Conversion in Basic type

Figure 2-14: Process of Port to Ethernet Conversion in Buffer type

### Ethernet to X Conversion Function

This software receives Ethernet frames when a user application calls the software's API.

#### Ethernet to CAN Conversion Function

[Gen5GW_ProtoConv_AD_C_05_001]

Search the Ethernet to CAN Routing Table by the CAN ID of the CAN frame packed in the received Ethernet frame and obtain the destination information. The obtained destination information is used as a parameter of the user-defined CAN transmission function.

Search the Ethernet to CAN Routing Table by the CAN ID of each of the multiple CAN frames packed in the Ethernet frame and execute CAN transmission.

This software handles the processing up to passing the CAN frame to the user-defined CAN send function, and the actual sending process is performed by the user-defined CAN send function.

The Ethernet to CAN Routing Table is searched by CAN frame ID to obtain the Destination Bus ID and Additional Information of the RS-CANFD to be transmitted.

[Covers: req-8552172-00905318]

[Covers: req-8552172-00903613]

[Gen5GW_ProtoConv_AD_C_05_002]

The maximum number of entries in the Ethernet to CAN Routing Table shall be 256.

[Covers: req-8552172-00904671]

If the CAN ID of the transmitted CAN frame does not exist in the Ethernet to CAN Routing Table, the CAN frame is notified by a user-defined notification function.

Because of concerns about packet loss when changing settings during routing, the Ethernet to CAN Routing Table should not be changed dynamically, but only at initialization.

The composition of Ethernet frames received by this software shall be the same as the frame composition of CAN to Ethernet conversion (Table 2-5).

[Gen5GW_ProtoConv_AD_C_05_003]

If a CAN frame transmission error occurs, the channel on which the error occurred is recorded.

Transmission of subsequent CAN frames continues even if an error occurs.

The contents to be stored when an error occurs are as follows

Error status is notified in 32 bits. See Figure 2-15 for the relationship between each bit and channel.

Figure 2-15: Related bit and Channel about CAN message send error

[Covers: req-8552172-00903613]

[Gen5GW_ProtoConv_AD_C_05_004]

CAN packet transmission enable/disable setting for each destination bus ID when transmitting a CAN packet.

If the corresponding bit of the transmission enable/disable setting is 0 when sending a CAN packet, the packet transmission is not performed.

[Covers: req-8552172-00905218]

[Covers: req-8552172-00905219]

#### Ethernet to LIN Conversion Function

This software receives Ethernet frames when a user application calls the software's API.

[Gen5GW_ProtoConv_AD_C_05_005]

Search the Ethernet to LIN Routing Table by the LIN ID of the LIN frame packed in the received Ethernet frame and obtain the destination information. The obtained destination information is used as a parameter of the user-defined LIN transmission function.

Search the Ethernet to LIN Routing Table by the LIN ID of each of the multiple LIN frames packed in the Ethernet frame and execute LIN transmission.

This software handles the processing up to passing the LIN frame to the user-defined LIN send function, and the actual sending process is performed by the user-defined LIN send function.

The Ethernet to LIN Routing Table is searched by LIN frame ID to obtain the Destination Bus ID and Additional Information to be transmitted.

[Covers: req-8552172-00905319]

[Covers: req-8552172-00903613]

[Gen5GW_ProtoConv_AD_C_05_006]

The maximum number of entries in the Ethernet to LIN Routing Table shall be 256.

[Covers: req-8552172-00904678]

If the LIN ID of the transmitted LIN frame does not exist in the Ethernet to LIN Routing Table, the LIN frame is notified by a user-defined notification function.

Because of concerns about packet loss when changing settings during routing, the Ethernet to LIN Routing Table should not be changed dynamically, but only at initialization.

The composition of Ethernet frames received by this software shall be the same as the frame composition of LIN to Ethernet conversion (Table 2-9).

#### Ethernet to Port Conversion Function

This software receives Ethernet frames when a user application calls the software's API.

[Gen5GW_ProtoConv_AD_C_05_007]

This software handles the processing up to passing the Port frame to the user-defined Port send function, and the actual sending process is performed by the user-defined Port send function.

[Covers: req-8552172-00905320]

[Covers: req-8552172-00903613]

#### Ethernet to CAN/LIN/Port Filtering Function

[Gen5GW_ProtoConv_AD_C_05_008]

If the MAC address is not registered on the Ethernet to CAN/LIN/Port Filtering Table, no filtering is performed.

If the source MAC address does not exist in the Ethernet to CAN/LIN/Port Filtering Table, the CAN/LIN/Port frame is discarded.

[Covers: req-8552172-00904663]

[Gen5GW_ProtoConv_AD_C_05_009]

If no IP address is registered on the Ethernet to CAN/LIN/Port Filtering Table, no filtering is performed.

If the IP address of the outgoing CAN frame does not exist in the Ethernet to CAN/LIN/Port Filtering Table, the CAN/LIN/Port frame is discarded.

[Covers: req-8552172-00904665]

[Gen5GW_ProtoConv_AD_C_05_010]

If no VLAN ID is registered on the Ethernet to CAN/LIN/Port Filtering Table, no filtering is performed.

If the VLAN ID of the outgoing CAN frame does not exist in the Ethernet to CAN/LIN/Port Filtering Table, the CAN/LIN/Port frame is discarded.

[Covers: req-8552172-00904664]

[Gen5GW_ProtoConv_AD_C_05_011]

If the Stream ID is not registered on the Ethernet to CAN/LIN/Port Filtering Table, no filtering is performed.

If the Stream ID of the transmitted CAN frame does not exist in the Ethernet to CAN/LIN/Port Filtering Table, the CAN/LIN/Port frame is discarded.

[Covers: req-8552172-00904666]

[Covers: req-8552172-00905219]

## API List

This section describes the interfaces between the software components in Table 2-16 and Table 2-17.

Table 2-16: API List

| No. | Interface Name | ASIL level | Description |
| --- | --- | --- | --- |
|  | R_ProtoConv_Init | ASIL D | Initialize Protocol Conversion. |
|  | R_ProtoConv_CantoEthTS | ASIL D | Convert CAN/CAN FD frame with TimeStamp to Ethernet frame. |
|  | R_ProtoConv_CantoEth | ASIL D | Convert CAN/CAN FD frame to Ethernet frame. |
|  | R_ProtoConv_LintoEth | ASIL D | Convert LIN frame to Ethernet frame. |
|  | R_ProtoConv_PorttoEth | ASIL D | Convert Port frame to Ethernet frame. |
|  | R_ProtoConv_TriggerXtoEthTx | ASIL D | Trigger CAN/LIN/Port to Ethernet Transmission |
|  | R_ProtoConv_MainFunction | ASIL D | Main Function for polling. |
|  | R_ProtoConv_EthtoX | ASIL D | Convert Ethernet frame to CAN/LIN/Parallel Port frames. |
|  | R_ProtoConv_GetSendStatus | ASIL D | Get packet setting acquisition status. |
|  | R_ProtoConv_UpdateSendStatus | ASIL D | Update packet setting acquisition status. |
|  | R_ProtoConv_EnqueueCan | ASIL D | Enqueue CAN message to the pre-defined memory. |
|  | R_ProtoConv_EnqueueLin | ASIL D | Enqueue LIN message to the pre-defined memory. |
|  | R_ProtoConv_EnqueuePort | ASIL D | Enqueue Port message to the pre-defined memory. |
|  | R_ProtoConv_EnqueueEth | ASIL D | Enqueue Ethernet message to the pre-defined memory. |
|  | R_ProtoConv_DequeueEth | ASIL D | Dequeue Ethernet message to the pre-defined memory. |
|  | R_ProtoConv_TriggerQueueDataXtoEth | ASIL D | Trigger the queued CAN/LIN/Port to Ethernet transmit. |

Table 2-17: API List (User defined)

| No. | Interface Name | ASIL level | Description |
| --- | --- | --- | --- |
|  | ProtoConv_EthTransmitUserDef | ASIL D | The user implements the process of sending Ethernet frames in this function. |
|  | ProtoConv_CanTransmitTSUserDef | ASIL D | The user implements the process of sending CAN/CAN FD frames with TimeStamp in this function. |
|  | ProtoConv_CanNotifyTSUserDef | ASIL D | Notifies that the CAN/CAN FD frames with TimeStamp are subject to inter-core communication. Implemented by the user in this function. |
|  | ProtoConv_CanTransmitUserDef | ASIL D | The user implements the process of sending CAN/CAN FD frames in this function. |
|  | ProtoConv_CanNotifyUserDef | ASIL D | Notifies that the CAN/CAN FD frame is subject to inter-core communication. Implemented by the user in this function. |
|  | ProtoConv_LinTransmitUserDef | ASIL D | The user implements the process of sending LIN frames in this function. |
|  | ProtoConv_LinNotifyUserDef | ASIL D | Notifies that the LIN frame is subject to inter-core communication. Implemented by the user in this function. |
|  | ProtoConv_PortTransmitUserDef | ASIL D | The user implements the process of sending Port frames in this function. |
|  | ProtoConv_GetTimeUserDef | ASIL D | The user implements the process of acquiring the time to be used for the timestamp in this function. |
|  | ProtoConv_EnterCriticalUserDef | ASIL D | The user implements the interruption inhibit process in this function. |
|  | ProtoConv_ExitCriticalUserDef | ASIL D | The user implements the interrupted permission process in this function. |

## Sub-Components

[iso26-18_p6_7.4.4]; [iso26-18_p6_7.4.6]; [iso26-18_p6_7.4.8]

Table 2-18: Sub-component List

| No. | Software Sub-component Name | ASIL level | Description |
| --- | --- | --- | --- |
|  | Initialization Function | ASIL D | Initialize Protocol Conversion. |
|  | CAN to Ethernet Conversion Function | ASIL D | Convert CAN frame to Ethernet frame. |
|  | LIN to Ethernet Conversion Function | ASIL D | Convert LIN frame to Ethernet frame. |
|  | Port to Ethernet Conversion Function | ASIL D | Convert Port frame to Ethernet frame. |
|  | Ethernet to X Conversion Function | ASIL D | Convert Ethernet frame to CAN/LIN/Port frames. |
|  | Utility Function | ASIL D | Common functions of Protocol Conversion. |

Table 2-19: Sub-component List (User defined)

| No. | Software Sub-component Name | ASIL level | Description |
| --- | --- | --- | --- |
| (1) | Interface Function | ASIL D | The implementation within each function is defined by the user. |

## Interfaces

### R_ProtoConv_Init

[iso26-18_p6_7.4.5] a; [iso26-18_p6_7.4.6]

[Gen5GW_ProtoConv_AD_C_10_001]

Table 2-20: R_ProtoConv_Init

| Function Name | R_ProtoConv_Init |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| ASIL Level | ASIL D | Status | New |  |  |
| Syntax | Std_ReturnType R_ProtoConv_Init(const R_ProtoConv_CfgType *pConfig) |  |  |  |  |
| Sync/Async | Synchronous |  |  |  |  |
| Reentrancy | Non-Reentrant |  |  |  |  |
| Interrupt State | Runs in the interrupt enable state |  |  |  |  |
| Parameters (In) | const R_ProtoConv_CfgType *pConfig |  | Pointer to initialization config. |  |  |
|  |  |  | Range | Not NULL_PTR |  |
| Parameters (In-Out) | N/A |  | - |  |  |
| Parameters (Out) | N/A |  | - |  |  |
| Return Value | Std_ReturnType |  | E_OK E_NOT_OK |  |  |
| Description | If this API has already been executed, it returns E_NOT_OK, report DET error PROTOCONV_E_ALREADY_INITIALIZED (if PROTOCONV_DEV_ERROR_DETECT == STD_ON) and exits. If the pConfig argument is NULL_PTR, it returns E_NOT_OK, report DET error PROTOCONV_E_PARAM_POINTER (if PROTOCONV_DEV_ERROR_DETECT == STD_ON) and exits. If E_OK is returned, execute ProtoConv_Init() and return its return value. |  |  |  |  |
| Preconditions | Uninitialized state |  |  |  |  |
| Remarks | N/A |  |  |  |  |
| DET/DEM Errors Handled | PROTOCONV_E_PARAM_POINTER |  |  |  | Parameter error (argument is NULL_PTR) |
|  | PROTOCONV_E_ALREADY_INITIALIZED |  |  |  | Invalid call |

[Covers: req-8552172-00903609]

[Covers: req-8552172-00904663]

[Covers: req-8552172-00904664]

[Covers: req-8552172-00904665]

[Covers: req-8552172-00904666]

[Covers: req-8552172-00909476]

[Covers: req-8552172-00909478]

[Covers: req-8552172-00528003]

[Covers: req-8552172-00528002]

[Covers: req-8552172-00528006]

[Covers: req-8552172-00527993]

[Covers: req-8552172-00527980]

### R_ProtoConv_CantoEthTS

[iso26-18_p6_7.4.5] a; [iso26-18_p6_7.4.6]

[Gen5GW_ProtoConv_AD_C_10_002]

Table 2-21: R_ProtoConv_CantoEthTS

| Function Name | R_ProtoConv_CantoEthTS |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| ASIL Level | ASIL D | Status |  | New |  |
| Syntax | Std_ReturnType R_ProtoConv_CantoEthTS(const uint8 ucBusIdx, const R_ProtoConv_CanFrameType *pCanFrame, const R_ProtoConv_TimeStampType *pTimeStamp) |  |  |  |  |
| Sync/Async | Synchronous |  |  |  |  |
| Reentrancy | Non-Reentrant |  |  |  |  |
| Interrupt State | Runs in the interrupt enable state |  |  |  |  |
| Parameters (In) | const uint8 ucBusIdx |  |  | Source Bus ID |  |
|  |  |  |  | Range | 0 - (PROTOCONV_CAN_CH_NUM-1) |
| Parameters (In) | const R_ProtoConv_CanFrameType *pCanFrame |  |  | Address of buffer where CAN frames are stored |  |
|  |  |  |  | Range | Not NULL_PTR |
| Parameters (In) | const R_ProtoConv_TimeStampType *pTimeStamp |  |  | Address of buffer where timestamp is stored |  |
|  |  |  |  | Range | Not NULL_PTR |
| Parameters (In-Out) | N/A |  |  | - |  |
| Parameters (Out) | N/A |  |  | - |  |
| Return Value | Std_ReturnType |  |  | E_OK E_NOT_OK |  |
| Description | If R_ProtoConv_Init() is not executed yet, it returns E_NOT_OK, reports DET error PROTOCONV_E_UNINIT (if PROTOCONV_DEV_ERROR_DETECT == STD_ON) and exits. If the argument pCanFrame or pTimeStamp is NULL_PTR, it returns E_NOT_OK, reports DET error PROTOCONV_E_PARAM_POINTER (if PROTOCONV_DEV_ERROR_DETECT == STD_ON) and exits. If the argument ucBusIdx is out of range, it returns E_NOT_OK, reports DET error PROTOCONV_E_INV_PARAM (if PROTOCONV_DEV_ERROR_DETECT == STD_ON) and exits. R_ProtoConv_CantoEthTS() executes ProtoConv_CantoEthTS() and returns its return value. |  |  |  |  |
| Preconditions | Initialized state |  |  |  |  |
| Remarks | N/A |  |  |  |  |
| DET/DEM Errors Handled | PROTOCONV_E_PARAM_POINTER |  | Parameter error (argument is NULL_PTR) |  |  |
|  | PROTOCONV_E_INV_PARAM |  | Invalid parameter passed to the APIs |  |  |
|  | PROTOCONV_E_UNINIT |  | Process error (API executed in uninitialized state) |  |  |
|  | PROTOCONV_E_MATCH |  | Unmatched Table Error (Routing Table) |  |  |
|  | PROTOCONV_E_TRANS |  | Transmit Error |  |  |

[Covers: req-8552172-00903610]

[Covers: req-8552172-00905227]

[Covers: req-8552172-00904662]

[Covers: req-8552172-00909476]

[Covers: req-8552172-00909478]

[Covers: req-8552172-00528003]

[Covers: req-8552172-00528002]

[Covers: req-8552172-00528006]

[Covers: req-8552172-00527993]

[Covers: req-8552172-00527980]

### R_ProtoConv_CantoEth

[iso26-18_p6_7.4.5] a; [iso26-18_p6_7.4.6]

[Gen5GW_ProtoConv_AD_C_10_003]

Table 2-22: R_ProtoConv_CantoEth

| Function Name | R_ProtoConv_CantoEth |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| ASIL Level | ASIL D | Status |  | New |  |
| Syntax | Std_ReturnType R_ProtoConv_CantoEth(const uint8 ucBusIdx, const R_ProtoConv_CanFrameType *pCanFrame) |  |  |  |  |
| Sync/Async | Synchronous |  |  |  |  |
| Reentrancy | Non-Reentrant |  |  |  |  |
| Interrupt State | Runs in the interrupt enable state |  |  |  |  |
| Parameters (In) | const uint8 ucBusIdx |  |  | Source Bus ID |  |
|  |  |  |  | Range | 0 - (PROTOCONV_CAN_CH_NUM-1) |
| Parameters (In) | const R_ProtoConv_CanFrameType *pCanFrame |  |  | Address of buffer where CAN frames are stored |  |
|  |  |  |  | Range | Not NULL_PTR |
| Parameters (In-Out) | N/A |  |  | - |  |
| Parameters (Out) | N/A |  |  | - |  |
| Return Value | Std_ReturnType |  |  | E_OK E_NOT_OK |  |
| Description | If R_ProtoConv_Init() is not executed yet, it returns E_NOT_OK, reports DET error PROTOCONV_E_UNINIT (if PROTOCONV_DEV_ERROR_DETECT == STD_ON) and exits. If the argument pCanFrame is NULL_PTR, it returns E_NOT_OK, reports DET error PROTOCONV_E_PARAM_POINTER (if PROTOCONV_DEV_ERROR_DETECT == STD_ON) and exits. If the argument ucBusIdx is out of range, it returns E_NOT_OK, reports DET error PROTOCONV_E_INV_PARAM (if PROTOCONV_DEV_ERROR_DETECT == STD_ON) and exits. R_ProtoConv_CantoEth() executes ProtoConv_CantoEth() and returns its return value. |  |  |  |  |
| Preconditions | Initialized state |  |  |  |  |
| Remarks | N/A |  |  |  |  |
| DET/DEM Errors Handled | PROTOCONV_E_PARAM_POINTER |  | Parameter error (argument is NULL_PTR) |  |  |
|  | PROTOCONV_E_INV_PARAM |  | Invalid parameter passed to the APIs |  |  |
|  | PROTOCONV_E_UNINIT |  | Process error (API executed in uninitialized state) |  |  |
|  | PROTOCONV_E_MATCH |  | Unmatched Table Error (Routing Table) |  |  |
|  | PROTOCONV_E_TRANS |  | Transmit Error |  |  |

[Covers: req-8552172-00903610]

[Covers: req-8552172-00905227]

[Covers: req-8552172-00904662]

[Covers: req-8552172-00909476]

[Covers: req-8552172-00909478]

[Covers: req-8552172-00528003]

[Covers: req-8552172-00528002]

[Covers: req-8552172-00528006]

[Covers: req-8552172-00527993]

[Covers: req-8552172-00527980]

### R_ProtoConv_LintoEth

[iso26-18_p6_7.4.5] a; [iso26-18_p6_7.4.6]

[Gen5GW_ProtoConv_AD_C_10_004]

Table 2-23: R_ProtoConv_LintoEth

| Function Name | R_ProtoConv_LintoEth |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| ASIL Level | ASIL D | Status | New |  |  |
| Syntax | Std_ReturnType R_ProtoConv_LintoEth(const uint8 ucBusIdx, const R_ProtoConv_LinFrameType *pLinFrame) |  |  |  |  |
| Sync/Async | Synchronous |  |  |  |  |
| Reentrancy | Non-Reentrant |  |  |  |  |
| Interrupt State | Runs in the interrupt enable state |  |  |  |  |
| Parameters (In) | const uint8 ucBusIdx |  | Source Bus ID |  |  |
|  |  |  | Range |  | 0 – (PROTOCONV_LIN_CH_NUM-1) |
| Parameters (In) | const R_ProtoConv_LinFrameType *pLinFrame |  | Address of buffer where LIN frames are stored |  |  |
|  |  |  | Range |  | Not NULL_PTR |
| Parameters (In-Out) | N/A |  | - |  |  |
| Parameters (Out) | N/A |  | - |  |  |
| Return Value | Std_ReturnType |  | E_OK E_NOT_OK |  |  |
| Description | If R_ProtoConv_Init() is not executed yet, it returns E_NOT_OK, reports DET error PROTOCONV_E_UNINIT (if PROTOCONV_DEV_ERROR_DETECT == STD_ON) and exits. If the argument pLinFrame is NULL_PTR, it returns E_NOT_OK, reports DET error PROTOCONV_E_PARAM_POINTER (if PROTOCONV_DEV_ERROR_DETECT == STD_ON) and exits. If the argument ucBusIdx is out of range, it returns E_NOT_OK, reports DET error PROTOCONV_E_INV_PARAM (if PROTOCONV_DEV_ERROR_DETECT == STD_ON) and exits. R_ProtoConv_LintoEth() executes ProtoConv_LintoEth() and returns its return value. |  |  |  |  |
| Preconditions | Initialized state |  |  |  |  |
| Remarks | N/A |  |  |  |  |
| DET/DEM Errors Handled | PROTOCONV_E_PARAM_POINTER |  |  | Parameter error (argument is NULL_PTR) |  |
|  | PROTOCONV_E_INV_PARAM |  |  | Invalid parameter passed to the APIs |  |
|  | PROTOCONV_E_UNINIT |  |  | Process error (API executed in uninitialized state) |  |
|  | PROTOCONV_E_MATCH |  |  | Unmatched Table Error (Routing Table) |  |
|  | PROTOCONV_E_TRANS |  |  | Transmit Error |  |

[Covers: req-8552172-00903611]

[Covers: req-8552172-00905227]

[Covers: req-8552172-00904662]

[Covers: req-8552172-00909476]

[Covers: req-8552172-00909478]

[Covers: req-8552172-00528003]

[Covers: req-8552172-00528002]

[Covers: req-8552172-00528006]

[Covers: req-8552172-00527993]

[Covers: req-8552172-00527980]

### R_ProtoConv_PorttoEth

[iso26-18_p6_7.4.5] a; [iso26-18_p6_7.4.6]

[Gen5GW_ProtoConv_AD_C_10_005]

Table 2-24: R_ProtoConv_PorttoEth

| Function Name | R_ProtoConv_PorttoEth |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| ASIL Level | ASIL D |  | Status |  | New |
| Syntax | Std_ReturnType R_ProtoConv_PorttoEth(const uint16 usLength, const uint8 *pPortData, const uint32 ulDestIdx, const R_ProtoConv_FrameType enTypeFlags) |  |  |  |  |
| Sync/Async | Synchronous |  |  |  |  |
| Reentrancy | Non-Reentrant |  |  |  |  |
| Interrupt State | Runs in the interrupt enable state |  |  |  |  |
| Parameters (In) | const uint16 usLength | Data Length |  |  |  |
|  |  | Range |  | MIN_PORT_TOTAL_SIZE - MAX_PORT_TOTAL_SIZE |  |
| Parameters (In) | const uint8 *pPortData | Address of buffer where Port Data are stored |  |  |  |
|  |  | Range |  | Not NULL_PTR |  |
| Parameters (In) | const uint32 ulDestIdx | Index of Destination Rooting Table |  |  |  |
|  |  | Range |  | 0 - (XTOETH_DEST_TABLE_ENTRY_NUM-1) |  |
| Parameters (In) | const R_ProtoConv_FrameType enTypeFlags | Frame Type |  |  |  |
|  |  | Range |  | PROTOCONV_NORMAL_FRAME or PROTOCONV_EMERGENCY_FRAME |  |
| Parameters (In-Out) | N/A | - |  |  |  |
| Parameters (Out) | N/A | - |  |  |  |
| Return Value | Std_ReturnType | E_OK E_NOT_OK |  |  |  |
| Description | If R_ProtoConv_Init() is not executed yet, it returns E_NOT_OK, reports DET error PROTOCONV_E_UNINIT (if PROTOCONV_DEV_ERROR_DETECT == STD_ON) and exits. If the argument pPortData is NULL_PTR, it returns E_NOT_OK, reports DET error PROTOCONV_E_PARAM_POINTER (if PROTOCONV_DEV_ERROR_DETECT == STD_ON) and exits. If the argument usLength or ulDestIdx is out of range, it returns E_NOT_OK, reports DET error PROTOCONV_E_INV_PARAM (if PROTOCONV_DEV_ERROR_DETECT == STD_ON) and exits. R_ProtoConv_PorttoEth() executes ProtoConv_PorttoEth() and returns its return value. |  |  |  |  |
| Preconditions | Initialized state |  |  |  |  |
| Remarks | N/A |  |  |  |  |
| DET/DEM Errors Handled | PROTOCONV_E_PARAM_POINTER |  |  | Parameter error (argument is NULL_PTR) |  |
|  | PROTOCONV_E_INV_PARAM |  |  | Invalid parameter passed to the APIs |  |
|  | PROTOCONV_E_UNINIT |  |  | Process error (API executed in uninitialized state) |  |
|  | PROTOCONV_E_MATCH |  |  | Unmatched Table Error (Routing Table) |  |
|  | PROTOCONV_E_TRANS |  |  | Transmit Error |  |

[Covers: req-8552172-00903612]

[Covers: req-8552172-00905227]

[Covers: req-8552172-00904662]

[Covers: req-8552172-00909476]

[Covers: req-8552172-00909478]

[Covers: req-8552172-00528003]

[Covers: req-8552172-00528002]

[Covers: req-8552172-00528006]

[Covers: req-8552172-00527993]

[Covers: req-8552172-00527980]

### R_ProtoConv_TriggerXtoEthTx

[iso26-18_p6_7.4.5] a; [iso26-18_p6_7.4.6]

[Gen5GW_ProtoConv_AD_C_10_006]

Table 2-25: R_ProtoConv_TriggerXtoEthTx

| Function Name | R_ProtoConv_TriggerXtoEthTx |  |  |  |
| --- | --- | --- | --- | --- |
| ASIL Level | ASIL D | Status | New |  |
| Syntax | Std_ReturnType R_ProtoConv_TriggerXtoEthTx(const uint32 ulBufIdx) |  |  |  |
| Sync/Async | Synchronous |  |  |  |
| Reentrancy | Non-Reentrant |  |  |  |
| Interrupt State | Runs in the interrupt enable state |  |  |  |
| Parameters (In) | const uint32 ulBufIdx |  | Target Buffer Index |  |
|  |  |  | Range | 0 - PROTOCONV_MIXBUF_MAX |
| Parameters (In-Out) | N/A |  | - |  |
| Parameters (Out) | N/A |  | - |  |
| Return Value | Std_ReturnType |  | E_OK E_NOT_OK |  |
| Description | If R_ProtoConv_Init() is not executed yet, it returns E_NOT_OK, reports DET error PROTOCONV_E_UNINIT (if PROTOCONV_DEV_ERROR_DETECT == STD_ON) and exits. If the argument ulBufIdx is out of range, it returns E_NOT_OK, reports DET error PROTOCONV_E_INV_PARAM (if PROTOCONV_DEV_ERROR_DETECT == STD_ON) and exits. R_ProtoConv_TriggerXtoEthTx() executes ProtoConv_TriggerXtoEthTx() and returns its return value. |  |  |  |
| Preconditions | Uninitialized state |  |  |  |
| Remarks | N/A |  |  |  |
|  | PROTOCONV_E_INV_PARAM |  | Invalid parameter passed to the APIs |  |
|  | PROTOCONV_E_UNINIT |  | Process error (API executed in uninitialized state) |  |
|  | PROTOCONV_E_TRANS |  | Transmit Error |  |
|  | PROTOCONV_E_PARAM_CONFIG |  | Config Error (NULL Pointer) |  |

[Covers: req-8552172-00903610]

[Covers: req-8552172-00903611]

[Covers: req-8552172-00903612]

[Covers: req-8552172-00905227]

[Covers: req-8552172-00904662]

[Covers: req-8552172-00909476]

[Covers: req-8552172-00909478]

[Covers: req-8552172-00528003]

[Covers: req-8552172-00528002]

[Covers: req-8552172-00528006]

[Covers: req-8552172-00527993]

[Covers: req-8552172-00527980]

### R_ProtoConv_MainFunction

[Gen5GW_ProtoConv_AD_C_10_007]

Table 2-26: R_ProtoConv_MainFunction

| Function Name | R_ProtoConv_MainFunction |  |  |
| --- | --- | --- | --- |
| ASIL Level | ASIL D | Status | New |
| Syntax | void R_ProtoConv_MainFunction(void) |  |  |
| Sync/Async | Synchronous |  |  |
| Reentrancy | Non-Reentrant |  |  |
| Interrupt State | Runs in the interrupt enable state |  |  |
| Parameters (In) | N/A |  | - |
| Parameters (In-Out) | N/A |  | - |
| Parameters (Out) | N/A |  | - |
| Return Value | N/A |  | - |
| Description | If R_ProtoConv_Init() is not executed yet, terminates the process. Execute ProtoConv_MixtoEthMain() and end the process. |  |  |
| Preconditions | Initialized state |  |  |
| Remarks | N/A |  |  |
| DET/DEM Errors Handled | N/A |  |  |

[Covers: req-8552172-00905314]

[Covers: req-8552172-00905315]

[Covers: req-8552172-00905316]

[Covers: req-8552172-00905317]

[Covers: req-8552172-00903610]

[Covers: req-8552172-00903611]

[Covers: req-8552172-00903612]

[Covers: req-8552172-00904667]

[Covers: req-8552172-00905227]

[Covers: req-8552172-00909476]

[Covers: req-8552172-00909478]

[Covers: req-8552172-00528006]

[Covers: req-8552172-00527993]

### R_ProtoConv_EthtoX

[iso26-18_p6_7.4.5] a; [iso26-18_p6_7.4.6]

[Gen5GW_ProtoConv_AD_C_10_008]

Table 2-27: R_ProtoConv_EthtoX

| Function Name | R_ProtoConv_EthtoX |  |  |  |
| --- | --- | --- | --- | --- |
| ASIL Level | ASIL D | Status | New |  |
| Syntax | Std_ReturnType R_ProtoConv_EthtoX(const uint16 usFrameType, const uint8 *aaSrcMacAddr, const uint8 *pEthFrame, const uint32 ulFrameLen, uint32 *pSendStatus) |  |  |  |
| Sync/Async | Synchronous |  |  |  |
| Reentrancy | Non-Reentrant |  |  |  |
| Interrupt State | Runs in the interrupt enable state |  |  |  |
| Parameters (In) | const uint16 usFrameType |  | Frame type for Ethernet frames |  |
|  |  |  | Range | PROTOCONV_FRAMETYPE_AVTP (0x22F0U) PROTOCONV_FRAMETYPE_IPV4 (0x0800U) PROTOCONV_FRAMETYPE_IPV6 (0x86DDU) PROTOCONV_FRAMETYPE_8021Q (0x8100U) |
| Parameters (In) | const uint8 *aaSrcMacAddr |  | Address where the MAC Address to be filtered is stored |  |
|  |  |  | Range | Not NULL_PTR |
| Parameters (In) | const uint8 *pEthFrame |  | Address of buffer where Ethernet frames are stored |  |
|  |  |  | Range | Not NULL_PTR |
| Parameters (In) | const uint32 ulFrameLen |  | Data size of Ethernet frame |  |
|  |  |  | Range | 46~1500 |
| Parameters (In-Out) | N/A |  | - |  |
| Parameters (Out) | uint32 *pSendStatus |  | Address to store packet transmission results |  |
|  |  |  | Range | Not NULL_PTR |
| Return Value | Std_ReturnType |  | E_OK E_NOT_OK |  |
| Description | If R_ProtoConv_Init() is not executed yet, returns E_NOT_OK, report DET error PROTOCONV_E_UNINIT (if PROTOCONV_DEV_ERROR_DETECT == STD_ON) and exits. If any of the arguments pEthFrame, aaSrcMacAddr, or pSendStatus is NULL_PTR, it returns E_NOT_OK, report DET error PROTOCONV_E_PARAM_POINTER (if PROTOCONV_DEV_ERROR_DETECT == STD_ON) and exits. ProtoConv_EthtoX() is executed and its return value is returned. |  |  |  |
| Preconditions | Initialized state |  |  |  |
| Remarks | Refer to the following item for the contents stored in *pSendStatus, which stores the CAN packet transmission result. 2.3 Ethernet to CAN Conversion Function (Gen5GW_ProtoConv_AD_C_05_008) |  |  |  |
| DET/DEM Errors Handled | PROTOCONV_E_PARAM_POINTER |  |  | Parameter error (argument is NULL_PTR) |
|  | PROTOCONV_E_UNINIT |  |  | Process error (API executed in uninitialized state) |
|  | PROTOCONV_E_PARAM_DATA_LENGTH |  |  | Format Error |
|  | PROTOCONV_E_TRANS |  |  | Transmit Error |
|  | PROTOCONV_E_BSWNOTIFY |  |  | BSW Notification Error |
|  | PROTOCONV_E_IPCKSUM |  |  | IP Header Checksum Error |
|  | PROTOCONV_E_UDPCKSUM |  |  | UDP Header Checksum Error |
|  | PROTOCONV_E_PACKEDCAN |  |  | Packed CAN Frame Error |
|  | PROTOCONV_E_PACKEDLIN |  |  | Packed LIN Frame Error |
|  | PROTOCONV_E_PACKEDPORT |  |  | Packed Port Frame Error |
|  | PROTOCONV_E_PACKEDMSG |  |  | Packed CAN/LIN/Port Frame Error |
|  | PROTOCONV_E_FILTER |  |  | Filtered and discarded |
|  | PROTOCONV_E_CFG_ACFTYPE |  |  | Incorrect or non-supported ACF type |

[Covers: req-8552172-00905318]

[Covers: req-8552172-00905319]

[Covers: req-8552172-00905320]

[Covers: req-8552172-00905321]

[Covers: req-8552172-00905221]

[Covers: req-8552172-00903613]

[Covers: req-8552172-00904663]

[Covers: req-8552172-00904662]

[Covers: req-8552172-00905224]

[Covers: req-8552172-00905235]

[Covers: req-8552172-00905234]

[Covers: req-8552172-00905238]

[Covers: req-8552172-00905225]

[Covers: req-8552172-00905237]

[Covers: req-8552172-00905226]

[Covers: req-8552172-00904664]

[Covers: req-8552172-00904665]

[Covers: req-8552172-00904666]

[Covers: req-8552172-00909476]

[Covers: req-8552172-00909478]

[Covers: req-8552172-00528003]

[Covers: req-8552172-00528002]

[Covers: req-8552172-00528006]

[Covers: req-8552172-00527993]

[Covers: req-8552172-00527980]

### R_ProtoConv_GetSendStatus

[Gen5GW_ProtoConv_AD_C_10_009]

Table 2-28: R_ProtoConv_GetSendStatus

| Function Name | R_ProtoConv_GetSendStatus |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| ASIL Level | ASIL D |  | Status |  | New |  |
| Syntax | Std_ReturnType R_ProtoConv_GetSendStatus(uint32 *pStatus) |  |  |  |  |  |
| Sync/Async | Synchronous |  |  |  |  |  |
| Reentrancy | Non-Reentrant |  |  |  |  |  |
| Interrupt State | Runs in the interrupt enable state |  |  |  |  |  |
| Parameters (In) | N/A | - |  |  |  |  |
| Parameters (In-Out) | N/A | - |  |  |  |  |
| Parameters (Out) | uint32 *pStatus | Pointer to store packet transmission suppression setting information |  |  |  |  |
|  |  | Range |  | Not NULL_PTR |  |  |
| Return Value | Std_ReturnType | E_OK E_NOT_OK |  |  |  |  |
| Description | If R_ProtoConv_Init() is not executed, returns E_NOT_OK, report DET error PROTOCONV_E_UNINIT (if PROTOCONV_DEV_ERROR_DETECT == STD_ON) and terminates the process. If the pointer (*pStatus) that stores the setting information of packet send suppression is NULL_PTR, return E_NOT_OK, report DET error PROTOCONV_E_PARAM_POINTER (if PROTOCONV_DEV_ERROR_DETECT == STD_ON) and terminates the process. Execute ProtoConv_GetSendStatus(), set return value as E_OK and end the process. |  |  |  |  |  |
| Preconditions | Initialized state |  |  |  |  |  |
| Remarks | Refer to the following items for the contents stored in *pStatus, which stores the CAN packet transmission suppression setting information. Ethernet to CAN Conversion Function2.1.5.1 Ethernet to CAN Conversion Function (Gen5GW_ProtoConv_AD_C_05_004) |  |  |  |  |  |
| DET/DEM Errors Handled | PROTOCONV_E_PARAM_POINTER |  |  |  |  | Parameter error (argument is NULL_PTR) |
|  | PROTOCONV_E_UNINIT |  |  |  |  | Process error (API executed in uninitialized state) |

[Covers: req-8552172-00905218]

[Covers: req-8552172-00909476]

[Covers: req-8552172-00909478]

[Covers: req-8552172-00528003]

[Covers: req-8552172-00528002]

[Covers: req-8552172-00528006]

[Covers: req-8552172-00527993]

[Covers: req-8552172-00527980]

### R_ProtoConv_UpdateSendStatus

[Gen5GW_ProtoConv_AD_C_10_010]

Table 2-29: R_ProtoConv_UpdateSendStatus

| Function Name | R_ProtoConv_GetSendStatus |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| ASIL Level | ASIL D | Status |  | New |  |
| Syntax | Std_ReturnType R_ProtoConv_UpdateSendStatus(const uint32 ulStatus) |  |  |  |  |
| Sync/Async | Synchronous |  |  |  |  |
| Reentrancy | Non-Reentrant |  |  |  |  |
| Interrupt State | Runs in the interrupt enable state |  |  |  |  |
| Parameters (In) | const uint32 ulStatus |  |  | Variable to store the global address of packet transmission suppression setting information |  |
|  |  |  |  | Range | 14-bit value |
| Parameters (In-Out) | N/A |  |  | - |  |
| Parameters (Out) | N/A |  |  | - |  |
| Return Value | Std_ReturnType |  |  | E_OK E_NOT_OK |  |
| Description | If R_ProtoConv_Init() is not executed, returns E_NOT_OK, report DET error PROTOCONV_E_UNINIT (if PROTOCONV_DEV_ERROR_DETECT == STD_ON) and terminates the process. Execute ProtoConv_UpdateSendStatus (), set return value as E_OK and end the process. |  |  |  |  |
| Preconditions | Initialized state |  |  |  |  |
| Remarks | Refer to the following items for the contents stored in ulStatus, which stores the CAN packet transmission suppression setting information. Ethernet to CAN Conversion Function2.1.5.1 Ethernet to CAN Conversion Function (Gen5GW_ProtoConv_AD_C_05_004) |  |  |  |  |
| DET/DEM Errors Handled | PROTOCONV_E_UNINIT |  | Process error (API executed in uninitialized state) |  |  |
|  | PROTOCONV_E_INV_PARAM |  | Invalid parameter passed to the APIs |  |  |

[Covers: req-8552172-00905219]

[Covers: req-8552172-00909476]

[Covers: req-8552172-00909478]

[Covers: req-8552172-00528003]

[Covers: req-8552172-00528002]

[Covers: req-8552172-00528006]

[Covers: req-8552172-00527993]

[Covers: req-8552172-00527980]

### R_ProtoConv_EnqueueCan

[iso26-18_p6_7.4.5] a; [iso26-18_p6_7.4.6]

[Gen5GW_ProtoConv_AD_C_10_011]

Table 2-30: R_ProtoConv_EnqueueCan

| Function Name | R_ProtoConv_EnqueueCan |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| ASIL Level | ASIL D | Status |  | New |  |
| Syntax | Std_ReturnType R_ProtoConv_EnqueueCan(const uint8 ucBusIdx, const R_ProtoConv_CanFrameType *pCanFrame, const R_ProtoConv_TimeStampType *pTimeStamp) |  |  |  |  |
| Sync/Async | Synchronous |  |  |  |  |
| Reentrancy | Non-Reentrant |  |  |  |  |
| Interrupt State | Runs in the interrupt enable state |  |  |  |  |
| Parameters (In) | const uint8 ucBusIdx |  |  | Source Bus ID |  |
|  |  |  |  | Range | 0 - (PROTOCONV_CAN_CH_NUM-1) |
| Parameters (In) | const R_ProtoConv_CanFrameType *pCanFrame |  |  | Address of buffer where CAN frames are stored |  |
|  |  |  |  | Range | Not NULL_PTR |
| Parameters (In) | const R_ProtoConv_TimeStampType *pTimeStamp |  |  | Address of buffer where timestamp is stored |  |
|  |  |  |  | Range | Not NULL_PTR |
| Parameters (In-Out) | N/A |  |  | - |  |
| Parameters (Out) | N/A |  |  | - |  |
| Return Value | Std_ReturnType |  |  | E_OK E_NOT_OK |  |
| Description | If R_ProtoConv_Init() is not executed yet, it returns E_NOT_OK, reports DET error PROTOCONV_E_UNINIT (if PROTOCONV_DEV_ERROR_DETECT == STD_ON) and exits. If the argument pCanFrame or pTimeStamp is NULL_PTR, it returns E_NOT_OK, reports DET error PROTOCONV_E_PARAM_POINTER (if PROTOCONV_DEV_ERROR_DETECT == STD_ON) and exits. If the argument ucBusIdx is out of range, it returns E_NOT_OK, reports DET error PROTOCONV_E_INV_PARAM (if PROTOCONV_DEV_ERROR_DETECT == STD_ON) and exits. R_ProtoConv_EnqueueCan() executes ProtoConv_EnqueueCan() and returns its return value. |  |  |  |  |
| Preconditions | Initialized state |  |  |  |  |
| Remarks | N/A |  |  |  |  |
| DET/DEM Errors Handled | PROTOCONV_E_PARAM_POINTER |  | Parameter error (argument is NULL_PTR) |  |  |
|  | PROTOCONV_E_INV_PARAM |  | Invalid parameter passed to the APIs |  |  |
|  | PROTOCONV_E_UNINIT |  | Process error (API executed in uninitialized state) |  |  |
|  | PROTOCONV_E_FIFO_FULL |  | Unexpectedly full FIFO memory |  |  |

[Covers: req-8552172-00909163]

[Covers: req-8552172-00903610]

[Covers: req-8552172-00905227]

[Covers: req-8552172-00904662]

[Covers: req-8552172-00909476]

[Covers: req-8552172-00909478]

[Covers: req-8552172-00528003]

[Covers: req-8552172-00528002]

[Covers: req-8552172-00528006]

[Covers: req-8552172-00527993]

[Covers: req-8552172-00527980]

### R_ProtoConv_EnqueueLin

[iso26-18_p6_7.4.5] a; [iso26-18_p6_7.4.6]

[Gen5GW_ProtoConv_AD_C_10_012]

Table 2-31: R_ProtoConv_EnqueueLin

| Function Name | R_ProtoConv_EnqueueLin |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| ASIL Level | ASIL D | Status | New |  |  |
| Syntax | Std_ReturnType R_ProtoConv_EnqueueLin(const uint8 ucBusIdx, const R_ProtoConv_LinFrameType *pLinFrame) |  |  |  |  |
| Sync/Async | Synchronous |  |  |  |  |
| Reentrancy | Non-Reentrant |  |  |  |  |
| Interrupt State | Runs in the interrupt enable state |  |  |  |  |
| Parameters (In) | const uint8 ucBusIdx |  | Source Bus ID |  |  |
|  |  |  | Range |  | 0 - (PROTOCONV_LIN_CH_NUM-1) |
| Parameters (In) | const R_ProtoConv_LinFrameType *pLinFrame |  | Address of buffer where LIN frames are stored |  |  |
|  |  |  | Range |  | Not NULL_PTR |
| Parameters (In-Out) | N/A |  | - |  |  |
| Parameters (Out) | N/A |  | - |  |  |
| Return Value | Std_ReturnType |  | E_OK E_NOT_OK |  |  |
| Description | If R_ProtoConv_Init() is not executed yet, it returns E_NOT_OK, reports DET error PROTOCONV_E_UNINIT (if PROTOCONV_DEV_ERROR_DETECT == STD_ON) and exits. If the argument pLinFrame is NULL_PTR, it returns E_NOT_OK, reports DET error PROTOCONV_E_PARAM_POINTER (if PROTOCONV_DEV_ERROR_DETECT == STD_ON) and exits. If the argument ucBusIdx is out of range, it returns E_NOT_OK, reports DET error PROTOCONV_E_INV_PARAM (if PROTOCONV_DEV_ERROR_DETECT == STD_ON) and exits. R_ProtoConv_EnqueueLin() executes R_ProtoConv_EnqueueLin() and returns its return value. |  |  |  |  |
| Preconditions | Initialized state |  |  |  |  |
| Remarks | N/A |  |  |  |  |
| DET/DEM Errors Handled | PROTOCONV_E_PARAM_POINTER |  |  | Parameter error (argument is NULL_PTR) |  |
|  | PROTOCONV_E_INV_PARAM |  |  | Invalid parameter passed to the APIs |  |
|  | PROTOCONV_E_UNINIT |  |  | Process error (API executed in uninitialized state) |  |
|  | PROTOCONV_E_FIFO_FULL |  |  | Unexpectedly full FIFO memory |  |

[Covers: req-8552172-00909163]

[Covers: req-8552172-00903611]

[Covers: req-8552172-00905227]

[Covers: req-8552172-00904662]

[Covers: req-8552172-00909476]

[Covers: req-8552172-00909478]

[Covers: req-8552172-00528003]

[Covers: req-8552172-00528002]

[Covers: req-8552172-00528006]

[Covers: req-8552172-00527993]

[Covers: req-8552172-00527980]

### R_ProtoConv_EnqueuePort

[iso26-18_p6_7.4.5] a; [iso26-18_p6_7.4.6]

[Gen5GW_ProtoConv_AD_C_10_013]

Table 2-32: R_ProtoConv_EnqueuePort

| Function Name | R_ProtoConv_EnqueuePort |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| ASIL Level | ASIL D |  |  | Status | New |
| Syntax | Std_ReturnType R_ProtoConv_EnqueuePort(const uint16 usLength, const uint8 *pPortData, const uint32 ulDestIdx) |  |  |  |  |
| Sync/Async | Synchronous |  |  |  |  |
| Reentrancy | Non-Reentrant |  |  |  |  |
| Interrupt State | Runs in the interrupt enable state |  |  |  |  |
| Parameters (In) | const uint16 usLength | Data Length |  |  |  |
|  |  | Range | MIN_PORT_TOTAL_SIZE - MAX_PORT_TOTAL_SIZE |  |  |
| Parameters (In) | const uint8 *pPortData | Address of buffer where Port Data are stored |  |  |  |
|  |  | Range | Not NULL_PTR |  |  |
| Parameters (In) | const uint32 ulDestIdx | Index of Destination Rooting Table |  |  |  |
|  |  | Range | 0 - (XTOETH_DEST_TABLE_ENTRY_NUM-1) |  |  |
| Parameters (In-Out) | N/A | - |  |  |  |
| Parameters (Out) | N/A | - |  |  |  |
| Return Value | Std_ReturnType | E_OK E_NOT_OK |  |  |  |
| Description | If R_ProtoConv_Init() is not executed yet, it returns E_NOT_OK, reports DET error PROTOCONV_E_UNINIT (if PROTOCONV_DEV_ERROR_DETECT == STD_ON) and exits. If the argument pPortData is NULL_PTR, it returns E_NOT_OK, reports DET error PROTOCONV_E_PARAM_POINTER (if PROTOCONV_DEV_ERROR_DETECT == STD_ON) and exits. If the argument usLength or ulDestIdx is out of range, it returns E_NOT_OK, reports DET error PROTOCONV_E_INV_PARAM (if PROTOCONV_DEV_ERROR_DETECT == STD_ON) and exits. R_ProtoConv_EnqueuePort() executes ProtoConv_EnqueuePort() and returns its return value. |  |  |  |  |
| Preconditions | Initialized state |  |  |  |  |
| Remarks | N/A |  |  |  |  |
| DET/DEM Errors Handled | PROTOCONV_E_PARAM_POINTER |  | Parameter error (argument is NULL_PTR) |  |  |
|  | PROTOCONV_E_INV_PARAM |  | Invalid parameter passed to the APIs |  |  |
|  | PROTOCONV_E_UNINIT |  | Process error (API executed in uninitialized state) |  |  |
|  | PROTOCONV_E_FIFO_FULL |  | Unexpectedly full FIFO memory |  |  |

[Covers: req-8552172-00909163]

[Covers: req-8552172-00903612]

[Covers: req-8552172-00905227]

[Covers: req-8552172-00904662]

[Covers: req-8552172-00909476]

[Covers: req-8552172-00909478]

[Covers: req-8552172-00528003]

[Covers: req-8552172-00528002]

[Covers: req-8552172-00528006]

[Covers: req-8552172-00527993]

[Covers: req-8552172-00527980]

### R_ProtoConv_EnqueueEth

[iso26-18_p6_7.4.5] a; [iso26-18_p6_7.4.6]

[Gen5GW_ProtoConv_AD_C_10_014]

Table 2-33: R_ProtoConv_EnqueueEth

| Function Name | R_ProtoConv_EnqueueEth |  |  |  |
| --- | --- | --- | --- | --- |
| ASIL Level | ASIL D | Status | New |  |
| Syntax | Std_ReturnType R_ProtoConv_EnqueueEth(const uint16 usFrameType, const uint8 *aaSrcMacAddr, const uint8 *pEthFrame, const uint32 ulFrameLen) |  |  |  |
| Sync/Async | Synchronous |  |  |  |
| Reentrancy | Non-Reentrant |  |  |  |
| Interrupt State | Runs in the interrupt enable state |  |  |  |
| Parameters (In) | const uint16 usFrameType |  | Frame type for Ethernet frames |  |
|  |  |  | Range | PROTOCONV_FRAMETYPE_AVTP (0x22F0U) PROTOCONV_FRAMETYPE_IPV4 (0x0800U) PROTOCONV_FRAMETYPE_IPV6 (0x86DDU) PROTOCONV_FRAMETYPE_8021Q (0x8100U) |
| Parameters (In) | const uint8 *aaSrcMacAddr |  | Address where the MAC Address to be filtered is stored |  |
|  |  |  | Range | Not NULL_PTR |
| Parameters (In) | const uint8 *pEthFrame |  | Address of buffer where Ethernet frames are stored |  |
|  |  |  | Range | Not NULL_PTR |
| Parameters (In) | const uint32 ulFrameLen |  | Data size of Ethernet frame |  |
|  |  |  | Range | 46~1500 |
| Parameters (In-Out) | N/A |  | - |  |
| Parameters (Out) | N/A |  | - |  |
| Return Value | Std_ReturnType |  | E_OK E_NOT_OK |  |
| Description | If R_ProtoConv_Init() is not executed yet, returns E_NOT_OK, report DET error PROTOCONV_E_UNINIT (if PROTOCONV_DEV_ERROR_DETECT == STD_ON) and exits. If any of the arguments pEthFrame or aaSrcMacAddr is NULL_PTR, it returns E_NOT_OK, report DET error PROTOCONV_E_PARAM_POINTER (if PROTOCONV_DEV_ERROR_DETECT == STD_ON) and exits. If ulFrameLen is out of range, it returns E_NOT_OK, report DET error PROTOCONV_E_PARAM_DATA_LENGTH (if PROTOCONV_DEV_ERROR_DETECT == STD_ON) and exits. ProtoConv_EnqueueEth() is executed and its return value is returned. |  |  |  |
| Preconditions | Initialized state |  |  |  |
| Remarks | N/A |  |  |  |
| DET/DEM Errors Handled | PROTOCONV_E_PARAM_POINTER |  |  | Parameter error (argument is NULL_PTR) |
|  | PROTOCONV_E_PARAM_DATA_LENGTH |  |  | Format error |
|  | PROTOCONV_E_UNINIT |  |  | Process error (API executed in uninitialized state) |
|  | PROTOCONV_E_FIFO_FULL |  |  | Unexpectedly full FIFO memory |

[Covers: req-8552172-00905318]

[Covers: req-8552172-00905319]

[Covers: req-8552172-00905320]

[Covers: req-8552172-00905321]

[Covers: req-8552172-00905221]

[Covers: req-8552172-00903613]

[Covers: req-8552172-00904663]

[Covers: req-8552172-00904662]

[Covers: req-8552172-00905224]

[Covers: req-8552172-00905235]

[Covers: req-8552172-00905234]

[Covers: req-8552172-00905238]

[Covers: req-8552172-00905225]

[Covers: req-8552172-00905237]

[Covers: req-8552172-00905226]

[Covers: req-8552172-00904664]

[Covers: req-8552172-00904665]

[Covers: req-8552172-00904666]

[Covers: req-8552172-00909163]

[Covers: req-8552172-00909476]

[Covers: req-8552172-00909478]

[Covers: req-8552172-00528003]

[Covers: req-8552172-00528002]

[Covers: req-8552172-00528006]

[Covers: req-8552172-00527993]

[Covers: req-8552172-00527980]

### R_ProtoConv_DequeueEth

[iso26-18_p6_7.4.5] a; [iso26-18_p6_7.4.6]

[Gen5GW_ProtoConv_AD_C_10_015]

Table 2-34: R_ProtoConv_DequeueEth

| Function Name | R_ProtoConv_DequeueEth |  |  |  |
| --- | --- | --- | --- | --- |
| ASIL Level | ASIL D | Status | New |  |
| Syntax | Std_ReturnType R_ProtoConv_DequeueEth(uint16 *pFrameType, uint8 **aaSrcMacAddr, uint8 **pEthFrame, uint32 *pFrameLen) |  |  |  |
| Sync/Async | Synchronous |  |  |  |
| Reentrancy | Non-Reentrant |  |  |  |
| Interrupt State | Runs in the interrupt enable state |  |  |  |
| Parameters (In) | N/A |  | - |  |
| Parameters (In-Out) | N/A |  | - |  |
| Parameters (Out) | uint16 *pFrameType |  | Frame type for Ethernet frames |  |
|  |  |  | Range | PROTOCONV_FRAMETYPE_AVTP (0x22F0U) PROTOCONV_FRAMETYPE_IPV4 (0x0800U) PROTOCONV_FRAMETYPE_IPV6 (0x86DDU) PROTOCONV_FRAMETYPE_8021Q (0x8100U) |
| Parameters (Out) | uint8 **aaSrcMacAddr |  | Address where the MAC Address to be filtered is stored |  |
|  |  |  | Range | Not NULL_PTR |
| Parameters (Out) | uint8 **pEthFrame |  | Address of buffer where Ethernet frames are stored |  |
|  |  |  | Range | Not NULL_PTR |
| Parameters (Out) | uint32 *pFrameLen |  | Data size of Ethernet frame |  |
|  |  |  | Range | 46~1500 |
| Return Value | Std_ReturnType |  | E_OK E_NOT_OK |  |
| Description | If R_ProtoConv_Init() is not executed yet, returns E_NOT_OK, report DET error PROTOCONV_E_UNINIT (if PROTOCONV_DEV_ERROR_DETECT == STD_ON) and exits. If any of the arguments pFrameType, aaSrcMacAddr, pEthFrame, or pFrameLen is NULL_PTR, it returns E_NOT_OK, report DET error PROTOCONV_E_PARAM_POINTER (if PROTOCONV_DEV_ERROR_DETECT == STD_ON) and exits. ProtoConv_DequeueEth() is executed and its return value is returned. |  |  |  |
| Preconditions | Initialized state |  |  |  |
| Remarks | N/A |  |  |  |
| DET/DEM Errors Handled | PROTOCONV_E_PARAM_POINTER |  |  | Parameter error (argument is NULL_PTR) |
|  | PROTOCONV_E_UNINIT |  |  | Process error (API executed in uninitialized state) |
|  | PROTOCONV_E_FIFO_EMPTY |  |  | Unexpectedly empty FIFO memory |

[Covers: req-8552172-00905318]

[Covers: req-8552172-00905319]

[Covers: req-8552172-00905320]

[Covers: req-8552172-00905321]

[Covers: req-8552172-00905221]

[Covers: req-8552172-00903613]

[Covers: req-8552172-00904663]

[Covers: req-8552172-00904662]

[Covers: req-8552172-00905224]

[Covers: req-8552172-00905235]

[Covers: req-8552172-00905234]

[Covers: req-8552172-00905238]

[Covers: req-8552172-00905225]

[Covers: req-8552172-00905237]

[Covers: req-8552172-00905226]

[Covers: req-8552172-00904664]

[Covers: req-8552172-00904665]

[Covers: req-8552172-00904666]

[Covers: req-8552172-00909163]

[Covers: req-8552172-00909476]

[Covers: req-8552172-00909478]

[Covers: req-8552172-00528003]

[Covers: req-8552172-00528002]

[Covers: req-8552172-00528006]

[Covers: req-8552172-00527993]

[Covers: req-8552172-00527980]

### R_ProtoConv_TriggerQueueDataXtoEth

[Gen5GW_ProtoConv_AD_C_10_016]

Table 2-35: R_ProtoConv_TriggerQueueDataXtoEth

| Function Name | R_ProtoConv_TriggerQueueDataXtoEth |  |  |  |
| --- | --- | --- | --- | --- |
| ASIL Level | ASIL D | Status | New |  |
| Syntax | Std_ReturnType R_ProtoConv_TriggerQueueDataXtoEth(void) |  |  |  |
| Sync/Async | Synchronous |  |  |  |
| Reentrancy | Non-Reentrant |  |  |  |
| Interrupt State | Runs in the interrupt enable state |  |  |  |
| Parameters (In) | N/A |  | - |  |
| Parameters (In-Out) | N/A |  | - |  |
| Parameters (Out) | N/A |  | - |  |
| Return Value | N/A |  | - |  |
| Description | If R_ProtoConv_Init() is not executed yet, it returns E_NOT_OK, reports DET error PROTOCONV_E_UNINIT (if PROTOCONV_DEV_ERROR_DETECT == STD_ON) and exits. R_ProtoConv_TriggerQueueDataXtoEth() executes ProtoConv_TriggerQueueDataXtoEth(). If ProtoConv_TriggerQueueDataXtoEth() ends abnormally and returns other than E_OK, R_ProtoConv_TriggerQueueDataXtoEth() returns E_NOT_OK. If ProtoConv_TriggerQueueDataXtoEth() ends normally, it returns E_OK. |  |  |  |
| Preconditions | Initialized state |  |  |  |
| Remarks | N/A |  |  |  |
| DET/DEM Errors Handled | PROTOCONV_E_UNINIT |  |  | Process error (API executed in uninitialized state) |
|  | PROTOCONV_E_FIFO_EMPTY |  |  | Unexpectedly empty FIFO memory |
|  | PROTOCONV_E_PARAM_POINTER |  |  | Parameter error (argument is NULL_PTR) |
|  | PROTOCONV_E_INV_PARAM |  |  | Invalid parameter passed to the APIs |
|  | PROTOCONV_E_MATCH |  |  | Unmatched Table Error (Routing Table) |
|  | PROTOCONV_E_TRANS |  |  | Transmit Error |
|  | PROTOCONV_E_BUF_FULL |  |  | Unexpectedly full Buffer error |
|  | PROTOCONV_E_PARAM_CONFIG |  |  | Config Error (NULL Pointer) |

[Covers: req-8552172-00909163]

[Covers: req-8552172-00903610]

[Covers: req-8552172-00903611]

[Covers: req-8552172-00903612]

[Covers: req-8552172-00905227]

[Covers: req-8552172-00904662]

[Covers: req-8552172-00909476]

[Covers: req-8552172-00909478]

[Covers: req-8552172-00528003]

[Covers: req-8552172-00528002]

[Covers: req-8552172-00528006]

[Covers: req-8552172-00527993]

[Covers: req-8552172-00527980]

## Interfaces (User defined)

This chapter shows the design details of functions in the Required Interface in Figure 2-1.

### ProtoConv_EthTransmitUserDef

[Gen5GW_ProtoConv_AD_C_20_001]

Table 2-36: ProtoConv_EthTransmitUserDef

| Function Name | ProtoConv_EthTransmitUserDef |  |  |
| --- | --- | --- | --- |
| ASIL Level | ASIL D | Status | New |
| Syntax | Std_ReturnType ProtoConv_EthTransmitUserDef(const uint8 *aaDestAddr, const uint16 usFrameType, const uint8 *pBufAddr, const uint32 ulDataSize) |  |  |
| Sync/Async | Synchronous |  |  |
| Reentrancy | Non-Reentrant |  |  |
| Interrupt State | Runs in the interrupt enable state |  |  |
| Parameters (In) | const uint8 *aaDestAddr | Destination MAC address |  |
|  |  | Range | Not NULL_PTR |
| Parameters (In) | const uint16 usFrameType | EtherType |  |
|  |  | Range | PROTOCONV_FRAMETYPE_AVTP (0x22F0U) PROTOCONV_FRAMETYPE_IPV4 (0x0800U) PROTOCONV_FRAMETYPE_IPV6 (0x86DDU) PROTOCONV_FRAMETYPE_8021Q (0x8100U) |
| Parameters (In) | const uint8 *pBufAddr | Ethernet Tx Buffer address |  |
|  |  | Range | Not NULL_PTR |
| Parameters (In) | const uint32 ulDataSize | Transmit data size |  |
|  |  | Range | ETH_FRAME_PAYLOAD_SIZE_MIN - 1500U |
| Parameters (In-Out) | N/A | - |  |
| Parameters (Out) | N/A | - |  |
| Return Value | Std_ReturnType | E_OK E_NOT_OK |  |
| Description | The user implements the process of sending Ethernet frames in this function. If the process ends normally, E_OK is returned. If the process ends abnormally, a value other than E_OK is returned; if a value other than E_OK is returned, the caller converts it to E_NOT_OK. |  |  |
| Preconditions | N/A |  |  |
| Remarks | N/A |  |  |
| DET/DEM Errors Handled | N/A |  |  |

[Covers: req-8552172-00903610]

[Covers: req-8552172-00905227]

[Covers: req-8552172-00909476]

[Covers: req-8552172-00909478]

[Covers: req-8552172-00528006]

[Covers: req-8552172-00527993]

### ProtoConv_CanTransmitTSUserDef

[Gen5GW_ProtoConv_AD_C_20_002]

Table 2-37: ProtoConv_CanTransmitTSUserDef

| Function Name | ProtoConv_CanTransmitTSUserDef |  |  |  |
| --- | --- | --- | --- | --- |
| ASIL Level | ASIL D | Status | New |  |
| Syntax | Std_ReturnType ProtoConv_CanTransmitTSUserDef(const R_ProtoConv_CanFrameType *pCanFrame, const uint8 ucDestBusIdx, const uint8 ucSrcBusIdx, const uint32 ulAddInfo, const R_ProtoConv_TimeStampType*pTimeStamp) |  |  |  |
| Sync/Async | Synchronous |  |  |  |
| Reentrancy | Non-Reentrant |  |  |  |
| Interrupt State | Runs in the interrupt enable state |  |  |  |
| Parameters (In) | const R_ProtoConv_CanFrameType *pCanFrame |  | Address of buffer where CAN frames are stored |  |
|  |  |  | Range | Not NULL_PTR |
| Parameters (In) | const uint8 ucDestBusIdx |  | Destination Bus ID |  |
|  |  |  | Range | 0 - (PROTOCONV_CAN_CH_NUM-1) |
| Parameters (In) | const uint8 ucSrcBusIdx |  | Source Bus ID |  |
|  |  |  | Range | 0 - (PROTOCONV_CAN_CH_NUM-1) |
| Parameters (In) | const uint32 ulAddInfo |  | Additional Information |  |
|  |  |  | Range | 32-bit value |
| Parameters (In) | const R_ProtoConv_TimeStampType *pTimeStamp |  | Address of buffer where timestamp is stored |  |
|  |  |  | Range | Not NULL_PTR |
| Parameters (In-Out) | N/A |  | - |  |
| Parameters (Out) | N/A |  | - |  |
| Return Value | Std_ReturnType |  | E_OK E_NOT_OK |  |
| Description | The user implements the CAN/CAN FD frame with TimeStamp transmission process in this function. If the process ends normally, E_OK is returned. If the process ends abnormally, a value other than E_OK is returned; if a value other than E_OK is returned, the caller converts it to E_NOT_OK. |  |  |  |
| Preconditions | N/A |  |  |  |
| Remarks | N/A |  |  |  |
| DET/DEM Errors Handled | N/A |  |  |  |

[Covers: req-8552172-00903613]

[Covers: req-8552172-00905235]

[Covers: req-8552172-00909476]

[Covers: req-8552172-00909478]

[Covers: req-8552172-00528006]

[Covers: req-8552172-00527993]

### ProtoConv_CanNotifyTSUserDef

[Gen5GW_ProtoConv_AD_C_20_003]

Table 2-38: ProtoConv_CanNotifyTSUserDef

| Function Name | ProtoConv_CanNotifyTSUserDef |  |  |  |
| --- | --- | --- | --- | --- |
| ASIL Level | ASIL D | Status | New |  |
| Syntax | Std_ReturnType ProtoConv_CanNotifyTSUserDef(const R_ProtoConv_CanFrameType *pCanFrame, const uint8 ucSrcBusIdx, const R_ProtoConv_TimeStampType *pTimeStamp) |  |  |  |
| Sync/Async | Synchronous |  |  |  |
| Reentrancy | Non-Reentrant |  |  |  |
| Interrupt State | Runs in the interrupt enable state |  |  |  |
| Parameters (In) | const R_ProtoConv_CanFrameType *pCanFrame |  | CAN frame |  |
|  |  |  | Range | Not NULL_PTR |
| Parameters (In) | const uint8 ucSrcBusIdx |  | Source Bus ID |  |
|  |  |  | Range | 0 - (PROTOCONV_CAN_CH_NUM-1) |
| Parameters (In) | const R_ProtoConv_TimeStampType *pTimeStamp |  | Address of buffer where timestamp is stored |  |
|  |  |  | Range | Not NULL_PTR |
| Parameters (In-Out) | N/A |  | - |  |
| Parameters (Out) | N/A |  | - |  |
| Return Value | Std_ReturnType |  | E_OK E_NOT_OK |  |
| Description | The user implements the notification process to BSW for CAN/CAN FD frame with TimeStamp in this function. If the process ends normally, E_OK is returned. If the process ends abnormally, a value other than E_OK is returned. If a value other than E_OK is returned, it is converted to E_NOT_OK by the caller. |  |  |  |
| Preconditions | N/A |  |  |  |
| Remarks | N/A |  |  |  |
| DET/DEM Errors Handled | N/A |  |  |  |

[Covers: req-8552172-00903613]

[Covers: req-8552172-00905238]

[Covers: req-8552172-00909476]

[Covers: req-8552172-00909478]

[Covers: req-8552172-00528006]

[Covers: req-8552172-00527993]

### ProtoConv_CanTransmitUserDef

[Gen5GW_ProtoConv_AD_C_20_004]

Table 2-39: ProtoConv_CanTransmitUserDef

| Function Name | ProtoConv_CanTransmitUserDef |  |  |  |
| --- | --- | --- | --- | --- |
| ASIL Level | ASIL D | Status | New |  |
| Syntax | Std_ReturnType ProtoConv_CanTransmitUserDef(const R_ProtoConv_CanFrameType *pCanFrame, const uint8 ucDestBusIdx, const uint8 ucSrcBusIdx, const uint32 ulAddInfo) |  |  |  |
| Sync/Async | Synchronous |  |  |  |
| Reentrancy | Non-Reentrant |  |  |  |
| Interrupt State | Runs in the interrupt enable state |  |  |  |
| Parameters (In) | const R_ProtoConv_CanFrameType *pCanFrame |  | Address of buffer where CAN frames are stored |  |
|  |  |  | Range | Not NULL_PTR |
| Parameters (In) | const uint8 ucDestBusIdx |  | Destination Bus ID |  |
|  |  |  | Range | 0 - (PROTOCONV_CAN_CH_NUM-1) |
| Parameters (In) | const uint8 ucSrcBusIdx |  | Source Bus ID |  |
|  |  |  | Range | 0 - (PROTOCONV_CAN_CH_NUM-1) |
| Parameters (In) | const uint32 ulAddInfo |  | Additional Information |  |
|  |  |  | Range | 32-bit value |
| Parameters (In-Out) | N/A |  | - |  |
| Parameters (Out) | N/A |  | - |  |
| Return Value | Std_ReturnType |  | E_OK E_NOT_OK |  |
| Description | The user implements the CAN/CAN FD frame transmission or IPDU-M CAN/CAN FD frame process in this function. If the process ends normally, E_OK is returned. If the process ends abnormally, a value other than E_OK is returned; if a value other than E_OK is returned, the caller converts it to E_NOT_OK. |  |  |  |
| Preconditions | N/A |  |  |  |
| Remarks | N/A |  |  |  |
| DET/DEM Errors Handled | N/A |  |  |  |

[Covers: req-8552172-00903613]

[Covers: req-8552172-00905224]

[Covers: req-8552172-00909476]

[Covers: req-8552172-00909478]

[Covers: req-8552172-00528006]

[Covers: req-8552172-00527993]

### ProtoConv_CanNotifyUserDef

[Gen5GW_ProtoConv_AD_C_20_005]

Table 2-40: ProtoConv_CanNotifyUserDef

| Function Name | ProtoConv_CanNotifyTSUserDef |  |  |  |
| --- | --- | --- | --- | --- |
| ASIL Level | ASIL D | Status | New |  |
| Syntax | Std_ReturnType ProtoConv_CanNotifyUserDef(const R_ProtoConv_CanFrameType *pCanFrame, const uint8 ucSrcBusIdx) |  |  |  |
| Sync/Async | Synchronous |  |  |  |
| Reentrancy | Non-Reentrant |  |  |  |
| Interrupt State | Runs in the interrupt enable state |  |  |  |
| Parameters (In) | const R_ProtoConv_CanFrameType *pCanFrame |  | CAN frame |  |
|  |  |  | Range | Not NULL_PTR |
| Parameters (In) | const uint8 ucSrcBusIdx |  | Source Bus ID |  |
|  |  |  | Range | 0 - (PROTOCONV_CAN_CH_NUM-1) |
| Parameters (In-Out) | N/A |  | - |  |
| Parameters (Out) | N/A |  | - |  |
| Return Value | Std_ReturnType |  | E_OK E_NOT_OK |  |
| Description | The user implements the notification process to BSW for CAN/CAN FD frames in this function. If the process ends normally, E_OK is returned. If the process ends abnormally, a value other than E_OK is returned. If a value other than E_OK is returned, it is converted to E_NOT_OK by the caller. |  |  |  |
| Preconditions | N/A |  |  |  |
| Remarks | N/A |  |  |  |
| DET/DEM Errors Handled | N/A |  |  |  |

[Covers: req-8552172-00903613]

[Covers: req-8552172-00905234]

[Covers: req-8552172-00909476]

[Covers: req-8552172-00909478]

[Covers: req-8552172-00528006]

[Covers: req-8552172-00527993]

### ProtoConv_LinTransmitUserDef

[Gen5GW_ProtoConv_AD_C_20_006]

Table 2-41: ProtoConv_LinTransmitUserDef

| Function Name | ProtoConv_LinTransmitUserDef |  |  |  |
| --- | --- | --- | --- | --- |
| ASIL Level | ASIL D | Status | New |  |
| Syntax | Std_ReturnType ProtoConv_LinTransmitUserDef(const R_ProtoConv_LinFrameType *pLinFrame, const uint8 ucDestBusIdx, const uint32 ulAddInfo) |  |  |  |
| Sync/Async | Synchronous |  |  |  |
| Reentrancy | Non-Reentrant |  |  |  |
| Interrupt State | Runs in the interrupt enable state |  |  |  |
| Parameters (In) | const R_ProtoConv_LinFrameType *pLinFrame |  | Address of buffer where LIN frames are stored |  |
|  |  |  | Range | Not NULL_PTR |
| Parameters (In) | const uint8 ucDestBusIdx |  | Destination Bus ID |  |
|  |  |  | Range | 0 - (PROTOCONV_LIN_CH_NUM-1) |
| Parameters (In) | const uint32 ulAddInfo |  | Additional Information |  |
|  |  |  | Range | 32-bit value |
| Parameters (In-Out) | N/A |  | - |  |
| Parameters (Out) | N/A |  | - |  |
| Return Value | Std_ReturnType |  | E_OK E_NOT_OK |  |
| Description | The user implements the LIN frame transmission process in this function. If the process ends normally, E_OK is returned. If the process ends abnormally, a value other than E_OK is returned; if a value other than E_OK is returned, the caller converts it to E_NOT_OK. |  |  |  |
| Preconditions | N/A |  |  |  |
| Remarks | N/A |  |  |  |
| DET/DEM Errors Handled | N/A |  |  |  |

[Covers: req-8552172-00903613]

[Covers: req-8552172-00905225]

[Covers: req-8552172-00909476]

[Covers: req-8552172-00909478]

[Covers: req-8552172-00528006]

[Covers: req-8552172-00527993]

### ProtoConv_LinNotifyUserDef

[Gen5GW_ProtoConv_AD_C_20_007]

Table 2-42: ProtoConv_LinNotifyUserDef

| Function Name | ProtoConv_LinNotifyTSUserDef |  |  |  |
| --- | --- | --- | --- | --- |
| ASIL Level | ASIL D | Status | New |  |
| Syntax | Std_ReturnType ProtoConv_LinNotifyUserDef(const R_ProtoConv_LinFrameType *pLinFrame, const uint8 ucSrcBusIdx) |  |  |  |
| Sync/Async | Synchronous |  |  |  |
| Reentrancy | Non-Reentrant |  |  |  |
| Interrupt State | Runs in the interrupt enable state |  |  |  |
| Parameters (In) | const R_ProtoConv_LinFrameType *pLinFrame |  | Address of buffer where LIN frames are stored |  |
|  |  |  | Range | Not NULL_PTR |
| Parameters (In) | const uint8 ucSrcBusIdx |  | Source Bus ID |  |
|  |  |  | Range | 0 - (PROTOCONV_LIN_CH_NUM-1) |
| Parameters (In-Out) | N/A |  | - |  |
| Parameters (Out) | N/A |  | - |  |
| Return Value | Std_ReturnType |  | E_OK E_NOT_OK |  |
| Description | The user implements the notification process to BSW for LIN frames in this function. If the process ends normally, E_OK is returned. If the process ends abnormally, a value other than E_OK is returned. If a value other than E_OK is returned, it is converted to E_NOT_OK by the caller. |  |  |  |
| Preconditions | N/A |  |  |  |
| Remarks | N/A |  |  |  |
| DET/DEM Errors Handled | N/A |  |  |  |

[Covers: req-8552172-00903613]

[Covers: req-8552172-00905237]

[Covers: req-8552172-00909476]

[Covers: req-8552172-00909478]

[Covers: req-8552172-00528006]

[Covers: req-8552172-00527993]

### ProtoConv_PortTransmitUserDef

[Gen5GW_ProtoConv_AD_C_20_008]

Table 2-43: ProtoConv_PortTransmitUserDef

| Function Name | ProtoConv_PortTransmitUserDef |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| ASIL Level | ASIL D | Status | New |  |  |
| Syntax | Std_ReturnType ProtoConv_PortTransmitUserDef(const uint16 usLength, const uint8 *pPortData); |  |  |  |  |
| Sync/Async | Synchronous |  |  |  |  |
| Reentrancy | Non-Reentrant |  |  |  |  |
| Interrupt State | Runs in the interrupt enable state |  |  |  |  |
| Parameters (In) | const uint16 usLength |  |  | Data Length |  |
|  |  |  |  | Range | MIN_PORT_TOTAL_SIZE - MAX_PORT_TOTAL_SIZE |
| Parameters (In) | const uint8 *pPortData |  |  | Address of buffer where Port Data are stored |  |
|  |  |  |  | Range | Not NULL_PTR |
| Parameters (In-Out) | N/A |  |  | - |  |
| Parameters (Out) | N/A |  |  | - |  |
| Return Value | Std_ReturnType |  |  | E_OK E_NOT_OK |  |
| Description | The user implements the Port frame transmission process in this function. If the process ends normally, E_OK is returned. If the process ends abnormally, a value other than E_OK is returned; if a value other than E_OK is returned, the caller converts it to E_NOT_OK. |  |  |  |  |
| Preconditions | N/A |  |  |  |  |
| Remarks | N/A |  |  |  |  |
| DET/DEM Errors Handled | N/A |  |  |  |  |

[Covers: req-8552172-00903613]

[Covers: req-8552172-00905226]

[Covers: req-8552172-00909476]

[Covers: req-8552172-00909478]

[Covers: req-8552172-00528006]

[Covers: req-8552172-00527993]

### ProtoConv_GetTimeUserDef

[Gen5GW_ProtoConv_AD_C_20_009]

Table 2-44: ProtoConv_GetTimeUserDef

| Function Name | ProtoConv_GetTimeUserDef |  |  |  |
| --- | --- | --- | --- | --- |
| ASIL Level | ASIL D | Status | New |  |
| Syntax | Std_ReturnType ProtoConv_GetTimeUserDef(uint16 *pSecHi , uint32 *pSec, uint32 *pnSec) |  |  |  |
| Sync/Async | Synchronous |  |  |  |
| Reentrancy | Non-Reentrant |  |  |  |
| Interrupt State | Runs in the interrupt enable state |  |  |  |
| Parameters (In) | N/A |  | - |  |
| Parameters (In-Out) | N/A |  | - |  |
| Parameters (Out) | uint16 *pSecHi |  | Pointer to store acquired time (seconds: bit32- bit47) |  |
|  |  |  | Range | Not NULL_PTR |
| Parameters (Out) | uint32 *pSec |  | Pointer to store acquired time (seconds: bit0 - bit31) |  |
|  |  |  | Range | Not NULL_PTR |
| Parameters (Out) | uint32 *pnSec |  | Pointer to store acquired time (in nanoseconds) |  |
|  |  |  | Range | Not NULL_PTR |
| Return Value | Std_ReturnType |  | E_OK E_NOT_OK |  |
| Description | Returns the current timestamp. The user implements the process of obtaining the time to set it in the avtp_timestamp field of the TSCF format in this function. If the process ends successfully, E_OK is returned and the following settings are made ・avtp_timestamp field = acquired time ・tv(avtp_timestamp valid) = TRUE ・tv(avtp_timestamp valid) = TRUE ・tu(timestamp uncertain) = FALSE If the process ends abnormally, return a value other than E_OK and make the following settings. ・avtp_timestamp field = 0 ・tv(avtp_timestamp valid) = FALSE ・tv(avtp_timestamp valid) = FALSE ・tu(timestamp uncertain) = TRUE |  |  |  |
| Preconditions | N/A |  |  |  |
| Remarks | See Refer to [4] 4.4.2 AVTP EtherType for details on each field of the TSCF format. |  |  |  |
| DET/DEM Errors Handled | N/A |  |  |  |

[Covers: req-8552172-00902551]

[Covers: req-8552172-00909476]

[Covers: req-8552172-00909478]

[Covers: req-8552172-00528006]

[Covers: req-8552172-00527993]

### ProtoConv_EnterCriticalUserDef

[Gen5GW_ProtoConv_AD_C_20_010]

Table 2-45: ProtoConv_EnterCriticalUserDef

| Function Name | ProtoConv_EnterCriticalUserDef |  |  |
| --- | --- | --- | --- |
| ASIL Level | ASIL D | Status | New |
| Syntax | void ProtoConv_EnterCriticalUserDef(void) |  |  |
| Sync/Async | Synchronous |  |  |
| Reentrancy | Non-Reentrant |  |  |
| Interrupt State | Runs in the interrupt enable state |  |  |
| Parameters (In) | N/A |  | - |
| Parameters (In-Out) | N/A |  | - |
| Parameters (Out) | N/A |  | - |
| Return Value | N/A |  | - |
| Description | The user implements the interrupt inhibit process in this function. |  |  |
| Preconditions | N/A |  |  |
| Remarks | See Refer to [5] 02_Problems_MainFunction_Competition.xlsx for the introduction process. |  |  |
| DET/DEM Errors Handled | N/A |  |  |

[Covers: req-8552172-00904667]

[Covers: req-8552172-00909476]

[Covers: req-8552172-00909478]

[Covers: req-8552172-00528006]

[Covers: req-8552172-00527993]

[Covers: req-8552172-00527978]

### ProtoConv_ExitCriticalUserDef

[Gen5GW_ProtoConv_AD_C_20_011]

Table 2-46: ProtoConv_ExitCriticalUserDef

| Function Name | ProtoConv_ExitCriticalUserDef |  |  |
| --- | --- | --- | --- |
| ASIL Level | ASIL D | Status | New |
| Syntax | void ProtoConv_ExitCriticalUserDef(void) |  |  |
| Sync/Async | Synchronous |  |  |
| Reentrancy | Non-Reentrant |  |  |
| Interrupt State | Runs in the interrupt enable state |  |  |
| Parameters (In) | N/A |  | - |
| Parameters (In-Out) | N/A |  | - |
| Parameters (Out) | N/A |  | - |
| Return Value | N/A |  | - |
| Description | The user implements the interrupt permission process in this function. |  |  |
| Preconditions | N/A |  |  |
| Remarks | See Refer to [5] 02_Problems_MainFunction_Competition.xlsx for the introduction process. |  |  |
| DET/DEM Errors Handled | N/A |  |  |

[Covers: req-8552172-00904667]

[Covers: req-8552172-00909476]

[Covers: req-8552172-00909478]

[Covers: req-8552172-00528006]

[Covers: req-8552172-00527993]

[Covers: req-8552172-00527978]

## Error handling

Table 2-47 lists the Error Handling for the APIs listed in 2.4 Interfaces in this chapter. Table 2-48 shows a list of Error Handling for 2.4.14 Interfaces (User defined).

### Interfaces

Table 2-47: Error Handling List (Interfaces)

| No. | Error Name | Error Factor | Error Handling |
| --- | --- | --- | --- |
|  | E_OK | Successful Completion | - |
|  | E_NOT_OK | Unsuccessful Completion | - |
|  | PROTOCONV_E_PARAM_POINTER | Parameter error (argument is NULL_PTR) | Returns the error to the user application that calls this API interfaces. Error handling is performed by the user application. |
|  | PROTOCONV_E_UNINIT | Process error (API executed in uninitialized state) |  |
|  | PROTOCONV_E_INV_PARAM | Invalid parameter passed to the APIs |  |
|  | PROTOCONV_E_ALREADY_INITIALIZED | Invalid Call Error |  |
|  | PROTOCONV_E_MATCH | Unmatched Table Error (Routing Table) |  |
|  | PROTOCONV_E_TRANS | Transmit Error |  |
|  | PROTOCONV_E_PARAM_DATA_LENGTH | Format Error |  |
|  | PROTOCONV_E_PACKEDCAN | Packed CAN Frame Error |  |
|  | PROTOCONV_E_PACKEDLIN | Packed LIN Frame Error |  |
|  | PROTOCONV_E_PACKEDPORT | Packed Parallel Port Frame Error |  |
|  | PROTOCONV_E_PACKEDMSG | Packed CAN/LIN/Parallel Port Frame Error |  |
|  | PROTOCONV_E_IPCKSUM | IP Header Checksum Error |  |
|  | PROTOCONV_E_UDPCKSUM | UDP Header Checksum Error |  |
|  | PROTOCONV_E_ BSWNOTIFY | BSW Notification Error |  |
|  | PROTOCONV_E_FILTER | Filtered and discarded |  |
|  | PROTOCONV_E_PARAM_CONFIG | Config Error (NULL Pointer) |  |
|  | PROTOCONV_E_CFG_ACFTYPE | Incorrect or non-supported ACF type |  |

### Interfaces (User defined)

Table 2-48 : Error Handling List (User defined interfaces)

| No. | Error Name | Error Factor | Error Handling |
| --- | --- | --- | --- |
| (1) | E_OK | Successful Completion | - |
| (2) | E_NOT_OK | User defined function Error | Returns the error to the user application that calls this API interfaces. Error handling is performed by the user application. |

## Data Design

### Data Type Definition

#### R_ProtoConv_CfgType

Table 2-49: R_ProtoConv_CfgType

| Location | CDD_ProtoConv.h |  |
| --- | --- | --- |
| 1st Element | const R_ProtoConv_CantoEthMainTblGeneralType pCantoEthMainTblGeneral | CAN to Ethernet Main Routing Table General Configuration Range: Not NULL_PTR |
| 2nd Element | const R_ProtoConv_LintoEthMainTblGeneralType pLintoEthMainTblGeneral | LIN to Ethernet Main Routing Table General Configuration Range: Not NULL_PTR |
| 3rd Element | const R_ProtoConv_XtoEthDestTblType *pXtoEthDestTbl | Ethernet Destination Routing Table Configuration Range: Not NULL_PTR |
| 4th Element | const R_ProtoConv_EthSrcCfgType *pEthSrcCfg | Ethernet Source Address/Port Configuration Range: Not NULL_PTR |
| 5th Element | const R_ProtoConv_EthtoCanMainTblType *pEthtoCanMainTbl | Ethernet to CAN Main Routing Table Configuration Range: Not NULL_PTR |
| 6th Element | const R_ProtoConv_EthtoLinMainTblType *pEthtoLinMainTbl | Ethernet to LIN Main Routing Table Configuration Range: Not NULL_PTR |
| 7th Element | uint32 ulIntegrityCheck | Integrity check code Range: PROTOCONV_CHECK_CODE |
| 8th Element | const R_ProtoConv_ConvCfgType *pConvCfg | CAN to Ethernet Conversion Type Configuration Range: Not NULL_PTR |
| 9th Element | const R_ProtoConv_BufCfgType *pBufCfg | Address and Size for Buffer Configuration Range: Not NULL_PTR |
| 10th Element | const R_ProtoConv_FilterCfgType *pFilterCfg | Ethernet to CAN Filtering Configuration Range: Not NULL_PTR |
| 11th Element | const R_ProtoConv_PduEthtoXMainTblType *pPduEthtoXMainTbl | IPDU-M Ethernet to CAN/LIN/Port Main Routing Table Configuration Range: Not NULL_PTR |
| Description | Protocol Conversion Initialization Configuration |  |

#### R_ProtoConv_CantoEthSubTblType

Table 2-50: R_ProtoConv_CantoEthSubTblType

| Location | CDD_ProtoConv.h |  |
| --- | --- | --- |
| 1st Element | uint32 ulDestIdx | Destination Index Range: 0 – (XTOETH_DEST_TABLE_ENTRY_NUM-1) |
| 2nd Element | R_ProtoConv_FrameType enTypeFlags | Frame Type Range: PROTOCONV_NORMAL_FRAME or PROTOCONV_EMERGENCY_FRAME |
| 3rd Element | uint32 ulPduId | PDU ID Range: 32-bit value |
| Description | Can to Ethernet Sub Routing Table Configuration |  |

#### R_ProtoConv_CantoEthMainTblType

Table 2-51: R_ProtoConv_CantoEthMainTblType

| Location | CDD_ProtoConv.h |  |
| --- | --- | --- |
| 1st Element | uint32 ulCanId | CAN ID Range: 0x0 – 0x7FF(CAN base format), 0x80000000 – 0x9FFFFFFF(CAN extended format) 0x40000000 – 0x400007FF (CANFD base format), 0xC0000000 – 0xDFFFFFFF(CANFD extended format |
| 2nd Element | uint32 ulCantoEthSubTblEntryNum | CAN to Ethernet Sub Routing Table Entry Number Range: 1 - 256 |
| 3rd Element | const R_ProtoConv_CantoEthSubTblType *pCantoEthSubTbl | Address of the specified CAN to Ethernet Sub Routing Table Range: Not NULL PTR |
| Description | Can to Ethernet Main Routing Table Configuration |  |

#### R_ProtoConv_CantoEthMainTblGeneralType

Table 2-52: R_ProtoConv_CantoEthMainTblGeneralType

| Location | CDD_ProtoConv.h |  |
| --- | --- | --- |
| 1st Element | uint32 ulCantoEthMainTblEntryNum | CAN to Ethernet Main Routing Table Entry Number Range: 1 - 256 |
| 2nd Element | const R_ProtoConv_CantoEthMainTblType *pCantoEthMainTbl | Address of CAN to Ethernet Main Routing Table Range: Not NULL_PTR |
| Description | Can to Ethernet Main Routing Table General Configuration |  |

#### R_ProtoConv_LintoEthSubTblType

Table 2-53: R_ProtoConv_LintoEthSubTblType

| Location | CDD_ProtoConv.h |  |
| --- | --- | --- |
| 1st Element | uint32 ulDestIdx | Destination Index Range: 0 – (XTOETH_DEST_TABLE_ENTRY_NUM-1) |
| 2nd Element | R_ProtoConv_FrameType enTypeFlags | Frame Type Range: PROTOCONV_NORMAL_FRAME or PROTOCONV_EMERGENCY_FRAME |
| 3rd Element | uint32 ulPduId | PDU ID Range: 32-bit value |
| Description | Lin to Ethernet Sub Routing Table Configuration |  |

#### R_ProtoConv_LintoEthMainTblType

Table 2-54: R_ProtoConv_LintoEthMainTblType

| Location | CDD_ProtoConv.h |  |
| --- | --- | --- |
| 1st Element | uint8 ucPId | LIN ID Range: 0x0 – 0x3F |
| 2nd Element | uint32 ulLintoEthSubTblEntryNum | LIN to Ethernet Sub Routing Table Entry Number Range: 1 - 256 |
| 3rd Element | const R_ProtoConv_LintoEthSubTblType *pLintoEthSubTbl | Address of the specified LIN to Ethernet Sub Routing Table Range: Not NULL PTR |
| Description | Lin to Ethernet Main Routing Table Configuration |  |

#### R_ProtoConv_LintoEthMainTblGeneralType

Table 2-55: R_ProtoConv_LintoEthMainTblGeneralType

| Location | CDD_ProtoConv.h |  |
| --- | --- | --- |
| 1st Element | uint32 ulLintoEthMainTblEntryNum | LIN to Ethernet Main Routing Table Entry Number Range: 1 - 256 |
| 2nd Element | const R_ProtoConv_LintoEthMainTblType *pLintoEthMainTbl | Address of LIN to Ethernet Main Routing Table Range: Not NULL_PTR |
| Description | Lin to Ethernet Main Routing Table General Configuration |  |

#### R_ProtoConv_XtoEthDestTblType

Table 2-56: R_ProtoConv_XtoEthDestTblType

| Location | CDD_ProtoConv.h |  |
| --- | --- | --- |
| 1st Element | uint32 aaDestMacAddr[PROTOCONV_MACADDR_LEN_IN_WORD] | MAC Address Range: 2 elements of 32-bit values |
| 2nd Element | R_ProtoConv_HeaderType stHeader | IP/UDP/AVTP Header Configuration Range: Refer to 2.7.1.17 |
| Description | Ethernet Destination Routing Table Configuration |  |

#### R_ProtoConv_EthSrcCfgType

Table 2-57: R_ProtoConv_EthSrcCfgType

| Location | CDD_ProtoConv.h |  |
| --- | --- | --- |
| 1st Element | uint32 aaSrcMacAddr[PROTOCONV_MACADDR_LEN_IN_WORD] | Reserved. Set to 0. Because Source MAC address is set by MCAL. |
| 2nd Element | uint32 ulSrcIpv4Addr | Source IPv4 Address Range: 32-bit value |
| 3rd Element | uint32 aaSrcIpv6Addr[PROTOCONV_IPADDR_LEN_IN_WORD] | Source IPv6 Address Range: 32-bit value 4 elements |
| 4th Element | uint16 usSrcUdpPort | Source UDP Port Range: 16-bit value |
| Description | Protocol Converter Source Address/Port Configuration |  |

#### R_ProtoConv_EthtoXSubTblType

Table 2-58: R_ProtoConv_ EthtoXSubTblType

| Location | CDD_ProtoConv.h |  |
| --- | --- | --- |
| 1st Element | uint8 ucDestBusIdx | Destination Bus ID Range: For CAN: 0 – (PROTOCONV_CAN_CH_NUM-1) For LIN: 0 – (PROTOCONV_LIN_CH_NUM-1) |
| 2nd Element | uint32 ulAddInfo | Additional Information Range: 32-bit value |
| Description | Ethernet to CAN/LIN Sub Routing Table Configuration |  |

#### R_ProtoConv_PduEthtoXSubTblType

Table 2-59: R_ProtoConv_PduEthtoXSubTblType

| Location | CDD_ProtoConv.h |  |
| --- | --- | --- |
| 1st Element | uint32 ulNewId | New ID Range: For CAN: 0x0 – 0x7FF(CAN base format), 0x80000000 – 0x9FFFFFFF(CAN extended format), 0x40000000 – 0x400007FF (CANFD base format), 0xC0000000 – 0xDFFFFFFF (CANFD extended format) For LIN: 0x0 – 0x3F |
| 2nd Element | R_ProtoConv_PduEthtoXType enPduConvMsn | Converted Module Type in IPDU-M Ethernet to CAN/LIN Conversion Range: PROTOCONV_PDUETHTOCAN or PROTOCONV_PDUETHTOLIN |
| 3rd Element | uint8 ucDestBusIdx | Destination Bus ID Range: For CAN: 0 – (PROTOCONV_CAN_CH_NUM-1) For LIN: 0 – (PROTOCONV_LIN_CH_NUM-1) |
| 4th Element | uint32 ulAddInfo | Additional Information Range: 32-bit value |
| Description | IPDU-M Ethernet to CAN/LIN Sub Routing Table Configuration |  |

#### R_ProtoConv_EthtoCanMainTblType

Table 2-60: R_ProtoConv_EthtoCanMainTblType

| Location | CDD_ProtoConv.h |  |
| --- | --- | --- |
| 1st Element | uint32 ulCanId | CAN ID Range: 0x0 – 0x7FF(CAN base format), 0x80000000 – 0x9FFFFFFF(CAN extended format) 0x40000000 – 0x400007FF (CANFD base format), 0xC0000000 – 0xDFFFFFFF(CANFD extended format |
| 2nd Element | uint32 ulEthtoCanSubTblEntryNum | Ethernet to CAN Sub Routing Table Entry Number Range: 1 - 256 |
| 3rd Element | const R_ProtoConv_EthtoXSubTblType *pEthtoCanSubTbl | Address of the specified Ethernet to CAN Sub Routing Table Range: Not NULL PTR |
| Description | Ethernet to CAN Main Routing Table Configuration |  |

#### R_ProtoConv_EthtoLinMainTblType

Table 2-61: R_ProtoConv_EthtoLinMainTblType

| Location | CDD_ProtoConv.h |  |
| --- | --- | --- |
| 1st Element | uint8 ucPId | LIN ID Range: 0x0 – 0x3F |
| 2nd Element | uint32 ulEthtoLinSubTblEntryNum | Ethernet to LIN Sub Routing Table Entry Number Range: 1 - 256 |
| 3rd Element | const R_ProtoConv_EthtoXSubTblType *pEthtoLinSubTbl | Address of the specified Ethernet to LIN Sub Routing Table Range: Not NULL PTR |
| Description | Ethernet to LIN Main Routing Table Configuration |  |

#### R_ProtoConv_PduEthtoXMainTblType

Table 2-62: R_ProtoConv_PduEthtoXMainTblType

| Location | CDD_ProtoConv.h |  |
| --- | --- | --- |
| 1st Element | uint32 ulPduId | PDU ID Range: 32-bit value |
| 2nd Element | uint32 ulPduEthtoXTblEntryNum | IPDU-M Ethernet to CAN/LIN/Port Routing Table Entry Number Range: 1 - 256 |
| 3rd Element | const R_ProtoConv_PduEthtoXSubTblType *pPduEthtoXSubTbl | Address of the specified IPDU-M Ethernet to CAN/LIN Sub Routing Table Range: Not NULL PTR |
| Description | IPDU-M Ethernet to CAN/LIN Routing Table Configuration |  |

#### R_ProtoConv_ConvCfgType

Table 2-63: R_ProtoConv_ConvCfgType

| Location | CDD_ProtoConv.h |  |
| --- | --- | --- |
| 1st Element | R_ProtoConv_ConvModeType enConvMode | Conversion Mode Range: PROTOCONV_BASICMODE or PROTOCONV_BUFFERMODE |
| 2nd Element | uint32 ulFrameLimit | Frame Limit Range(Basic Type): NUM0(Fix) Range(Buffer Type): 1U – 186U |
| 3rd Element | uint32 ulMixTimeLimit | Time Limit (used for CAN/LIN/Parallel Port transmission) Range(Basic Type): NUM0(Fix) Range(Buffer Type): 1U – 100U |
| Description | Protocol Converter Configuration. |  |

#### R_ProtoConv_BufCfgType

Table 2-64: R_ProtoConv_BufCfgType

| Location | CDD_ProtoConv.h |  |
| --- | --- | --- |
| 1st Element | uint8 *pCanTxAddr | Address of CAN Tx Buffer for CAN to Ethernet Conversion Range: 8 bit value (Refer to 2.7.1) |
| 2nd Element | uint32 ulCanTxBufSize | Size of CAN Tx Buffer for CAN to Ethernet Conversion Range: CANRXBUFF_SIZE_MAX (Fixed) |
| 3rd Element | uint8 *pCanRxAddr | Address of Can Rx Buffer for CAN to Ethernet Conversion Range: 8 bit value (Refer to 2.7.1) |
| 4th Element | uint32 ulCanRxBufSize | Size of Can Rx Buffer for CAN to Ethernet Conversion Range: CANBUF_SIZE_MIN – CANRXBUFF_SIZE_MAX |
| 5th Element | uint8 *pLinTxAddr | Address of LIN Tx Buffer for LIN to Ethernet Conversion Range: 8 bit value (Refer to 2.7.1) |
| 6th Element | uint32 ulLinTxBufSize | Size of LIN Tx Buffer for LIN to Ethernet Conversion Range: LINRXBUFF_SIZE_MAX (Fixed) |
| 7th Element | uint8 *pLinRxAddr | Address of LIN Rx Buffer for LIN to Ethernet Conversion Range: 8 bit value (Refer to 2.7.1) |
| 8th Element | uint32 ulLinRxBufSize | Size of LIN Rx Buffer for LIN to Ethernet Conversion Range: LINBUF_SIZE_MIN – LINRXBUFF_SIZE_MAX |
| 9th Element | uint8 *pPortTxAddr | Address of Port Tx Buffer for Port to Ethernet Conversion Range: 8 bit value (Refer to 2.7.1) |
| 10th Element | uint32 ulPortTxBufSize | Size of Port Tx Buffer for Port to Ethernet Conversion Range: CANRXBUFF_SIZE_MAX (Fixed) |
| 11th Element | uint8 *pPortRxAddr | Address of Port Rx Buffer for Port to Ethernet Conversion Range: 8 bit value (Refer to 2.7.1) |
| 12th Element | uint32 ulPortRxBufSize | Size of Port Rx Buffer for Port to Ethernet Conversion Range: CANBUF_SIZE_MIN – CANRXBUFF_SIZE_MAX |
| 13th Element | uint8 *pEthTxAddr | Address of Ethernet Tx Buffer for CAN/LIN/Port to Ethernet Conversion Range: 8 bit value (Refer to 2.7.1) |
| 14th Element | uint32 ulEthTxBufSize | Size of Ethernet Tx Buffer for CAN/LIN/Port to Ethernet Conversion Range: ETHBUF_SIZE_MIN – ETH_FRAME_SIZE_MAX |
| 15th Element | uint8 *pEthRxAddr | Address of Ethernet Rx Buffer for CAN/LIN/Port to Ethernet Conversion Range: 8 bit value (Refer to 2.7.1) |
| 16th Element | uint32 ulEthRxBufSize | Size of Ethernet Rx Buffer for CAN/LIN/Port to Ethernet Conversion Range: ETHBUF_SIZE_MIN – ETH_FRAME_SIZE_MAX |
| 17th Element | uint32 ulMixRxBufNum | Total number of Mix Rx Buffer for CAN/LIN/Port to Ethernet Conversion Range: 0 – PROTOCONV_MIXBUF_MAX |
| 18th Element | uint8 *pMixRxAddr[PROTOCONV_MIXBUF_MAX] | Address of Mix Rx Buffer for CAN/LIN/Port to Ethernet Conversion Range: 16 elements of 8 bit value (Refer to 2.7.1) |
| 19th Element | uint32 ulMixRxBufSize[PROTOCONV_MIXBUF_MAX] | Size of Mix Rx Buffer for CAN/LIN/Port to Ethernet Conversion Range: 16 elements of value in range of MIXBUF_SIZE_MIN – MIXEDRXBUFF_SIZE_MAX |
| Description | Buffer Address and Size Configuration. |  |

#### R_ProtoConv_HeaderType

Table 2-65: R_ProtoConv_HeaderType

| Location | CDD_ProtoConv.h |  |
| --- | --- | --- |
| 1st Element | uint8 ucIpSetFlag | IP/UDP Header Set Flag Range: PROTOCONV_CLR or PROTOCONV_SET |
| 2nd Element | uint8 ucIpType | IP Type (IPv4/IPv6) Range: PROTOCONV_FLAG_IPV4 or PROTOCONV_FLAG_IPV6 |
| 3rd Element | uint8 ucAvtpFormat | AVTP format type (NTSCF/TSCF) Range: PROTOCONV_FLAG_NTSCF or PROTOCONV_FLAG_TSCF or PROTOCONV_AVTP_NONE |
| 4th Element | uint32 aaDestIpAddr[PROTOCONV_IPADDR_LEN_IN_WORD] | IPv4/IPv6 Destination IP Address Range: 32-bit value 4 elements |
| 5th Element | uint16 usDestUdpPort | Destination UDP Port Range: 16-bit value |
| 6th Element | uint16 usVlanId | VLAN ID Range:12-bit value |
| 7th Element | uint8 ucStreamIdValid | Stream ID Valid Flag Range: PROTOCONV_INVALID or PROTOCONV_VALID |
| 8th Element | uint32 aaStreamId[PROTOCONV_STREAMID_LEN_IN_WORD] | Stream ID Range: 2 elements of 32-bit value |
| 9th Element | R_ProtoConv_EthFormatType enFormatType | Ethernet Format Type Range: PROTOCONV_IEEE1722_ACF or PROTOCONV_IPDUM_FORMAT |
| Description | IP/UDP/AVTP Header Configuration |  |

#### R_ProtoConv_CanFrameType

Table 2-66: R_ProtoConv_CanFrameType

| Location | CDD_ProtoConv.h |  |
| --- | --- | --- |
| 1st Element | uint32 ulCanId | CAN ID (32 bit) (Extend format CAN ID(29 bit) + IDE Flag + FDF bit) Range: 0x0 – 0x7FF(CAN base format), 0x80000000 – 0x9FFFFFFF(CAN extended format), 0x40000000 – 0x400007FF (CANFD base format), 0xC0000000 – 0xDFFFFFFF (CANFD extended format) |
| 2nd Element | uint32 ulCanDlc | CAN DLC (32 bit) (AUTOSAR IPDU-M LONG Format) Range: 0 - 8, 12, 16, 20, 24, 32, 48, 64 |
| 3rd Element | uint8 *pCanData | CAN Payload (Address of the buffer which stores CAN data) Range: 8 bit value |
| Description | Structure for CAN frame. |  |

#### R_ProtoConv_LinFrameType

Table 2-67: R_ProtoConv_LinFrameType

| Location | CDD_ProtoConv.h |  |
| --- | --- | --- |
| 1st Element | uint8 ucPId | LIN ID (8 bits) Range: 0x0 – 0x3F |
| 2nd Element | uint8 ucDlc | LIN DLC (8 bits) Range: 0 - 8 |
| 3rd Element | uint8 *pData | LIN Payload (Address of the buffer which stores LIN data) Range: 8 bit value |
| Description | Structure for LIN frame. |  |

#### R_ProtoConv_TimeStampType

Table 2-68: R_ProtoConv_TimeStampType

| Location | CDD_ProtoConv.h |  |
| --- | --- | --- |
| 1st Element | uint32 ulNanosecond | Nanosecond (32 bits) Range: 32-bit value |
| 2nd Element | uint32 ulSecond | Second (32 bits) Range: 32-bit value |
| Description | Structure for TimeStamp. |  |

#### R_ProtoConv_MacAddressType

Table 2-69: R_ProtoConv_MacAddressType

| Location | CDD_ProtoConv.h |  |
| --- | --- | --- |
| 1st Element | uint32 aaMacAddr[PROTOCONV_MACADDR_LEN_IN_WORD] | MAC Address Range: 32-bit values 2 elements |
| Description | MAC Address Type. |  |

#### R_ProtoConv_IpAddressType

Table 2-70: R_ProtoConv_IpAddressType

| Location | CDD_ProtoConv.h |  |
| --- | --- | --- |
| 1st Element | uint32 aaIpAddr[PROTOCONV_IPADDR_LEN_IN_WORD] | IP Address Range: 32-bit value 4 elements |
| Description | IP Address Type. |  |

#### R_ProtoConv_StreamIdType

Table 2-71: R_ProtoConv_StreamIdType

| Location | CDD_ProtoConv.h |  |
| --- | --- | --- |
| 1st Element | uint32 aaStreamId[PROTOCONV_STREAMID_LEN_IN_WORD] | StreamID Range: 32-bit values 2 elements |
| Description | StreamID Type. |  |

#### R_ProtoConv_FilterCfgType

Table 2-72: R_ProtoConv_FilterCfgType

| Location | CDD_ProtoConv.h |  |
| --- | --- | --- |
| 1st Element | uint16 usSrcMacFilterNum | Number of target MAC Address Type Range:0 –200U |
| 2nd Element | const R_ProtoConv_MacAddressType *aaSrcMacAddr | Address of target MAC Address Type Range: Not NULL_PTR (200 elements) |
| 3rd Element | uint16 usSrcIpFilterNum | Number of target IP Address Type Range:0 - 200U |
| 4th Element | const R_ProtoConv_IpAddressType *aaSrcIpAddr | Address of target IP Address Type Range: Not NULL_PTR (200 elements) |
| 5th Element | uint16 usVlanIdFilterNum | Number of target VlanID Type Range:0 - 8U |
| 6th Element | const uint16 *aaVlanId | Address of target VlanID Type Range: Not NULL_PTR (8 elements of 16-bit value) |
| 7th Element | uint16 usStreamIdFilterNum | Number of target StreamID Type Range:0 - 200U |
| 8th Element | const R_ProtoConv_StreamIdType *aaStreamId | Address of target StreamID Type Range: Not NULL_PTR (200 elements) |
| Description | Filtering Configuration |  |

#### R_ProtoConv_CanFifoType

Table 2-73: R_ProtoConv_CanFifoType

| Location | CDD_ProtoConv_Fifo.h |  |
| --- | --- | --- |
| 1st Element | R_ProtoConv_CanFrameType stCanFrame[PROTOCONV_CAN_FIFO_DEPTH] | CAN frame Range: 32 elements (refer to 2.7.1.18) |
| 2nd Element | R_ProtoConv_TimeStampType stTimeStamp[PROTOCONV_CAN_FIFO_DEPTH] | TimeStamp Range: 32 elements (refer to 2.7.1.20) |
| 3rd Element | uint8 aaCanData [PROTOCONV_CAN_FIFO_DEPTH][CANFRAME_LEN_MAX] | CAN data Range: 8-bit values (refer to 2.7.1.18) |
| 4th Element | uint32 ulHead | Head value Range: 32-bit value |
| 5th Element | uint32 ulTail | Tail value Range: 32-bit value |
| 6th Element | uint32 ulFlag | FIFO flag Range: refer to 2.7.4.5 |
| Description | Structure of CAN FIFO management type |  |

#### R_ProtoConv_LinFifoType

Table 2-74: R_ProtoConv_LinFifoType

| Location | CDD_ProtoConv_Fifo.h |  |
| --- | --- | --- |
| 1st Element | R_ProtoConv_LinFrameType stLinFrame[PROTOCONV_LIN_FIFO_DEPTH] | LIN frame Range: 32 elements (refer to 2.7.1.19) |
| 2nd Element | uint8 aaLinData [PROTOCONV_LIN_FIFO_DEPTH][LINFRAME_LEN_MAX] | LIN data 8-bit values (refer to 2.7.1.19) |
| 3rd Element | uint32 ulHead | Head value Range: 32-bit value |
| 4th Element | uint32 ulTail | Tail value Range: 32-bit value |
| 5th Element | uint32 ulFlag | FIFO flag Range: refer to 2.7.4.5 |
| Description | Structure of LIN FIFO management type |  |

#### R_ProtoConv_PortFifoType

Table 2-75: R_ProtoConv_PortFifoType

| Location | CDD_ProtoConv_Fifo.h |  |
| --- | --- | --- |
| 1st Element | uint16 usLength | Data length Range: MIN_PORT_TOTAL_SIZE - MAX_PORT_TOTAL_SIZE |
| 2nd Element | uint8 aaPortData[MAX_PORT_TOTAL_SIZE] | Port data Range: 8-bit values |
| 3rd Element | uint32 ulDestIdx | Index to Ethernet Destination Routing Table Range: 0 - 255 |
| 4th Element | uint32 ulHead | Head value Range: 32-bit value |
| 5th Element | uint32 ulTail | Tail value Range: 32-bit value |
| 6th Element | uint32 ulFlag | FIFO flag Range: refer to 2.7.4.5 |
| Description | Structure of Port FIFO management type |  |

#### R_ProtoConv_EthFrameType

Table 2-76: R_ProtoConv_EthFrameType

| Location | CDD_ProtoConv_Fifo.h |  |
| --- | --- | --- |
| 1st Element | uint8 aaSrcMacAddr[MACADDR_LEN_IN_BYTE] | Address of target MAC Address Type Range: Not NULL_PTR (200 elements) |
| 2nd Element | uint16 usFrameType | Ethernet Type Range: PROTOCONV_FRAMETYPE_AVTP (0x22F0U), PROTOCONV_FRAMETYPE_IPV4 (0x0800U), PROTOCONV_FRAMETYPE_IPV6 (0x86DDU), PROTOCONV_FRAMETYPE_8021Q (0x8100U) |
| 3rd Element | uint32 ulFrameLen | Data size of Ethernet frame Range: 46 -1500 |
| 4th Element | uint8 *pEthFrame | Ethernet Payload (Address of the buffer which stores Ethernet data) Range: 8-bit values |
| Description | Structure for Ethernet frame. |  |

#### R_ProtoConv_EthFifoType

Table 2-77: R_ProtoConv_EthFifoType

| Location | CDD_ProtoConv_Fifo.h |  |
| --- | --- | --- |
| 1st Element | R_ProtoConv_EthFrameType stEthFrame[PROTOCONV_ETH_FIFO_DEPTH] | ETH frame Range: 32 elements (refer to 2.7.1.28) |
| 2nd Element | uint8 aaEthData[PROTOCONV_ETH_FIFO_DEPTH][ETH_FRAME_SIZE_MAX] | ETH data Range: 8-bit values |
| 3rd Element | uint32 ulHead | Head value Range: 32-bit value |
| 4th Element | uint32 ulTail | Tail value Range: 32-bit value |
| 5th Element | uint32 ulFlag | FIFO flag Range: refer to 2.7.4.5 |
| Description | Structure of Ethernet FIFO management type |  |

### Public Definition

Table 2-78: Public definition

| Location | CDD_ProtoConv.h |  |
| --- | --- | --- |
| define | PROTOCONV_VENDOR_ID = PROTOCONV_VENDOR_ID_VALUE | ProtoConv Vendor ID |
|  | PROTOCONV_MODULE_ID = PROTOCONV_MODULE_ID_VALUE | ProtoConv Module ID |
|  | PROTOCONV_INSTANCE_ID = PROTOCONV_INSTANCE_ID_VALUE | ProtoConv Instance ID |
|  | PROTOCONV_AR_RELEASE_MAJOR_VERSION = (4U) | ProtoConv AUTOSAR Release Major Version |
|  | PROTOCONV_AR_RELEASE_MINOR_VERSION = (8U) | ProtoConv AUTOSAR Release Minor Version |
|  | PROTOCONV_AR_RELEASE_REVISION_VERSION = (0U) | ProtoConv AUTOSAR Release Revision Version |
|  | PROTOCONV_SW_MAJOR_VERSION = (1U) | ProtoConv Software Major Version |
|  | PROTOCONV_SW_MINOR_VERSION = (0U) | ProtoConv Software Minor Version |
|  | PROTOCONV_SW_PATCH_VERSION = (0U) | ProtoConv Software Patch Version |
|  | PROTOCONV_PBTYPES_AR_RELEASE_MAJOR_VERSION = PROTOCONV_AR_RELEASE_MAJOR_VERSION | ProtoConv Postbuilt Type AUTOSAR Release Major Version |
|  | PROTOCONV_PBTYPES_AR_RELEASE_MINOR_VERSION = PROTOCONV_AR_RELEASE_MINOR_VERSION | ProtoConv Postbuilt Type AUTOSAR Release Minor Version |
|  | PROTOCONV_PBTYPES_AR_RELEASE_REVISION_VERSION = PROTOCONV_AR_RELEASE_REVISION_VERSION | ProtoConv Postbuilt Type AUTOSAR Release Revision Version |
|  | PROTOCONV_PBTYPES_SW_MAJOR_VERSION = PROTOCONV_SW_MAJOR_VERSION | ProtoConv Postbuilt Type Software Release Major Version |
|  | PROTOCONV_PBTYPES_SW_MINOR_VERSION = PROTOCONV_SW_MINOR_VERSION | ProtoConv Postbuilt Type Software Release Minor Version |
|  | PROTOCONV_CHECK_CODE = (0x12345678UL) | Integrity check code |
|  | PROTOCONV_FRAMETYPE_AVTP = (0x22F0U) | AVTP Frame Type |
|  | PROTOCONV_FRAMETYPE_IPV4 = (0x0800U) | IPv4 Frame Type |
|  | PROTOCONV_FRAMETYPE_IPV6 = (0x86DDU) | IPv6 Frame Type |
|  | PROTOCONV_FRAMETYPE_8021Q = (0x8100U) | 8021Q Frame Type |
|  | PROTOCONV_MACADDR_LEN_IN_WORD = (2U) | MAC Address length in word |
|  | PROTOCONV_IPADDR_LEN_IN_WORD = (4U) | IPv4/Ipv6 Address length in word. |
|  | PROTOCONV_STREAMID_LEN_IN_WORD = (2U) | Stream ID length in word |
|  | PROTOCONV_IPV4ADDR_LEN_IN_BYTE = (4U) | IPv4 Address length in Byte. |
|  | PROTOCONV_IPV6ADDR_LEN_IN_BYTE = (16U) | IPv6 Address length in Byte. |
|  | PROTOCONV_CLR = (0U) | Flag Clear |
|  | PROTOCONV_SET = (1U) | Flag Set |
|  | PROTOCONV_INVALID = (0U) | Invalid flag |
|  | PROTOCONV_VALID = (1U) | Valid flag |
|  | PROTOCONV_FLAG_IPV4 = (0U) | IPv4 flag |
|  | PROTOCONV_FLAG_IPV6 = (1U) | IPv6 flag |
|  | PROTOCONV_FLAG_NTSCF = (0U) | NTSCF flag |
|  | PROTOCONV_FLAG_TSCF = (1U) | TSCF flag |
|  | PROTOCONV_AVTP_NONE = (2U) | AVTP exclude flag |
|  | PROTOCONV_MIXBUF_MAX = MIX_RX_BUF_NUM | The maximum number of Mix Buffers |
|  | R_PROTOCONV_INIT_SID = (uint8)0x00U | Service ID for R_ProtoConv_Init |
|  | R_PROTOCONV_CANTOETHTS_SID = (uint8)0x01U | Service ID for R_ProtoConv_CantoEthTS |
|  | R_PROTOCONV_CANTOETH_SID = (uint8)0x02U | Service ID for R_ProtoConv_CantoEth |
|  | R_PROTOCONV_LINTOETH_SID = (uint8)0x03U | Service ID for R_ProtoConv_LintoEth |
|  | R_PROTOCONV_PORTTOETH_SID = (uint8)0x04U | Service ID for R_ProtoConv_PorttoEth |
|  | R_PROTOCONV_TRIGGERXTOETHTX_SID = (uint8)0x05U | Service ID for R_ProtoConv_TriggerXtoEthTx |
|  | R_PROTOCONV_MAINFUNCTION_SID = (uint8)0x06U | Service ID for R_ProtoConv_MainFunction |
|  | R_PROTOCONV_ETHTOX_SID = (uint8)0x07U | Service ID for R_ProtoConv_EthtoX |
|  | R_PROTOCONV_GETSENDSTATUS_SID = (uint8)0x08U | Service ID for R_ProtoConv_GetSendStatus |
|  | R_PROTOCONV_UPDATESENDSTATUS_SID = (uint8)x09U | Service ID for R_ProtoConv_UpdateSendStatus |
|  | R_PROTOCONV_ENQUEUECAN_SID = (uint8)x0AU | Service ID for R_ProtoConv_EnqueueCan |
|  | R_PROTOCONV_ENQUEUELIN_SID = (uint8)x0BU | Service ID for R_ProtoConv_EnqueueLin |
|  | R_PROTOCONV_ENQUEUEPORT_SID = (uint8)x0CU | Service ID for R_ProtoConv_EnqueuePort |
|  | R_PROTOCONV_ENQUEUEETH_SID = (uint8)x0DU | Service ID for R_ProtoConv_EnqueueEth |
|  | R_PROTOCONV_DEQUEUEETH_SID = (uint8)x0EU | Service ID for R_ProtoConv_DequeueEth |
|  | R_PROTOCONV_TRIGGERQUEUEDATAXTOETH_SID = (uint8)x0FU | Service ID for R_ProtoConv_TriggerQueueDataXtoEth |
| Description | Protocol Global Macro Definitions |  |

### Error code definition

Table 2-79: Error code definition

| Location | Std_Types.h |  |
| --- | --- | --- |
| define | E_OK = (0U) | Successful Completion |
|  | E_NOT_OK = (1U) | Unsuccessful Completion |
| Location | CDD_ProtoConv.h |  |
| define | PROTOCONV_E_PARAM_POINTER = (uint8)0x02U /* 2U */ | Null Parameter Error (Null Pointer) |
|  | PROTOCONV_E_UNINIT = (uint80x03U /* 3U */ | Initialization API unexecuted |
|  | PROTOCONV_E_INV_PARAM = (uint8)0x04U /* 4U */ | Invalid Parameter Error |
|  | PROTOCONV_E_ALREADY_INITIALIZED = (uint8)0x05U /* 5U */ | Invalid Call |
|  | PROTOCONV_E_MATCH = (uint8)0x06U /* 6U */ | Unmatched Routing Table Error |
|  | PROTOCONV_E_TRANS = (uint8)0x07U /* 7U */ | Transmit error |
|  | PROTOCONV_E_PARAM_DATA_LENGTH = (uint8)0x08U /* 8U*/ | Format Error |
|  | PROTOCONV_E_PACKEDCAN = (uint8)0x09U /* 9U*/ | Packed CAN Frame Error |
|  | PROTOCONV_E_PACKEDLIN = (uint8)0x0AU /* 10U*/ | Packed LIN Frame Error |
|  | PROTOCONV_E_PACKEDPORT = (uint8)0x0BU /* 11U*/ | Packed Port Frame Error |
|  | PROTOCONV_E_PACKEDMSG = (uint8)0x0CU /* 12U*/ | Packed CAN/LIN/Port Frame Error |
|  | PROTOCONV_E_IPCKSUM = (uint8)0x0DU /* 13U*/ | IP Header Checksum Error |
|  | PROTOCONV_E_UDPCKSUM = (uint8)0x0EU /* 14U*/ | UDP Header Checksum Error |
|  | PROTOCONV_E_NOTFRAME = (uint8)0x0FU /* 15U*/ | No CAN/LIN/Port Frame Error |
|  | PROTOCONV_E_BSWNOTIFY = (uint80x010U /* 16U*/ | BSW Notification Error |
|  | PROTOCONV_E_FILTER = (uint8)0x11U /* 17U*/ | CAN/LIN/Port frame Discarding Error |
|  | PROTOCONV_E_PARAM_CONFIG = (uint8)0x12U /* 18U*/ | Config Error (NULL Pointer) |
|  | PROTOCONV_E_FIFO_FULL = (uint8)0x13U /* 19U* | Unexpectedly full FIFO memory error |
|  | PROTOCONV_E_FIFO_EMPTY = (uint8)0x14U /* 20U* | Unexpectedly empty FIFO memory error |
|  | PROTOCONV_E_CFG_ACFTYPE = (uint8)0x15U /* 21U* | Incorrect or non-supported ACF type |
| Description | Definition (Error code) |  |

### Enumeration

#### R_ProtoConv_ConvModeType

Table 2-80: R_ProtoConv_ConvModeType

| Location | CDD_ProtoConv.h |  |
| --- | --- | --- |
| define | PROTOCONV_BASICMODE = (0U) | Basic Type |
|  | PROTOCONV_BUFFERMODE = (1U) | Buffer Type |
| Description | Protocol Conversion Type Enumeration in CAN/LIN/Port to Ethernet Conversion |  |

#### R_ProtoConv_FrameType

Table 2-81: R_ProtoConv_FrameType

| Location | CDD_ProtoConv.h |  |
| --- | --- | --- |
| define | PROTOCONV_NORMAL_FRAME = (0U) | Normal Frame Type |
|  | PROTOCONV_EMERGENCY_FRAME = (1U) | Emergency Frame Type |
| Description | Frame Type Enumeration in CAN/LIN/Port to Ethernet Conversion |  |

#### R_ProtoConv_EthFormatType

Table 2-82: R_ProtoConv_EthFormatType

| Location | CDD_ProtoConv.h |  |
| --- | --- | --- |
| define | PROTOCONV_IEEE1722_ACF = (0U) | ACF Frame Type |
|  | PROTOCONV_IPDUM_FORMAT = (1U) | IPDU-M Frame Type |
| Description | Ethernet Frame Type Enumeration in Ethernet to CAN/LIN/Port Conversion |  |

#### R_ProtoConv_PduEthtoXType

Table 2-83: R_ProtoConv_PduEthtoXType

| Location | CDD_ProtoConv.h |  |
| --- | --- | --- |
| define | PROTOCONV_PDUETHTOCAN = (0U) | PDU Ethernet Frame to CAN Conversion Type |
|  | PROTOCONV_PDUETHTOLIN = (1U) | PDU Ethernet Frame to LIN Conversion Type |
| Description | Converted Module Type Enumeration in IPDU-M Ethernet to CAN/LIN Conversion |  |

#### R_ProtoConv_FifoFlagType

Table 2-84: R_ProtoConv_FifoFlagType

| Location | CDD_ProtoConv_Fifo.c |  |
| --- | --- | --- |
| define | PROTOCONV_FIFO_EMPTY = (0U) | Empty FIFO memory |
|  | PROTOCONV_FIFO_AVAILABLE = (1U) | Available FIFO memory |
|  | PROTOCONV_FIFO_FULL =(2U) | Full FIFO memory |
| Description | FIFO Flag Type Enumeration in FIFO memory management |  |

# Sub-Component Design

## Initialization Function Design

### Features

[iso26-18_p6_7.4.4]; [iso26-18_p6_7.4.5] a

Table 3-1: Function List

| No. | Interface Name | ASIL level | Description |
| --- | --- | --- | --- |
| (1) | ProtoConv_Init | ASIL D | Initialize Protocol Conversion. |

### Interfaces

#### ProtoConv_Init

[Gen5GW_ProtoConv_AD_C_11_001]

Table 3-2: ProtoConv_Init

| Function Name | ProtoConv_Init |  |  |  |
| --- | --- | --- | --- | --- |
| ASIL Level | ASIL D | Status | New |  |
| Syntax | void ProtoConv_Init(const R_ProtoConv_CfgType *pConfig) |  |  |  |
| Sync/Async | Synchronous |  |  |  |
| Reentrancy | Non-Reentrant |  |  |  |
| Interrupt State | Runs in the interrupt enable state |  |  |  |
| Parameters (In) | const R_ProtoConv_CfgType *pConfig |  | Pointer to initialization config. |  |
|  |  |  | Range | Not NULL_PTR |
| Parameters (In-Out) | N/A |  | - |  |
| Parameters (Out) | N/A |  | - |  |
| Return Value | N/A |  |  |  |
| Description | If the member ulIntegrityCheck of the argument pConfig is invalid, return E_NOT_OK, report DET error PROTOCONV_E_CFG (if PROTOCONV_DEV_ERROR_DETECT == STD_ON) and exit. If any of the member pointers of argument pConfig is NULL_PTR, return E_NOT_OK, report DET error PROTOCONV_E_PARAM_CONFIG (if PROTOCONV_DEV_ERROR_DETECT == STD_ON) and exit. If E_OK is returned, execute ProtoConv_XtoEthInit() and return its return value. If E_OK is returned, execute ProtoConv_EthtoXInit() and return its return value. If E_OK is returned, execute ProtoConv_FifoInit() and end the process. |  |  |  |
| Preconditions | Uninitialized state |  |  |  |
| Remarks | N/A |  |  |  |
| DET/DEM Errors Handled | N/A |  |  |  |

[Covers: req-8552172-00903609]

[Covers: req-8552172-00904663]

[Covers: req-8552172-00904664]

[Covers: req-8552172-00904665]

[Covers: req-8552172-00904666]

[Covers: req-8552172-00909476]

[Covers: req-8552172-00909478]

[Covers: req-8552172-00528003]

[Covers: req-8552172-00528002]

[Covers: req-8552172-00528006]

[Covers: req-8552172-00527993]

[Covers: req-8552172-00527980]

[Covers: req-8552172-00909163]

[Covers: req-8552172-00909164]

### Error handling

[iso26-18_p6_7.4.12]

Table 3-3: Error Handling List

| No. | Error Name | Error Factor | Error Handling |
| --- | --- | --- | --- |
| (1) | E_OK | Successful Completion | - |
| (2) | E_NOT_OK | Unsuccessful Completion | - |

## CAN/LIN/Port to Ethernet Conversion Function Design

### Features

Table 3-4: Function List

| No. | Interface Name | ASIL level | Description |
| --- | --- | --- | --- |
|  | ProtoConv_XtoEthInit | ASIL D | Initialize CAN/LIN/Port to Ethernet Conversion Function. |
|  | ProtoConv_CantoEthTS | ASIL D | Convert CAN/CAN FD frame with TimeStamp to Ethernet frame. |
|  | ProtoConv_CantoEth | ASIL D | Convert CAN/CAN FD frame to Ethernet frame. |
|  | ProtoConv_LintoEth | ASIL D | Convert LIN frame to Ethernet frame. |
|  | ProtoConv_PorttoEth | ASIL D | Convert Port frame to Ethernet frame. |
|  | ProtoConv_TriggerXtoEthTx | ASIL D | Trigger CAN/LIN/Port to Ethernet Transmission |
|  | ProtoConv_MixtoEthMain | ASIL D | Main Function for polling. |
|  | ProtoConv_EnqueueCan | ASIL D | Enqueue CAN message to the pre-defined memory. |
|  | ProtoConv_EnqueueLin | ASIL D | Enqueue LIN message to the pre-defined memory. |
|  | ProtoConv_EnqueuePort | ASIL D | Enqueue Port message to the pre-defined memory. |
|  | ProtoConv_TriggerQueueDataXtoEth | ASIL D | Trigger the queued CAN/LIN/Port to Ethernet transmit. |
|  | ProtoConv_CreateAndSendEthFrame | ASIL D | Create and send the converted Ethernet frame. |
|  | ProtoConv_StorePdutoEthBufData | ASIL D | Store CAN/LIN/Port frames in IPDU-M format to Buffers. |

### Interfaces

#### ProtoConv_XtoEthInit

[Gen5GW_ProtoConv_AD_C_12_001]

Table 3-5: ProtoConv_XtoEthInit

| Function Name | ProtoConv_XtoEthInit |  |  |  |
| --- | --- | --- | --- | --- |
| ASIL Level | ASIL D | Status | New |  |
| Syntax | void ProtoConv_XtoEthInit(const R_ProtoConv_CfgType *pConfig) |  |  |  |
| Sync/Async | Synchronous |  |  |  |
| Reentrancy | Non-Reentrant |  |  |  |
| Interrupt State | Runs in the interrupt enable state |  |  |  |
| Parameters (In) | const R_ProtoConv_CfgType *pConfig |  | Pointer to initialization config. |  |
|  |  |  | Range | Not NULL_PTR |
| Parameters (In-Out) | N/A |  | - |  |
| Parameters (Out) | N/A |  | - |  |
| Return Value | N/A |  |  |  |
| Description | The values of CAN/LIN to Ethernet Routing Table and Ethernet Destination Table are saved. The set value of the argument pConfig is saved. The values of the address and size of the send/receive buffer area of the member of argument pConfig initialize CAN Rx Buffer and Ethernet Tx Buffer for CAN to Ethernet conversion. Initialize buffering time and Ethernet destination information by the value of the member of pConfig argument. Index sequence numbers. Initialize Sequence Number for AVTP Header and Encapsulation Sequence Number. If the process ends normally, E_OK is returned. |  |  |  |
| Preconditions | Uninitialized state |  |  |  |
| Remarks | N/A |  |  |  |
| DET/DEM Errors Handled | N/A |  |  |  |

[Covers: req-8552172-00903609]

[Covers: req-8552172-00905312]

[Covers: req-8552172-00909164]

[Covers: req-8552172-00909476]

[Covers: req-8552172-00909478]

[Covers: req-8552172-00528003]

[Covers: req-8552172-00528002]

[Covers: req-8552172-00528006]

[Covers: req-8552172-00527993]

[Covers: req-8552172-00527980]

#### ProtoConv_CantoEthTS

[Gen5GW_ProtoConv_AD_C_12_002]

Table 3-6: ProtoConv_CantoEthTS

| Function Name | ProtoConv_CantoEth |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| ASIL Level | ASIL D | Status | New |  |  |
| Syntax | Std_ReturnType R_ProtoConv_CantoEthTS(const uint8 ucBusIdx, const R_ProtoConv_CanFrameType *pCanFrame, const R_ProtoConv_TimeStampType *pTimeStamp) |  |  |  |  |
| Sync/Async | Synchronous |  |  |  |  |
| Reentrancy | Non-Reentrant |  |  |  |  |
| Interrupt State | Runs in the interrupt enable state |  |  |  |  |
| Parameters (In) | const uint8 ucBusIdx |  | Source Bus ID |  |  |
|  |  |  | Range | 0 - (PROTOCONV_CAN_CH_NUM-1) |  |
| Parameters (In) | const R_ProtoConv_CanFrameType *pCanFrame |  | Address of buffer where CAN frames is stored |  |  |
|  |  |  | Range | Not NULL_PTR |  |
| Parameters (In) | const R_ProtoConv_TimeStampType*pTimeStamp |  | Address of buffer where timestamp is stored |  |  |
|  |  |  | Range | Not NULL_PTR |  |
| Parameters (In-Out) | N/A |  | - |  |  |
| Parameters (Out) | N/A |  | - |  |  |
| Return Value | Std_ReturnType |  | E_OK E_NOT_OK |  |  |
| Description | usBusIdx is the index of the bus and specifies the Can to Ethernet Routing Table to be searched. usBusIdx is also used as the value to be stored in the can_bus_id field of the ACF CAN format. pTimeStamp is the timestamp to be stored in the message_timestamp field of the ACF CAN format, not used for IPDU-M format. 1-1. Get CAN frame information from Parameter pCanFrame. 1-2. Search the CAN to Ethernet Routing Table by the CAN ID of the CAN frame, and obtain the Ethernet destination information. If the CAN ID does not match the CAN to Ethernet Routing Table, returns E_NOT_OK, report DET error PROTOCONV_E_MATCH (if PROTOCONV_DEV_ERROR_DETECT == STD_ON) and process is terminated. The following processing is performed according to the CAN to Ethernet conversion method. [In case of Basic Type] 2-1. Add CAN frame to CAN Rx Buffer. 2-2. Store Ethernet destination information as Ethernet header in Ethernet Tx Buffer. 2-3. Store the data in the Can Rx Buffer in the Ethernet Tx Buffer. 2-4. Execute ProtoConv_EthTransmitUserDef(). NOTE2 2-5 END [In case of Buffer Type] [CAN frame is an Emergency frame] 2-1. Add CAN frame to CAN Rx Buffer. 2-2. Store Ethernet destination information as Ethernet header in Ethernet Tx Buffer. 2-3. Store the data in the Can Rx Buffer in the Ethernet Tx Buffer. 2-4. Execute ProtoConv_EthTransmitUserDef().NOTE2 2-5 END [CAN frame is not an Emergency frame] When Buffer overflows if adding the next CAN/CANFD frame to the Mix Rx buffer, perform Step 2-1 ~ 2-3 to send the frame that has already been stored. 2-1. Store Ethernet destination information as Ethernet header in Ethernet Tx Buffer. 2-2. Store the data in the Mix Rx Buffer to the Ethernet Tx Buffer. 2-3. Execute ProtoConv_EthTransmitUserDef(). NOTE2 3-1. Add CAN frames to the Mix Rx Buffer. When one of the transmission conditions※1 is satisfied, perform Step 3-2 ~ 3-4 to send the Ethernet frame. 3-2. Stores the acquired Ethernet destination information as an Ethernet header in the Ethernet Tx Buffer. 3-3 Store the data in the Mix Rx Buffer is stored in the Ethernet Tx Buffer. 3-4. Execute ProtoConv_EthTransmitUserDef(). NOTE2 If the process ends normally, E_OK is returned. NOTE1: When the TSCF format is generated, execute ProtoConv_GetTimeUserDef() to acquire the time used for the timestamp, and if the acquisition is successful, set the timestamp in the header information. If it fails, the timestamp invalidity flag is set in the header information. NOTE2: If the function ProtoConv_EthTransmitUserDef() returns an error, this API returns E_NOT_OK, reports DET error PROTOCONV_E_TRANS (if PROTOCONV_DEV_ERROR_DETECT == STD_ON) and exits. NOTE3: The receive buffer add and transmit processes are interrupt disabled using ProtoConv_EnterCriticalUserDef() and ProtoConv_ExitCriticalUserDef(). NOTE4: If a VLAN ID is set in the searched CAN to Ethernet Routing Table (VLAN ID≠0), the same VLAN ID is set in the Ethernet header. |  |  |  |  |
| Preconditions | Initialized state |  |  |  |  |
| Remarks | ※1 For the transmission conditions, refer to Table 2-6: Ethernet Conversion Type |  |  |  |  |
| DET/DEM Errors Handled | PROTOCONV_E_INV_PARAM |  |  |  | Invalid parameter passed to the APIs |
|  | PROTOCONV_E_MATCH |  |  |  | Unmatched Table Error (Routing Table) |
|  | PROTOCONV_E_TRANS |  |  |  | Transmit Error |

[Covers: req-8552172-00903610]

[Covers: req-8552172-00904662]

[Covers: req-8552172-00904667]

[Covers: req-8552172-00905227]

[Covers: req-8552172-00909476]

[Covers: req-8552172-00909478]

[Covers: req-8552172-00528003]

[Covers: req-8552172-00528002]

[Covers: req-8552172-00528006]

[Covers: req-8552172-00527993]

[Covers: req-8552172-00527980]

#### ProtoConv_CantoEth

[Gen5GW_ProtoConv_AD_C_12_003]

Table 3-7: ProtoConv_CantoEth

| Function Name | ProtoConv_CantoEth |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| ASIL Level | ASIL D | Status | New |  |  |
| Syntax | Std_ReturnType ProtoConv_CantoEth(const uint8 ucBusIdx, const R_ProtoConv_CanFrameType *pCanFrame) |  |  |  |  |
| Sync/Async | Synchronous |  |  |  |  |
| Reentrancy | Non-Reentrant |  |  |  |  |
| Interrupt State | Runs in the interrupt enable state |  |  |  |  |
| Parameters (In) | const uint8 ucBusIdx |  | Source Bus ID |  |  |
|  |  |  | Range |  | 0 - (PROTOCONV_CAN_CH_NUM-1) |
| Parameters (In) | const R_ProtoConv_CanFrameType *pCanFrame |  | Address of buffer where CAN frames are stored |  |  |
|  |  |  | Range |  | Not NULL_PTR |
| Parameters (In-Out) | N/A |  | - |  |  |
| Parameters (Out) | N/A |  | - |  |  |
| Return Value | Std_ReturnType |  | E_OK E_NOT_OK |  |  |
| Description | This API is a Timestamp-less version of the API ProtoConv_CantoEthTS. The differences from ProtoConv_CantoEthTS are as follows - The parameter pTimeStamp is not present. - In the case of ACF format, Store in the format of an Abbreviated CAN/CAN FD ACF message. |  |  |  |  |
| Preconditions | Initialized state |  |  |  |  |
| Remarks | ※1 For the transmission conditions, refer to Table 2-6: Ethernet Conversion Type |  |  |  |  |
| DET/DEM Errors Handled | PROTOCONV_E_INV_PARAM |  |  | Invalid parameter passed to the APIs |  |
|  | PROTOCONV_E_MATCH |  |  | Unmatched Table Error (Routing Table) |  |
|  | PROTOCONV_E_TRANS |  |  | Transmit Error |  |

[Covers: req-8552172-00903610]

[Covers: req-8552172-00904662]

[Covers: req-8552172-00904667]

[Covers: req-8552172-00905227]

[Covers: req-8552172-00909476]

[Covers: req-8552172-00909478]

[Covers: req-8552172-00528003]

[Covers: req-8552172-00528002]

[Covers: req-8552172-00528006]

[Covers: req-8552172-00527993]

[Covers: req-8552172-00527980]

#### ProtoConv_LintoEth

[Gen5GW_ProtoConv_AD_C_12_004]

Table 3-8: ProtoConv_LintoEth

| Function Name | ProtoConv_LintoEth |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| ASIL Level | ASIL D | Status | New |  |  |
| Syntax | Std_ReturnType ProtoConv_LintoEth(const uint8 ucBusIdx, const R_ProtoConv_LinFrameType *pLinFrame) |  |  |  |  |
| Sync/Async | Synchronous |  |  |  |  |
| Reentrancy | Non-Reentrant |  |  |  |  |
| Interrupt State | Runs in the interrupt enable state |  |  |  |  |
| Parameters (In) | const uint8 ucBusIdx |  | Source Bus ID |  |  |
|  |  |  | Range |  | 0 - (PROTOCONV_LIN_CH_NUM-1) |
| Parameters (In) | const R_ProtoConv_LinFrameType *pLinFrame |  | Address of buffer where LIN frames are stored |  |  |
|  |  |  | Range |  | Not NULL_PTR |
| Parameters (In-Out) | N/A |  | - |  |  |
| Parameters (Out) | N/A |  | - |  |  |
| Return Value | Std_ReturnType |  | E_OK E_NOT_OK |  |  |
| Description | usBusIdx is the index of the bus and specifies the Lin to Ethernet Routing Table to be searched. usBusIdx is also used as the value to be stored in the lin_bus_id field of the ACF LIN format. 1-1. Get LIN frame information from Parameter pLinFrame. 1-2. Search the LIN to Ethernet Routing Table by the LIN ID of the LIN frame, and obtain the Ethernet destination information. If the LIN ID does not match the LIN to Ethernet Routing Table, returns E_NOT_OK, report DET error PROTOCONV_E_MATCH (if PROTOCONV_DEV_ERROR_DETECT == STD_ON) and process is terminated. The following processing is performed according to the LIN to Ethernet conversion type. [In case of Basic Type] 2-1. Add LIN frame to LIN Rx Buffer. 2-2. Store Ethernet destination information as Ethernet header in Ethernet Tx Buffer. 2-3. Storesthe data in the LIN Rx Buffer in the Ethernet Tx Buffer. 2-4. Execute ProtoConv_EthTransmitUserDef() NOTE2 2-5. END [In case of Buffer Type] [LIN frame is an Emergency frame] 2-1. Add LIN frame to LIN Rx Buffer. 2-2. Store Ethernet destination information as Ethernet header in Ethernet Tx Buffer. 2-3. Store the data in the LIN Rx Buffer in the Ethernet Tx Buffer. 2-4. Execute ProtoConv_EthTransmitUserDef() NOTE2 2-5. END [LIN frame is not an Emergency frame] When Buffer overflows if adding the next LIN frame to the Mix Rx buffer, perform Step 2-1 ~ 2-3 to send the Ethernet frame that has already been stored. 2-1. Store the Ethernet destination information as Ethernet header in Ethernet Tx Buffer. 2-2. Store the data in the Mix Rx Buffer to the Ethernet Tx Buffer. 2-3. Execute ProtoConv_EthTransmitUserDef(). NOTE2 3-1. Add LIN frames to the Mix Rx Buffer. When one of the transmission conditions※1 is satisfied, perform Step 3-2 ~ 3- to send the Ethernet frame. 3-2. Store the Ethernet destination information as an Ethernet header in the Ethernet Tx Buffer. 3-3. Store the data in the Mix Rx Buffer is stored in the Ethernet Tx Buffer. 3-4. Execute ProtoConv_EthTransmitUserDef(). NOTE2 3-5. END If the process ends normally, E_OK is returned. NOTE1: When the TSCF format is generated, Execute ProtoConv_GetTimeUserDef() to acquire the time used for the timestamp, and if the acquisition is successful, set the timestamp in the header information. If it fails, the timestamp invalidity flag is set in the header information. NOTE2: If the function ProtoConv_EthTransmitUserDef() returns an error, this API returns E_NOT_OK, report DET error PROTOCONV_E_TRANS (if PROTOCONV_DEV_ERROR_DETECT == STD_ON) and exits. NOTE3: The receive buffer add and transmit processes are interrupt disabled using ProtoConv_EnterCriticalUserDef() and ProtoConv_ExitCriticalUserDef(). NOTE4: If a VLAN ID is set in the searched LIN to Ethernet Routing Table (VLAN ID≠0), the same VLAN ID is set in the Ethernet header. |  |  |  |  |
| Preconditions | Initialized state |  |  |  |  |
| Remarks | ※1 For the transmission conditions, refer to Table 2-10: Ethernet Conversion Type |  |  |  |  |
| DET/DEM Errors Handled | PROTOCONV_E_INV_PARAM |  |  | Invalid parameter passed to the APIs |  |
|  | PROTOCONV_E_MATCH |  |  | Unmatched Table Error (Routing Table) |  |
|  | PROTOCONV_E_TRANS |  |  | Transmit Error |  |

[Covers: req-8552172-00903611]

[Covers: req-8552172-00904662]

[Covers: req-8552172-00904667]

[Covers: req-8552172-00905227]

[Covers: req-8552172-00909476]

[Covers: req-8552172-00909478]

[Covers: req-8552172-00528003]

[Covers: req-8552172-00528002]

[Covers: req-8552172-00528006]

[Covers: req-8552172-00527993]

[Covers: req-8552172-00527980]

#### ProtoConv_PorttoEth

[Gen5GW_ProtoConv_AD_C_12_005]

Table 3-9: ProtoConv_PorttoEth

| Function Name | ProtoConv_LintoEth |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| ASIL Level | ASIL D |  | Status | New |  |  |  |
| Syntax | Std_ReturnType ProtoConv_PorttoEth(const uint16 usLength, const uint8 *pPortData, const uint32 ulDestIdx, const R_ProtoConv_FrameType enTypeFlags) |  |  |  |  |  |  |
| Sync/Async | Synchronous |  |  |  |  |  |  |
| Reentrancy | Non-Reentrant |  |  |  |  |  |  |
| Interrupt State | Runs in the interrupt enable state |  |  |  |  |  |  |
| Parameters (In) | const uint16 usLength | Data Length |  |  |  |  |  |
|  |  | Range |  |  | MIN_PORT_TOTAL_SIZE - MAX_PORT_TOTAL_SIZE |  |  |
| Parameters (In) | const uint8 *pPortData | Address of buffer where Port Data are stored |  |  |  |  |  |
|  |  | Range |  |  | Not NULL_PTR |  |  |
| Parameters (In) | const uint32 ulDestIdx |  | Index of Destination Rooting Table |  |  |  |  |
|  |  |  | Range |  |  | 0 - (XTOETH_DEST_TABLE_ENTRY_NUM-1) |  |
| Parameters (In) | const R_ProtoConv_FrameType enTypeFlags |  | Frame Type |  |  |  |  |
|  |  |  | Range |  |  | PROTOCONV_NORMAL_FRAME or PROTOCONV_EMERGENCY_FRAME |  |
| Parameters (In-Out) | N/A | - |  |  |  |  |  |
| Parameters (Out) | N/A | - |  |  |  |  |  |
| Return Value | Std_ReturnType | E_OK E_NOT_OK |  |  |  |  |  |
| Description | 1-1. Get PORT data information from Parameter usLength, pPortData, 1-2. Obtain the Ethernet destination information by retrieving the argument ulDestIdx. The following processing is performed according to the PORT to Ethernet conversion type. [In case of Basic Type] 2-1. Add PORT frame to PORT Rx Buffer. 2-2. Store Ethernet destination information as Ethernet header in Ethernet Tx Buffer. 2-3. Store the data in the PORT Rx Buffer in the Ethernet Tx Buffer. 2-4. Execute ProtoConv_EthTransmitUserDef() NOTE2 2-5. END [In case of Buffer Type] [PORT frame is an Emergency frame] 2-1. Add PORT frame to PORT Rx Buffer. 2-2. Store Ethernet destination information as Ethernet header in Ethernet Tx Buffer. 2-3. Store the data in the PORT Rx Buffer in the Ethernet Tx Buffer. 2-4. Execute ProtoConv_EthTransmitUserDef() NOTE2 2-5. END [PORT frame is not an Emergency frame] When Buffer overflows if adding the next PORT frame to the Mix Rx buffer, Step 2-1 ~ 2-3 is performed to send the Ethernet frame that has already been stored. 2-1. Store the Ethernet destination information as Ethernet header in Ethernet Tx Buffer. 2-2. Store the data in the Mix Rx Buffer to the Ethernet Tx Buffer. 2-3. Execute ProtoConv_EthTransmitUserDef(). NOTE2 3-1. Add PORT frames to the Mix Rx Buffer. When one of the transmission conditions※1 is satisfied, Step 3-2 ~ 3-4 is performed to send the Ethernet frame. 3-2. Store the Ethernet destination information as an Ethernet header in the Ethernet Tx Buffer. 3-3. Store the data in the Mix Rx Buffer is stored in the Ethernet Tx Buffer. 3-4. Execute ProtoConv_EthTransmitUserDef(). NOTE2 3-5. END If the process ends normally, E_OK is returned. NOTE1: When the TSCF format is generated, Execute ProtoConv_GetTimeUserDef() to acquire the time used for the timestamp, and if the acquisition is successful, set the timestamp in the header information. If it fails, the timestamp invalidity flag is set in the header information. NOTE2: If the function ProtoConv_EthTransmitUserDef() returns an error, this API returns E_NOT_OK, report DET error PROTOCONV_E_TRANS (if PROTOCONV_DEV_ERROR_DETECT == STD_ON) and exits. NOTE3: The receive buffer add and transmit processes are interrupt disabled using ProtoConv_EnterCriticalUserDef() and ProtoConv_ExitCriticalUserDef(). NOTE4: If a VLAN ID is set in the searched LIN to Ethernet Routing Table (VLAN ID≠0), the same VLAN ID is set in the Ethernet header. |  |  |  |  |  |  |
| Preconditions | Initialized state |  |  |  |  |  |  |
| Remarks | For Port, Basic and buffer mode is the same process. |  |  |  |  |  |  |
| DET/DEM Errors Handled | PROTOCONV_E_INV_PARAM |  |  |  |  |  | Invalid parameter passed to the APIs |
|  | PROTOCONV_E_MATCH |  |  |  |  |  | Unmatched Table Error (Routing Table) |
|  | PROTOCONV_E_TRANS |  |  |  |  |  | Transmit Error |

[Covers: req-8552172-00903612]

[Covers: req-8552172-00904662]

[Covers: req-8552172-00904667]

[Covers: req-8552172-00905227]

[Covers: req-8552172-00909476]

[Covers: req-8552172-00909478]

[Covers: req-8552172-00528003]

[Covers: req-8552172-00528002]

[Covers: req-8552172-00528006]

[Covers: req-8552172-00527993]

[Covers: req-8552172-00527980]

#### ProtoConv_TriggerXtoEthTx

[iso26-18_p6_7.4.5] a; [iso26-18_p6_7.4.6]

[Gen5GW_ProtoConv_AD_C_12_006]

Table 3-10: ProtoConv_TriggerXtoEthTx

| Function Name | ProtoConv_TriggerXtoEthTx |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| ASIL Level | ASIL D | Status | New |  |  |
| Syntax | Std_ReturnType ProtoConv_TriggerXtoEthTx(const uint32 ulBufIdx) |  |  |  |  |
| Sync/Async | Synchronous |  |  |  |  |
| Reentrancy | Non-Reentrant |  |  |  |  |
| Interrupt State | Runs in the interrupt enable state |  |  |  |  |
| Parameters (In) | const uint32 ulBufIdx |  | Target Buffer Index |  |  |
|  |  |  | Range | 0 – (PROTOCONV_MIXBUF_MAX-1) |  |
| Parameters (In-Out) | N/A |  | - |  |  |
| Parameters (Out) | N/A |  | - |  |  |
| Return Value | Std_ReturnType |  | E_OK E_NOT_OK |  |  |
| Description | If the argument ulBufIdx is out of range, it returns E_NOT_OK, reports DET error PROTOCONV_E_INV_PARAM (if PROTOCONV_DEV_ERROR_DETECT == STD_ON) and exits. ProtoConv_CreateAndSendEthFrame() is executed and the process is terminated. |  |  |  |  |
| Preconditions | Uninitialized state |  |  |  |  |
| Remarks | N/A |  |  |  |  |
| DET/DEM Errors Handled | PROTOCONV_E_INV_PARAM |  |  |  | Invalid parameter passed to the APIs |
|  | PROTOCONV_E_TRANS |  |  |  | Transmit Error |
|  | PROTOCONV_E_PARAM_CONFIG |  |  |  | Config Error (NULL Pointer) |

[Covers: req-8552172-00903610]

[Covers: req-8552172-00903611]

[Covers: req-8552172-00903612]

[Covers: req-8552172-00905227]

[Covers: req-8552172-00904662]

[Covers: req-8552172-00909476]

[Covers: req-8552172-00909478]

[Covers: req-8552172-00528003]

[Covers: req-8552172-00528002]

[Covers: req-8552172-00528006]

[Covers: req-8552172-00527993]

[Covers: req-8552172-00527980]

#### ProtoConv_MixtoEthMain

[Gen5GW_ProtoConv_AD_C_12_007]

Table 3-11: ProtoConv_MixtoEthMain

| Function Name | ProtoConv_MixtoEthMain |  |  |
| --- | --- | --- | --- |
| ASIL Level | ASIL D | Status | New |
| Syntax | void ProtoConv_MixtoEthMain(void) |  |  |
| Sync/Async | Synchronous |  |  |
| Reentrancy | Non-Reentrant |  |  |
| Interrupt State | Runs in the interrupt enable state |  |  |
| Parameters (In) | N/A |  | - |
| Parameters (In-Out) | N/A |  | - |
| Parameters (Out) | N/A |  | - |
| Return Value | N/A |  | - |
| Description | If Mix Rx Buffer or Ethernet Tx Buffer is NULL_PTR, the process is terminated. If there is data in the buffer and the conversion cycle time has elapsed since the last transmission, ProtoConv_CreateAndSendEthFrame() is executed and the process is terminated. |  |  |
| Preconditions | N/A |  |  |
| Remarks | Calls should be made with a cycle of at least 1ms (it is recommended that they should be made every 1ms). |  |  |
| DET/DEM Errors Handled | N/A |  |  |

[Covers: req-8552172-00905314]

[Covers: req-8552172-00905315]

[Covers: req-8552172-00905316]

[Covers: req-8552172-00905317]

[Covers: req-8552172-00904667]

[Covers: req-8552172-00905227]

[Covers: req-8552172-00909476]

[Covers: req-8552172-00909478]

[Covers: req-8552172-00528006]

[Covers: req-8552172-00527993]

#### ProtoConv_EnqueueCan

[iso26-18_p6_7.4.5] a; [iso26-18_p6_7.4.6]

[Gen5GW_ProtoConv_AD_C_12_008]

Table 3-12: ProtoConv_EnqueueCan

| Function Name | ProtoConv_EnqueueCan |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| ASIL Level | ASIL D | Status |  | New |  |
| Syntax | Std_ReturnType ProtoConv_EnqueueCan(const uint8 ucBusIdx, const R_ProtoConv_CanFrameType *pCanFrame, const R_ProtoConv_TimeStampType *pTimeStamp) |  |  |  |  |
| Sync/Async | Synchronous |  |  |  |  |
| Reentrancy | Non-Reentrant |  |  |  |  |
| Interrupt State | Runs in the interrupt enable state |  |  |  |  |
| Parameters (In) | const uint8 ucBusIdx |  |  | Source Bus ID |  |
|  |  |  |  | Range | 0 - (PROTOCONV_CAN_CH_NUM-1) |
| Parameters (In) | const R_ProtoConv_CanFrameType *pCanFrame |  |  | Address of buffer where CAN frames are stored |  |
|  |  |  |  | Range | Not NULL_PTR |
| Parameters (In) | const R_ProtoConv_TimeStampType *pTimeStamp |  |  | Address of buffer where timestamp is stored |  |
|  |  |  |  | Range | Not NULL_PTR |
| Parameters (In-Out) | N/A |  |  | - |  |
| Parameters (Out) | N/A |  |  | - |  |
| Return Value | Std_ReturnType |  |  | E_OK E_NOT_OK |  |
| Description | If the argument pCanFrame or pTimeStamp is NULL_PTR, it returns E_NOT_OK, reports DET error PROTOCONV_E_PARAM_POINTER (if PROTOCONV_DEV_ERROR_DETECT == STD_ON) and exits. If the argument ucBusIdx is out of range, it returns E_NOT_OK, reports DET error PROTOCONV_E_INV_PARAM (if PROTOCONV_DEV_ERROR_DETECT == STD_ON) and exits. If the CAN FIFO memory at source Bus ID ucBusIdx is full, return E_NOT_OK and exit. Otherwise, enqueue CAN frame into CAN FIFO memory. If the CAN FIFO memory at source Bus ID ucBusIdx reaches the maximum configured FIFO depth, set the Tail value of CAN FIFO memory to NUM0, else increase the Tail value. If the Tail value and the Head value of CAN FIFO memory at source Bus ID ucBusIdx are equal, set Flag member CAN FIFO memory to PROTOCONV_FIFO_FULL, else set it to PROTOCONV_FIFO_AVAILABLE. If the process ends normally, E_OK is returned. |  |  |  |  |
| Preconditions | Initialized state |  |  |  |  |
| Remarks | N/A |  |  |  |  |
| DET/DEM Errors Handled | PROTOCONV_E_FIFO_FULL |  | Unexpectedly full FIFO memory |  |  |

[Covers: req-8552172-00909163]

[Covers: req-8552172-00903610]

[Covers: req-8552172-00905227]

[Covers: req-8552172-00904662]

[Covers: req-8552172-00909476]

[Covers: req-8552172-00909478]

[Covers: req-8552172-00528006]

[Covers: req-8552172-00527993]

#### ProtoConv_EnqueueLin

[iso26-18_p6_7.4.5] a; [iso26-18_p6_7.4.6]

[Gen5GW_ProtoConv_AD_C_12_009]

Table 3-13: ProtoConv_EnqueueLin

| Function Name | ProtoConv_EnqueueLin |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| ASIL Level | ASIL D | Status | New |  |  |
| Syntax | Std_ReturnType ProtoConv_EnqueueLin(const uint8 ucBusIdx, const R_ProtoConv_LinFrameType *pLinFrame) |  |  |  |  |
| Sync/Async | Synchronous |  |  |  |  |
| Reentrancy | Non-Reentrant |  |  |  |  |
| Interrupt State | Runs in the interrupt enable state |  |  |  |  |
| Parameters (In) | const uint8 ucBusIdx |  | Source Bus ID |  |  |
|  |  |  | Range |  | 0 - (PROTOCONV_LIN_CH_NUM-1) |
| Parameters (In) | const R_ProtoConv_LinFrameType *pLinFrame |  | Address of buffer where LIN frames are stored |  |  |
|  |  |  | Range |  | Not NULL_PTR |
| Parameters (In-Out) | N/A |  | - |  |  |
| Parameters (Out) | N/A |  | - |  |  |
| Return Value | Std_ReturnType |  | E_OK E_NOT_OK |  |  |
| Description | If the argument pLinFrame is NULL_PTR, it returns E_NOT_OK, reports DET error PROTOCONV_E_PARAM_POINTER (if PROTOCONV_DEV_ERROR_DETECT == STD_ON) and exits. If the argument ucBusIdx is out of range, it returns E_NOT_OK, reports DET error PROTOCONV_E_INV_PARAM (if PROTOCONV_DEV_ERROR_DETECT == STD_ON) and exits. If the LIN FIFO memory at source Bus ID ucBusIdx is full, return E_NOT_OK and exit. Otherwise, enqueue LIN frame into LIN FIFO memory. If the LIN FIFO memory at source Bus ID ucBusIdx reaches the maximum configured FIFO depth, set the Tail value of LIN FIFO memory to NUM0, else increase the Tail value. If the Tail value and the Head value of LIN FIFO memory at source Bus ID ucBusIdx are equal, set Flag member LIN FIFO memory to PROTOCONV_FIFO_FULL, else set it to PROTOCONV_FIFO_AVAILABLE. If the process ends normally, E_OK is returned. |  |  |  |  |
| Preconditions | Initialized state |  |  |  |  |
| Remarks | N/A |  |  |  |  |
| DET/DEM Errors Handled | PROTOCONV_E_FIFO_FULL |  |  | Unexpectedly full FIFO memory |  |

[Covers: req-8552172-00909163]

[Covers: req-8552172-00903611]

[Covers: req-8552172-00905227]

[Covers: req-8552172-00904662]

[Covers: req-8552172-00909476]

[Covers: req-8552172-00909478]

[Covers: req-8552172-00528003]

[Covers: req-8552172-00528002]

[Covers: req-8552172-00528006]

[Covers: req-8552172-00527993]

[Covers: req-8552172-00527980]

#### ProtoConv_EnqueuePort

[iso26-18_p6_7.4.5] a; [iso26-18_p6_7.4.6]

[Gen5GW_ProtoConv_AD_C_12_010]

Table 3-14: ProtoConv_EnqueuePort

| Function Name | ProtoConv_EnqueuePort |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| ASIL Level | ASIL D |  |  | Status | New |
| Syntax | Std_ReturnType ProtoConv_EnqueuePort(const uint16 usLength, const uint8 *pPortData, const uint32 ulDestIdx) |  |  |  |  |
| Sync/Async | Synchronous |  |  |  |  |
| Reentrancy | Non-Reentrant |  |  |  |  |
| Interrupt State | Runs in the interrupt enable state |  |  |  |  |
| Parameters (In) | const uint16 usLength | Data Length |  |  |  |
|  |  | Range | MIN_PORT_TOTAL_SIZE - MAX_PORT_TOTAL_SIZE |  |  |
| Parameters (In) | const uint8 *pPortData | Address of buffer where Port Data are stored |  |  |  |
|  |  | Range | Not NULL_PTR |  |  |
| Parameters (In) | const uint32 ulDestIdx | Index of Destination Rooting Table |  |  |  |
|  |  | Range | 0 – (XTOETH_DEST_TABLE_ENTRY_NUM-1) |  |  |
| Parameters (In-Out) | N/A | - |  |  |  |
| Parameters (Out) | N/A | - |  |  |  |
| Return Value | Std_ReturnType | E_OK E_NOT_OK |  |  |  |
| Description | If the argument pPortData is NULL_PTR, it returns E_NOT_OK, reports DET error PROTOCONV_E_PARAM_POINTER (if PROTOCONV_DEV_ERROR_DETECT == STD_ON) and exits. If the argument usLength or ulDestIdx is out of range, it returns E_NOT_OK, reports DET error PROTOCONV_E_INV_PARAM (if PROTOCONV_DEV_ERROR_DETECT == STD_ON) and exits. If the Port FIFO memory is full, return E_NOT_OK and exit. Otherwise, enqueue Port frame into Port FIFO memory. If the Port FIFO memory reaches the maximum configured FIFO depth, set the Tail value of Port FIFO memory to NUM0, else increase the Tail value. If the Tail value and the Head value of Port FIFO memory are equal, set Flag member Port FIFO memory to PROTOCONV_FIFO_FULL, else set it to PROTOCONV_FIFO_AVAILABLE. If the process ends normally, E_OK is returned. |  |  |  |  |
| Preconditions | Initialized state |  |  |  |  |
| Remarks | N/A |  |  |  |  |
| DET/DEM Errors Handled | PROTOCONV_E_FIFO_FULL |  | Unexpectedly full FIFO memory |  |  |

[Covers: req-8552172-00909163]

[Covers: req-8552172-00903612]

[Covers: req-8552172-00905227]

[Covers: req-8552172-00904662]

[Covers: req-8552172-00909476]

[Covers: req-8552172-00909478]

[Covers: req-8552172-00528003]

[Covers: req-8552172-00528002]

[Covers: req-8552172-00528006]

[Covers: req-8552172-00527993]

[Covers: req-8552172-00527980]

#### ProtoConv_TriggerQueueDataXtoEth

[Gen5GW_ProtoConv_AD_C_12_011]

Table 3-15: ProtoConv_TriggerQueueDataXtoEth

| Function Name | ProtoConv_TriggerQueueDataXtoEth |  |  |  |
| --- | --- | --- | --- | --- |
| ASIL Level | ASIL D | Status | New |  |
| Syntax | Std_ReturnType ProtoConv_TriggerQueueDataXtoEth(void) |  |  |  |
| Sync/Async | Synchronous |  |  |  |
| Reentrancy | Non-Reentrant |  |  |  |
| Interrupt State | Runs in the interrupt enable state |  |  |  |
| Parameters (In) | N/A |  | - |  |
| Parameters (In-Out) | N/A |  | - |  |
| Parameters (Out) | N/A |  | - |  |
| Return Value | N/A |  | - |  |
| Description | Conduct the dequeue process and convert the queued CAN/LIN/Port frame to Ethernet frame and then send the converted Ethernet frame If the return value is other than E_OK, E_NOT_OK is returned the return value. If the process ends normally, E_OK is returned. |  |  |  |
| Preconditions | Initialized state |  |  |  |
| Remarks | N/A |  |  |  |
| DET/DEM Errors Handled | PROTOCONV_E_FIFO_EMPTY |  |  | Unexpectedly empty FIFO memory |
|  | PROTOCONV_E_INV_PARAM |  |  | Invalid parameter passed to the APIs |
|  | PROTOCONV_E_MATCH |  |  | Unmatched Table Error (Routing Table) |
|  | PROTOCONV_E_TRANS |  |  | Transmit Error |

[Covers: req-8552172-00909163]

[Covers: req-8552172-00903610]

[Covers: req-8552172-00903611]

[Covers: req-8552172-00903612]

[Covers: req-8552172-00905227]

[Covers: req-8552172-00904662]

[Covers: req-8552172-00909476]

[Covers: req-8552172-00909478]

[Covers: req-8552172-00528003]

[Covers: req-8552172-00528002]

[Covers: req-8552172-00528006]

[Covers: req-8552172-00527993]

[Covers: req-8552172-00527980]

#### ProtoConv_CreateAndSendEthFrame

[Gen5GW_ProtoConv_AD_C_12_012]

Table 3-16: ProtoConv_CreateAndSendEthFrame

| Function Name | ProtoConv_CreateAndSendEthFrame |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| ASIL Level | ASIL D | Status | New |  |  |
| Syntax | Std_ReturnType ProtoConv_CreateAndSendEthFrame(const uint32 ulEntryIdx, ProtoConv_BufInfoType *pBufInfo, const R_ProtoConv_HeaderType *pHeaderField) |  |  |  |  |
| Sync/Async | Synchronous |  |  |  |  |
| Reentrancy | Non-Reentrant |  |  |  |  |
| Interrupt State | Runs in the interrupt enable state |  |  |  |  |
| Parameters (In) | const uint32 ulEntryIdx |  | Route table index |  |  |
|  |  |  | Range |  | 0 – (PROTOCONV_MIXBUF_MAX – 1) |
| Parameters (In) | const R_ProtoConv_HeaderType *pHeaderField |  | IP/UDP/AVTP header structure |  |  |
|  |  |  | Range |  | Not NULL_PTR |
| Parameters (Out) | N/A |  | - |  |  |
| Parameters (In-Out) | ProtoConv_BufInfoType *pBufInfo |  | Buffer Information structure |  |  |
|  |  |  | Range |  | Not NULL_PTR |
| Return Value | Std_ReturnType |  | E_OK E_NOT_OK |  |  |
| Description | Store Ethernet header to Ethernet Tx Buffer The total data size is obtained by summing the member value Rx Buffer storage size of Buffer Information structure, header storage size, and Ethernet header/FCS size. Copy the data part from specified Rx Buffer to Ethernet Tx Buffer. If the total data size does not meet the minimum Ethernet standard size, 0 padding is performed. If padding is done, the Ethernet frame size is updated to the size after padding. Exclude Ethernet header and FCS size from the total data size. Determine the value of the frame type to be passed to the send function. Convert the destination MAC address stored in the current Ethernet destination information of 32-bit, number of entries 2, into a destination MAC address of 8-bit, number of entries 6. Execute ProtoConv_EthTransmitUserDef() to send the converted Ethernet frame. If ProtoConv_EthTransmitUserDef() returns an error, this API returns E_NOT_OK, report DET error PROTOCONV_E_TRANS (if PROTOCONV_DEV_ERROR_DETECT == STD_ON) and exits. If the process ends normally, E_OK is returned. |  |  |  |  |
| Preconditions | N/A |  |  |  |  |
| Remarks | N/A |  |  |  |  |
| DET/DEM Errors Handled | PROTOCONV_E_TRANS |  |  | Transmit Error |  |

[Covers: req-8552172-00903610]

[Covers: req-8552172-00903611]

[Covers: req-8552172-00903612]

[Covers: req-8552172-00905227]

[Covers: req-8552172-00904662]

[Covers: req-8552172-00909476]

[Covers: req-8552172-00909478]

[Covers: req-8552172-00528003]

[Covers: req-8552172-00528002]

[Covers: req-8552172-00528006]

[Covers: req-8552172-00527993]

[Covers: req-8552172-00527980]

#### ProtoConv_StorePdutoEthBufData

[Gen5GW_ProtoConv_AD_C_12_013]

Table 3-17: ProtoConv_StorePdutoEthBufData

| Function Name | StorePdutoEthBufData |  |  |  |
| --- | --- | --- | --- | --- |
| ASIL Level | ASIL D | Status | New |  |
| Syntax | void ProtoConv_StorePdutoEthBufData(const uint32 ulPduId, const uint32 ulPduDlc, const uint8 *pPduData, ProtoConv_BufInfoType *pBufInfo) |  |  |  |
| Sync/Async | Synchronous |  |  |  |
| Reentrancy | Non-Reentrant |  |  |  |
| Interrupt State | Runs in the interrupt enable state |  |  |  |
| Parameters (In) | const uint32 ulPduId |  | PDU ID |  |
|  |  |  | Range | 32-bit value |
| Parameters (In) | const uint32 ulPduDlc |  | PDU DLC |  |
|  |  |  | Range | CAN: PROTOCONV_DLC_8 or less, PROTOCONV_DLC_12, PROTOCONV_DLC_16, PROTOCONV_DLC_20, PROTOCONV_DLC_24, PROTOCONV_DLC_32, PROTOCONV_DLC_48, PROTOCONV_DLC_64 LIN: LINFRAME_LEN_MAX or less Port: MIN_PORT_TOTAL_SIZE - MAX_PORT_TOTAL_SIZE |
| Parameters (In) | const uint8 *pPduData |  | PDU Data |  |
|  |  |  | Range | Not NULL_PTR |
| Parameters (In-Out) | ProtoConv_BufInfoType *pBufInfo |  | Buffer Information pointer |  |
|  |  |  | Range | Not NULL_PTR |
| Parameters (Out) | N/A |  | - |  |
| Return Value | N/A |  |  |  |
| Description | Extract the ID and DLC from the IPDU-M frame and store it in the local variables LulPduId and LucDlc. Calculate the specified Rx Buffer storage size. If the specified Rx Buffer storage size overflows※1 when a PDU frame is stored, the function returns E_NOT_OK and exits. Store PDU frame (PDU ID (4 bytes) + PDU length (4 bytes) + Data (64 bytes maximum for CAN, 8 bytes maximum for LIN, 256 bytes maximum for Port)) into the specified Rx Buffer. Increment the frame storage count. If the process ends normally, E_OK is returned. |  |  |  |
| Preconditions | N/A |  |  |  |
| Remarks | ※1 Upper limit is based on specified module Rx buffer: CAN: pConfig->pBufCfg->ulCanRxBufSize LIN: pConfig->pBufCfg->ulLinRxBufSize Port: pConfig->pBufCfg->ulPortRxBufSize Mix: pConfig->pBufCfg->ulMixRxBufSize[PROTOCONV_MIXBUF_MAX] |  |  |  |
| DET/DEM Errors Handled | N/A |  |  |  |

[Covers: req-8552172-00903610]

[Covers: req-8552172-00903611]

[Covers: req-8552172-00903612]

[Covers: req-8552172-00903437]

[Covers: req-8552172-00909476]

[Covers: req-8552172-00909478]

[Covers: req-8552172-00528003]

[Covers: req-8552172-00528002]

[Covers: req-8552172-00528006]

[Covers: req-8552172-00527993]

[Covers: req-8552172-00527980]

#### ProtoConv_FifoInit

[Gen5GW_ProtoConv_AD_C_12_014]

Table 3-18: ProtoConv_FifoInit

| Function Name | ProtoConv_FifoInit |  |  |
| --- | --- | --- | --- |
| ASIL Level | ASIL D | Status | New |
| Syntax | void ProtoConv_FifoInit(void) |  |  |
| Sync/Async | Synchronous |  |  |
| Reentrancy | Non-Reentrant |  |  |
| Interrupt State | Runs in the interrupt enable state |  |  |
| Parameters (In) | N/A |  | - |
| Parameters (In-Out) | N/A |  | - |
| Parameters (Out) | N/A |  | - |
| Return Value | N/A |  | - |
| Description | Initialize the value of CAN FIFO frame count and the values of members (ulHead and ulTail) of CAN FIFO structure of all CAN channels (PROTOCONV_CAN_CH_NUM) to NUM0. Set the status flag of CAN FIFO structure of all CAN channels as PROTOCONV_FIFO_AVAILABLE. Initialize the value of LIN FIFO frame count and the values of members (ulHead and ulTail) of LIN FIFO structure of all LIN channels (PROTOCONV_LIN_CH_NUM) to NUM0. Set the status flag of LIN FIFO structure of all LIN channels as PROTOCONV_FIFO_AVAILABLE. Initialize the value of Port FIFO frame count and the values of members (ulHead and ulTail) of Port FIFO structure to NUM0. Set the status flag of Port FIFO structure as PROTOCONV_FIFO_AVAILABLE. Initialize the value of Ethernet FIFO frame count and the values of members (ulHead and ulTail) of Ethernet FIFO structure to NUM0. Set the status flag of Ethernet FIFO structure as PROTOCONV_FIFO_AVAILABLE. |  |  |
| Preconditions | N/A |  |  |
| Remarks | N/A |  |  |
| DET/DEM Errors Handled | N/A |  |  |

[Covers: req-8552172-00903609]

[Covers: req-8552172-00909476]

[Covers: req-8552172-00909478]

[Covers: req-8552172-00528006]

[Covers: req-8552172-00527993]

[Covers: req-8552172-00909163]

[Covers: req-8552172-00909164]

### Error handling

Table 3-19: Error Handling List

| No. | Error Name | Error Factor | Error Handling |
| --- | --- | --- | --- |
| (1) | E_OK | Successful Completion | - |
| (2) | E_NOT_OK | Unsuccessful Completion |  |
| (3) | PROTOCONV_E_MATCH | Unmatched Table Error (Routing Table) | Returns the error to the user application that calls this API interfaces. Error handling is performed by the user application. |
| (5) | PROTOCONV_E_FIFO_FULL | Unexpectedly full FIFO memory |  |
| (6) | PROTOCONV_E_FIFO_EMPTY | Unexpectedly empty FIFO memory |  |
| (7) | PROTOCONV_E_INV_PARAM | Invalid parameter passed to the APIs |  |
| (8) | PROTOCONV_E_TRANS | Transmit Error |  |
| (10) | PROTOCONV_E_PARAM_CONFIG | Config Error (NULL Pointer) |  |

## Ethernet to CAN/LIN/Port Conversion Function Design

### Features

Table 3-20: Function List

| No. | Function Name | ASIL level | Description |
| --- | --- | --- | --- |
|  | ProtoConv_EthtoXInit | ASIL D | Initialize Ethernet to CAN/LIN/Port Conversion Function. |
|  | ProtoConv_EthtoX | ASIL D | Convert Ethernet frame to CAN/LIN/Parallel Port frames |
|  | ProtoConv_GetSendStatus | ASIL D | Get CAN packet sendability setting |
|  | ProtoConv_UpdateSendStatus | ASIL D | Update CAN packet sendability setting |
|  | ProtoConv_EnqueueEth | ASIL D | Enqueue Ethernet message to the pre-defined memory. |
|  | ProtoConv_DequeueEth | ASIL D | Dequeue Ethernet message from the pre-defined memory. |
|  | ProtoConv_SearchCanRoutTbl | ASIL D | Search CAN to Ethernet Routing Table |
|  | ProtoConv_UnpackCan | ASIL D | Unpack the CAN frame in ACF format from the payload section |
|  | ProtoConv_UnpackCanIPDUM | ASIL D | Unpack the CAN frame in IPDU-M format from the payload section |
|  | ProtoConv_SearchLinRoutTbl | ASIL D | Search LIN to Ethernet Routing Table |
|  | ProtoConv_UnpackLin | ASIL D | Unpack the LIN frame in ACF format from the payload section |
|  | ProtoConv_UnpackLinIPDUM | ASIL D | Unpack the LIN frame in IPDU-M format from the payload section |
|  | ProtoConv_UnpackPort | ASIL D | Unpack the Port frame in ACF format from the payload section |
|  | ProtoConv_UnpackPortIPDUM | ASIL D | Unpack the Port frame in IPDU-M format from the payload section |

### Interfaces

#### ProtoConv_EthtoXInit

[Gen5GW_ProtoConv_AD_C_13_001]

Table 3-21: ProtoConv_EthtoXInit

| Function Name | ProtoConv_EthtoXInit |  |  |  |
| --- | --- | --- | --- | --- |
| ASIL Level | ASIL D | Status | New |  |
| Syntax | void ProtoConv_EthtoXInit(const R_ProtoConv_CfgType *pConfig) |  |  |  |
| Sync/Async | Synchronous |  |  |  |
| Reentrancy | Non-Reentrant |  |  |  |
| Interrupt State | Runs in the interrupt enable state |  |  |  |
| Parameters (In) | const R_ProtoConv_CfgType *pConfig |  | Pointer to initialization config. |  |
|  |  |  | Range | Not NULL_PTR |
| Parameters (In-Out) | N/A |  | - |  |
| Parameters (Out) | N/A |  | - |  |
| Return Value | N/A |  |  |  |
| Description | Initialize the global variable (GulSendStatus) that stores the CAN packet send suppression setting information (all channels are ready to send). The set value of the argument pConfig is saved. The value of the address and size of the send/receive buffer area of the member of argument pConfig. Initialize Ethernet Rx Buffer and CAN/LIN/Port Tx Buffer for Ethernet to CAN/LIN/Port conversion. If the process ends normally, E_OK is returned. |  |  |  |
| Preconditions | Uninitialized state |  |  |  |
| Remarks | N/A |  |  |  |
| DET/DEM Errors Handled | N/A |  |  |  |

[Covers: req-8552172-00903609]

[Covers: req-8552172-00905312]

[Covers: req-8552172-00904671]

[Covers: req-8552172-00904678]

[Covers: req-8552172-00905221]

[Covers: req-8552172-00904663]

[Covers: req-8552172-00905219]

[Covers: req-8552172-00905218]

[Covers: req-8552172-00904664]

[Covers: req-8552172-00904665]

[Covers: req-8552172-00904666]

[Covers: req-8552172-00909476]

[Covers: req-8552172-00909478]

[Covers: req-8552172-00528003]

[Covers: req-8552172-00528002]

[Covers: req-8552172-00528006]

[Covers: req-8552172-00527993]

[Covers: req-8552172-00527980]

[iso26-18_p6_7.4.5] a

[iso26-18_p6_7.4.5] a

#### ProtoConv_EthtoX

[Gen5GW_ProtoConv_AD_C_13_002]

Table 3-22: ProtoConv_EthtoX

| Function Name | ProtoConv_EthtoCan |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| ASIL Level | ASIL D | Status | New |  |  |
| Syntax | Std_ReturnType ProtoConv_EthtoX(const uint16 usFrameType, const uint8 *aaSrcMacAddr, const uint8 *pEthFrame, const uint32 ulFrameLen, uint32 *pSendStatus, ProtoConv_EthtoXBufInfoType *pBufInfo) |  |  |  |  |
| Sync/Async | Synchronous |  |  |  |  |
| Reentrancy | Non-Reentrant |  |  |  |  |
| Interrupt State | Runs in the interrupt enable state |  |  |  |  |
| Parameters (In) | const uint16 usFrameType |  | Frame type for Ethernet frames |  |  |
|  |  |  | Range | PROTOCONV_FRAMETYPE_AVTP (0x22F0U) PROTOCONV_FRAMETYPE_IPV4 (0x0800U) PROTOCONV_FRAMETYPE_IPV6 (0x86DDU) PROTOCONV_FRAMETYPE_8021Q (0x8100U) |  |
| Parameters (In) | const uint8 *aaSrcMacAddr |  | Address where MAC addresses to be filtered are stored |  |  |
|  |  |  | Range | Not NULL_PTR |  |
| Parameters (In) | const uint8 *pEthFrame |  | Address of buffer where Ethernet frame (Payload) is stored |  |  |
|  |  |  | Range | Not NULL_PTR |  |
| Parameters (In) | const uint32 ulFrameLen |  | Ethernet frame length (Payload) |  |  |
|  |  |  | Range | 46~1500 |  |
| Parameters (In-Out) | ProtoConv_EthtoXBufInfoType *pBufInfo |  | Address to store buffer information |  |  |
|  |  |  | Range | Not NULL_PTR |  |
| Parameters (Out) | uint32 *pSendStatus |  | Address to store CAN packet transmission results |  |  |
|  |  |  | Range | Not NULL_PTR |  |
| Return Value | Std_ReturnType |  | E_OK E_NOT_OK |  |  |
| Description | ・Copy Ethernet frame (Payload) of argument pEthFrame to Ethernet Rx Buffer. ・When the frame type is PROTOCONV_FRAMETYPE_IPV4 (0x0800U), IPv4 and UDP headers are checked. 1) Version, header length, no fragment: returns E_NOT_OK, report DET error PROTOCONV_E_PARAM_DATA_LENGTH (if PROTOCONV_DEV_ERROR_DETECT == STD_ON) 2) IP header checksum value: returns E_NOT_OK, report DET error PROTOCONV_E_IPCKSUM (if PROTOCONV_DEV_ERROR_DETECT == STD_ON) 3) (Data length of UDP header + IP header length) ≦ Ethernet frame length: returns E_NOT_OK, report DET error PROTOCONV_E_PARAM_DATA_LENGTH (if PROTOCONV_DEV_ERROR_DETECT == STD_ON) 4) UDP checksum value: returns E_NOT_OK, report DET error PROTOCONV_E_UDPCKSUM (if PROTOCONV_DEV_ERROR_DETECT == STD_ON) ※Only when the checksum value in the header is non-zero ・When the frame type is PROTOCONV_FRAMETYPE_IPV6 (0x86DDU), IPv6 and UDP headers are checked. 1) Version, header length, no fragment: returns E_NOT_OK, report DET error PROTOCONV_E_PARAM_DATA_LENGTH (if PROTOCONV_DEV_ERROR_DETECT == STD_ON) 2) (UDP header data length + IP header length) ≤ Ethernet frame length: returns E_NOT_OK, report DET error PROTOCONV_E_PARAM_DATA_LENGTH (if PROTOCONV_DEV_ERROR_DETECT == STD_ON) 3) UDP checksum value: Calculates the checksum value and returns E_NOT_OK, report DET error PROTOCONV_E_UDPCKSUM (if PROTOCONV_DEV_ERROR_DETECT == STD_ON) ※Only when the checksum value in the header is non-zero ・Check NTSCF or TSCF header, returns E_NOT_OK, report DET error PROTOCONV_E_PARAM_DATA_LENGTH (if PROTOCONV_DEV_ERROR_DETECT == STD_ON) If the AVTP format is PROTOCONV_AVTP_NONE (2U), no check is performed 1) When the subtype is SUBTYPE_NTSCF (0x82U), If the subtype is SUBTYPE_NTSCF (0x82U), the process from 3) onward is repeated for multiple CAN/LIN/Parallel Port frames that are packed in the payload length and NTSCF payload section. 2) When the subtype is SUBTYPE_TSCF (0x05U), If the subtype is SUBTYPE_TSCF (0x05U), repeat the process from 3) for multiple CAN/LIN/Parallel Port frames that are packed in the payload length and TSCF payload section. 3) Check if the converted message is ACF or IPDU-M Packing format. If it is ACF, go to process 3.1). If it is IPDU-M Packing format, go to process 3.2). 3.1) If the ACF frame type is valid, the function categorizes and processes each type of frame. For CAN frames, the process from 3a-1) onward is repeated for multiple CAN frames that are packed in the payload length. For LIN frames, the process from 3b-1) onward is repeated for multiple LIN frames that are packed in the payload length. For Parallel Port frames, the process from 3c-1) onward is repeated for multiple Parallel Port frames that are packed in the payload length. 3.2) Check the PDU ID to process the IPDU-M Ethernet to multiple CAN/LIN/Parallel Port frames transmissions. If the PDU ID has the value of the configured PORT_PDU_ID, the process from 3d-1) onward is repeated for multiple Port messages packed in the payload length. If the PDU ID does not have the value of the configured PORT_PDU_ID, search IPDU-M Ethernet to CAN/LIN Routing Tables by the PDU ID to obtain the value representing for the corresponding converted module LenPduConvMsn. [In case of successful retrieval] If LenPduConvMsn has value of PROTOCONV_PDUETHTOCAN, the process from 3a-1) onward is repeated for multiple CAN messages packed in the payload length. If LenPduConvMsn has value of PROTOCONV_PDUETHTOLIN, the process from 3f-1) onward is repeated for multiple LIN messages packed in the payload length. [In case of retrieval failure] If the search fails, return E_NOT_OK and exit. [For CAN messages] 3a-1) Check the validity of the DLC in the CAN frame and return E_NOT_OK, report DET error PROTOCONV_E_PACKEDCAN (if PROTOCONV_DEV_ERROR_DETECT == STD_ON) in case of an error and terminate. 3a-2) Search the Ethernet to CAN Routing Table by CAN-ID 3a-3) [In case of successful retrieval] Refer to the setting information of CAN packet transmission inhibition, and do nothing if transmission is prohibited. If transmission is permitted, the user-defined ProtoConv_CanTransmitTSUserDef() is executed to transmit the ACF CAN/CANFD frame or the user-defined ProtoConv_CanTransmitUserDef() is executed to transmit the Abbreviated ACF CAN/CANFD frame or for IPDU-M CAN transmission. If an error occurs during function execution, return E_NOT_OK and exit. 3a-4) [In case of retrieval failure] For ACF CAN/CANFD frame Execute user-defined ProtoConv_CanNotifyTSUserDef() to transmit between CAN frame cores. For Abbreviated ACF CAN/CANFD frame or for IPDU-M CAN transmission. Execute user-defined ProtoConv_CanNotifyUserDef() to transmit between CAN frame cores. If an error occurs during function execution, return E_NOT_OK, report DET error PROTOCONV_E_BSWNOTIFY (if PROTOCONV_DEV_ERROR_DETECT == STD_ON) and exit. [For LIN messages] 3b-1) Check the validity of the DLC in the LIN frame and return E_NOT_OK, report DET error PROTOCONV_E_PACKEDLIN (if PROTOCONV_DEV_ERROR_DETECT == STD_ON) in case of an error and terminate. 3b-2) Search the Ethernet to LIN Routing Table by LIN-ID 3b-3) [In case of successful retrieval] Refer to the setting information of LIN packet transmission inhibition, and do nothing if transmission is prohibited. If transmission is permitted, the user-defined ProtoConv_LinTransmitUserDef() is executed to transmit the LIN frame. If an error occurs during function execution, return E_NOT_OK and exit. 3b-4) [In case of retrieval failure] Execute user-defined ProtoConv_LinNotifyUserDef() to transmit between LIN frame cores. If an error occurs during function execution, return E_NOT_OK, report DET error PROTOCONV_E_BSWNOTIFY (if PROTOCONV_DEV_ERROR_DETECT == STD_ON), set the global variable GucCheckError to PROTOCONV_E_BSWNOTIFY and exit. [For ACF Parallel Port messages] 3c-1) Check the validity of the bit_width field and acf_msg_length field in the Parallel Port frame and return E_NOT_OK, report DET error PROTOCONV_E_PACKEDPORT (if PROTOCONV_DEV_ERROR_DETECT == STD_ON) in case of an error and terminate the process. If there is no error, copy Port data to Port Tx Buffer. 3c-2) [In case of the next message’s type is still Parallel Port] Repeat the process from 3c-1) for multiple Parallel Port frames 3c-3) [In case of the next message’s type is not Parallel Port (done reading all consecutive Port data] The user-defined ProtoConv_PortTxUserDef() is executed to transmit the ACF Parallel Port frame. If an error occurs during function execution, return E_NOT_OK and exit. [For IPDU-M Port messages transmission] 3d-1) Check the validity of the DLC in the PDU framed and return E_NOT_OK, report DET error PROTOCONV_E_PACKEDPORT (if PROTOCONV_DEV_ERROR_DETECT == STD_ON) in case of an error and terminate the process. If there is no error, copy Port data to Port Tx Buffer. 3d-2) [In case of the next message’s type is still Parallel Port] Repeat the process from 3d-1) for multiple Parallel Port frames 3d-3) [In case of the next message’s type is not Parallel Port (done reading all consecutive Port data] The user-defined ProtoConv_PortTransmitUserDef() is executed to transmit the ACF Parallel Port frame. If an error occurs during function execution, return E_NOT_OK and exit. ・Performs filtering corresponding to the element specified at initialization. If filtered, returns E_NOT_OK, report DET error PROTOCONV_E_FILTER (if PROTOCONV_DEV_ERROR_DETECT == STD_ON) and exit. ・Returns E_OK if the process ends normally |  |  |  |  |
| Preconditions | Initialized state |  |  |  |  |
| Remarks | N/A |  |  |  |  |
| DET/DEM Errors Handled | PROTOCONV_E_TRANS |  |  |  | Transmit Error |
|  | PROTOCONV_E_BSWNOTIFY |  |  |  | BSW Notification Error |
|  | PROTOCONV_E_IPCKSUM |  |  |  | IP Header Checksum Error |
|  | PROTOCONV_E_UDPCKSUM |  |  |  | UDP Header Checksum Error |
|  | PROTOCONV_E_PACKEDCAN |  |  |  | Packed CAN Frame Error |
|  | PROTOCONV_E_PACKEDLIN |  |  |  | Packed LIN Frame Error |
|  | PROTOCONV_E_PACKEDPORT |  |  |  | Packed Parallel Port Frame Error |
|  | PROTOCONV_E_PACKEDMSG |  |  |  | Packed CAN/LIN/Port Frame Error |
|  | PROTOCONV_E_FILTER |  |  |  | Filtered and discarded |
|  | PROTOCONV_E_CFG_ACFTYPE |  |  |  | Incorrect or non-supported ACF type |

[Covers: req-8552172-00905318]

[Covers: req-8552172-00905319]

[Covers: req-8552172-00905320]

[Covers: req-8552172-00905321]

[Covers: req-8552172-00905221]

[Covers: req-8552172-00903613]

[Covers: req-8552172-00904663]

[Covers: req-8552172-00904662]

[Covers: req-8552172-00905224]

[Covers: req-8552172-00905235]

[Covers: req-8552172-00905234]

[Covers: req-8552172-00905238]

[Covers: req-8552172-00905225]

[Covers: req-8552172-00905237]

[Covers: req-8552172-00905226]

[Covers: req-8552172-00904664]

[Covers: req-8552172-00904665]

[Covers: req-8552172-00904666]

[Covers: req-8552172-00909476]

[Covers: req-8552172-00909478]

[Covers: req-8552172-00528003]

[Covers: req-8552172-00528002]

[Covers: req-8552172-00528006]

[Covers: req-8552172-00527993]

[Covers: req-8552172-00527980]

#### ProtoConv_GetSendStatus

[Gen5GW_ProtoConv_AD_C_13_003]

Table 3-23: ProtoConv_GetSendStatus

| Function Name | ProtoConv_GetSendStatus |  |  |  |
| --- | --- | --- | --- | --- |
| ASIL Level | ASIL D | Status | New |  |
| Syntax | void ProtoConv_GetSendStatus(uint32 *pStatus) |  |  |  |
| Sync/Async | Synchronous |  |  |  |
| Reentrancy | Non-Reentrant |  |  |  |
| Interrupt State | Runs in the interrupt enable state |  |  |  |
| Parameters (In) | N/A |  | - |  |
| Parameters (In-Out) | N/A |  | - |  |
| Parameters (Out) | uint32 *pStatus |  | Pointer to store CAN/LIN/Parallel Port packet transmission suppression setting information |  |
|  |  |  | Range | Not NULL_PTR |
| Return Value | N/A |  | - |  |
| Description | Set the value of the global variable (GulSendStatus) that stores the CAN/LIN/Parallel Port packet sending suppression setting information to the pointer (*pStatus) that stores the argument CAN/LIN/Parallel Port packet send suppression setting information. |  |  |  |
| Preconditions | Initialized state |  |  |  |
| Remarks | Refer to the following items for the contents stored in *pStatus, which stores the CAN/LIN/Parallel Port packet transmission suppression setting information. 2.1.5 Ethernet to X Conversion Function This software receives Ethernet frames when a user application calls the software's API. Ethernet to CAN Conversion Function (Gen5GW_ProtoConv_AD_C_06_008) |  |  |  |
| DET/DEM Errors Handled | N/A |  |  |  |

[Covers: req-8552172-00905218]

[Covers: req-8552172-00909476]

[Covers: req-8552172-00909478]

[Covers: req-8552172-00528006]

[Covers: req-8552172-00527993]

#### ProtoConv_UpdateSendStatus

[Gen5GW_ProtoConv_AD_C_13_004]

Table 3-24: ProtoConv_UpdateSendStatus

| Function Name | ProtoConv_UpdateSendStatus |  |  |  |
| --- | --- | --- | --- | --- |
| ASIL Level | ASIL D | Status | New |  |
| Syntax | void ProtoConv_UpdateSendStatus(const uint32 ulStatus) |  |  |  |
| Sync/Async | Synchronous |  |  |  |
| Reentrancy | Non-Reentrant |  |  |  |
| Interrupt State | Runs in the interrupt enable state |  |  |  |
| Parameters (In) | const uint32 ulStatus |  | Variable to store CAN/LIN/Parallel Port packet transmission suppression setting information |  |
|  |  |  | Range | 14-bit value |
| Parameters (In-Out) | N/A |  | - |  |
| Parameters (Out) | N/A |  | - |  |
| Return Value | N/A |  | - |  |
| Description | Update the value of the global variable (GulSendStatus) that stores the CAN/LIN/Parallel Port packet sending suppression setting information by a 32-bit value (ulStatus) that stores the argument CAN/LIN/Parallel Port packet send suppression setting information. |  |  |  |
| Preconditions | Initialized state |  |  |  |
| Remarks | Refer to the following items for the contents stored in ulStatus, which stores the CAN/LIN/Parallel Port packet transmission suppression setting information. 2.1.5 Ethernet to X Conversion Function This software receives Ethernet frames when a user application calls the software's API. Ethernet to CAN Conversion Function (Gen5GW_ProtoConv_AD_C_06_008) |  |  |  |
| DET/DEM Errors Handled | N/A |  |  |  |

[Covers: req-8552172-00905219]

[Covers: req-8552172-00909476]

[Covers: req-8552172-00909478]

[Covers: req-8552172-00528006]

[Covers: req-8552172-00527993]

#### ProtoConv_EnqueueEth

[Gen5GW_ProtoConv_AD_C_13_005]

Table 3-25: ProtoConv_EnqueueEth

| Function Name | ProtoConv_EnqueueEth |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| ASIL Level | ASIL D | Status | New |  |  |
| Syntax | Std_ReturnType ProtoConv_EnqueueEth(const uint16 usFrameType, const uint8 *aaSrcMacAddr, const uint8 *pEthFrame, const uint32 ulFrameLen) |  |  |  |  |
| Sync/Async | Synchronous |  |  |  |  |
| Reentrancy | Non-Reentrant |  |  |  |  |
| Interrupt State | Runs in the interrupt enable state |  |  |  |  |
| Parameters (In) | const uint16 usFrameType |  | Frame type for Ethernet frames |  |  |
|  |  |  | Range | PROTOCONV_FRAMETYPE_AVTP (0x22F0U) PROTOCONV_FRAMETYPE_IPV4 (0x0800U) PROTOCONV_FRAMETYPE_IPV6 (0x86DDU) PROTOCONV_FRAMETYPE_8021Q (0x8100U) |  |
| Parameters (In) | const uint8 *aaSrcMacAddr |  | Address where MAC addresses to be filtered are stored |  |  |
|  |  |  | Range | Not NULL_PTR |  |
| Parameters (In) | const uint8 *pEthFrame |  | Address of buffer where Ethernet frame (Payload) is stored |  |  |
|  |  |  | Range | Not NULL_PTR |  |
| Parameters (In) | const uint32 ulFrameLen |  | Ethernet frame length (Payload) |  |  |
|  |  |  | Range | 46~1500 |  |
| Parameters (In-Out) | N/A |  | - |  |  |
| Parameters (Out) | uint32 *pSendStatus |  | Address to store CAN packet transmission results |  |  |
|  |  |  | Range | Not NULL_PTR |  |
| Return Value | Std_ReturnType |  | E_OK E_NOT_OK |  |  |
| Description | If the argument aaSrcMacAddr or pEthFrame is NULL_PTR, it returns E_NOT_OK, reports DET error PROTOCONV_E_PARAM_POINTER (if PROTOCONV_DEV_ERROR_DETECT == STD_ON) and exits. If the argument usFrameType or ulFrameLen is out of range, it returns E_NOT_OK, reports DET error PROTOCONV_E_INV_PARAM (if PROTOCONV_DEV_ERROR_DETECT == STD_ON) and exits. If the Ethernet FIFO memory is full, return E_NOT_OK and exit. Otherwise, enqueue Ethernet frame into Port Ethernet memory. If the Ethernet FIFO memory reaches the maximum configured FIFO depth, set the Tail value of Ethernet FIFO memory to NUM0, else increase the Tail value. If the Tail value and the Head value of Ethernet FIFO memory are equal, set Flag member Ethernet FIFO memory to PROTOCONV_FIFO_FULL, else set it to PROTOCONV_FIFO_AVAILABLE. If the process ends normally, E_OK is returned. |  |  |  |  |
| Preconditions | Initialized state |  |  |  |  |
| Remarks | N/A |  |  |  |  |
| DET/DEM Errors Handled | PROTOCONV_E_FIFO_FULL |  |  |  | Unexpectedly full FIFO memory |

[Covers: req-8552172-00909163]

[Covers: req-8552172-00905318]

[Covers: req-8552172-00905319]

[Covers: req-8552172-00905320]

[Covers: req-8552172-00905321]

[Covers: req-8552172-00905221]

[Covers: req-8552172-00903613]

[Covers: req-8552172-00904663]

[Covers: req-8552172-00904662]

[Covers: req-8552172-00909476]

[Covers: req-8552172-00909478]

[Covers: req-8552172-00528003]

[Covers: req-8552172-00528002]

[Covers: req-8552172-00528006]

[Covers: req-8552172-00527993]

[Covers: req-8552172-00527980]

#### ProtoConv_DequeueEth

[Gen5GW_ProtoConv_AD_C_13_006]

Table 3-26: ProtoConv_DequeueEth

| Function Name | ProtoConv_DequeueEth |  |  |  |
| --- | --- | --- | --- | --- |
| ASIL Level | ASIL D | Status | New |  |
| Syntax | Std_ReturnType ProtoConv_DequeueEth(uint16 *pFrameType, uint8 **aaSrcMacAddr, uint8 **pEthFrame, uint32 *pFrameLen) |  |  |  |
| Sync/Async | Synchronous |  |  |  |
| Reentrancy | Non-Reentrant |  |  |  |
| Interrupt State | Runs in the interrupt enable state |  |  |  |
| Parameters (In) | N/A |  | - |  |
| Parameters (In-Out) | N/A |  | - |  |
| Parameters (Out) | uint16 *pFrameType |  | Frame type for Ethernet frames |  |
|  |  |  | Range | PROTOCONV_FRAMETYPE_AVTP (0x22F0U) PROTOCONV_FRAMETYPE_IPV4 (0x0800U) PROTOCONV_FRAMETYPE_IPV6 (0x86DDU) PROTOCONV_FRAMETYPE_8021Q (0x8100U) |
| Parameters (Out) | uint8 **aaSrcMacAddr |  | Address where the MAC Address to be filtered is stored |  |
|  |  |  | Range | Not NULL_PTR |
| Parameters (Out) | uint8 **pEthFrame |  | Address of buffer where Ethernet frames are stored |  |
|  |  |  | Range | Not NULL_PTR |
| Parameters (Out) | uint32 *pFrameLen |  | Data size of Ethernet frame |  |
|  |  |  | Range | 46~1500 |
| Return Value | Std_ReturnType |  | E_OK E_NOT_OK |  |
| Description | If the Ethernet FIFO memory is empty, return E_NOT_OK, report DET error PROTOCONV_E_FIFO_EMPTY (if DET is enabled) and terminate the process. If the Ethernet FIFO memory is not empty, dequeue Port frame from Port FIFO memory. If the Ethernet FIFO memory reaches the maximum configured depth, reset the Head value of Ethernet FIFO memory to NUM0. Otherwise, increment the Head value. If the Tail value and the Head value of Ethernet FIFO memory are equal, set Flag member Ethernet FIFO memory to PROTOCONV_FIFO_EMPTY, else set it to PROTOCONV_FIFO_AVAILABLE. If the process ends normally, E_OK is returned. |  |  |  |
| Preconditions | Initialized state |  |  |  |
| Remarks | N/A |  |  |  |
| DET/DEM Errors Handled | PROTOCONV_E_FIFO_EMPTY |  |  | Unexpectedly empty FIFO memory |

[Covers: req-8552172-00909163]

[Covers: req-8552172-00905318]

[Covers: req-8552172-00905319]

[Covers: req-8552172-00905320]

[Covers: req-8552172-00905321]

[Covers: req-8552172-00905221]

[Covers: req-8552172-00903613]

[Covers: req-8552172-00904663]

[Covers: req-8552172-00904662]

[Covers: req-8552172-00909476]

[Covers: req-8552172-00909478]

[Covers: req-8552172-00528003]

[Covers: req-8552172-00528002]

[Covers: req-8552172-00528006]

[Covers: req-8552172-00527993]

[Covers: req-8552172-00527980]

#### ProtoConv_SearchCanRoutTbl

[Gen5GW_ProtoConv_AD_C_13_008]

Table 3-27: ProtoConv_SearchCanRoutTbl

| Function Name | ProtoConv_SearchCanRoutTbl |  |  |  |
| --- | --- | --- | --- | --- |
| ASIL Level | ASIL D | Status | New |  |
| Syntax | Std_ReturnType ProtoConv_SearchCanRoutTbl(const R_ProtoConv_CanFrameType *pCanFrame, uint8 *pDestBusIdx, uint32 *pAddInfo) |  |  |  |
| Sync/Async | Synchronous |  |  |  |
| Reentrancy | Non-Reentrant |  |  |  |
| Interrupt State | Runs in the interrupt enable state |  |  |  |
| Parameters (In) | const R_ProtoConv_CanFrameType *pCanFrame |  | CAN frame |  |
|  |  |  | Range | Not NULL_PTR |
| Parameters (In-Out) | N/A |  | - |  |
| Parameters (Out) | uint8 *pDestBusIdx |  | Address where destination Bus ID is stored |  |
|  |  |  | Range | Not NULL_PTR |
| Parameters (Out) | uint8 *pAddInfo |  | Address where additional information is stored |  |
|  |  |  | Range | Not NULL_PTR |
| Return Value | Std_ReturnType |  | E_OK E_NOT_OK |  |
| Description | Matching process between CAN frames and Ethernet to CAN Main Routing Table is performed. Perform the binary searching for the number of Ethernet to CAN Main Routing Table (ProtoConv_GulEthtoCanMainTblNum). If the entry CAN ID is found, perform the following process. - Save the Ethernet to CAN Sub Routing Table at the found index - Calculate the index to Ethernet to CAN Sub Routing Table - Calculate the index to Ethernet to CAN Destination Routing table - Save the destination Bus ID stored in Ethernet to CAN Destination Routing table at the calculated index to *pDestBusIdx - Save the Additional Information stored in Ethernet to CAN Destination Routing table at the calculated index to *pAddInfo Otherwise, do nothing. If the process ends normally, E_OK is returned. |  |  |  |
| Preconditions | N/A |  |  |  |
| Remarks | N/A |  |  |  |
| DET/DEM Errors Handled | N/A |  |  |  |

[Covers: req-8552172-00905318]

[Covers: req-8552172-00905321]

[Covers: req-8552172-00903613]

[Covers: req-8552172-00904663]

[Covers: req-8552172-00904662]

[Covers: req-8552172-00904664]

[Covers: req-8552172-00904665]

[Covers: req-8552172-00904666]

[Covers: req-8552172-00909476]

[Covers: req-8552172-00909478]

[Covers: req-8552172-00528003]

[Covers: req-8552172-00528002]

[Covers: req-8552172-00528006]

[Covers: req-8552172-00527993]

[Covers: req-8552172-00527980]

#### ProtoConv_UnpackCan

[Gen5GW_ProtoConv_AD_C_13_009]

Table 3-28: ProtoConv_UnpackCan

| Function Name | UnpackCan |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| ASIL Level | ASIL D | Status | New |  |  |
| Syntax | Std_ReturnType ProtoConv_UnpackCan(const uint8 *pPayload, const uint32 ulPayloadLen, uint32 *pDataPosition, R_ProtoConv_CanFrameType *pCanFrame, ProtoConv_CanMsgInfoType *pCanMsgInfo, uint8 *pErrorStatus) |  |  |  |  |
| Sync/Async | Synchronous |  |  |  |  |
| Reentrancy | Non-Reentrant |  |  |  |  |
| Interrupt State | Runs in the interrupt enable state |  |  |  |  |
| Parameters (In) | const uint8 *pPayLoad |  | Address of buffer where payload is stored |  |  |
|  |  |  | Range |  | Not NULL_PTR |
| Parameters (In) | const uint32 ulPayloadLen |  | Payload length |  |  |
|  |  |  | Range |  | 8 - 1488 (maximum Ethernet payload minus maximum NTSCF header length) |
| Parameters (In-Out) | uint32 *pDataPosition |  | (IN) Unpacking start position (OUT)Next unpacking start position |  |  |
|  |  |  | Range |  | Not NULL_PTR |
| Parameters (Out) | R_ProtoConv_CanFrameType *pCanFrame |  | CAN frame taken out |  |  |
|  |  |  | Range |  | Not NULL_PTR |
| Parameters (Out) | ProtoConv_CanMsgInfoType *pCanMsgInfo |  | CAN message information taken out |  |  |
|  |  |  | Range |  | Not NULL_PTR |
| Parameters (Out) | uint8 *pErrorStatus |  | Error status taken out |  |  |
|  |  |  | Range |  | Not NULL_PTR |
| Return Value | Std_ReturnType |  | E_OK E_NOT_OK |  |  |
| Description | Extract one CAN frame in ACF format from the payload. The value of pDataPosition shall be the frame extraction position. Retrieve the CAN message information from the frame retrieval position. [If CAN frame type is ACF_CAN] Store TimeStamp length (MSG_TIMESTAMP) to the local variable. [Otherwise] Do nothing. If adding the total value of ACF_CAN_BRIEF_MSG_INFO_LEN and local variables storing TimeStamp length to the frame extraction position value makes the data length exceeded, set PROTOCONV_E_NOTFRAME to *pErrorStatus and the process shall be terminated. [If CAN frame type is ACF_CAN] Retrieve the Nanosecond and Second member values of TimeStamp from the frame fetch position. [Otherwise] Do nothing. Retrieve the CAN ID from the frame retrieval position. Extract the ACF fields from CAN message information and store them in the correspoding members of argument CAN message information structure pCanMsgInfo. Calculate CAN DLC based on ACF frame length, TimeStamp length and padding value and store it in the local variable. If the DLC is greater than CANFRAME_LEN_MAX (64U) or adding the DLC to the updated frame retrieval position exceeds the data length, the function returns E_NOT_OK, reports DET error PROTOCONV_E_PACKEDCAN (if DET is enabled), sets *pErrorStatus= PROTOCONV_E_PACKEDCAN and terminates the process. Copy the CAN frame to the buffer. Store the retrieved CAN ID, CAN DLC and CAN data to the argument CAN frame structure Store the retrieved Nanosecond and Second values to the argument TimeStamp structure Update the frame retrieval position after extracting CAN data (*pDataPosition = frame extraction position + length of CAN message information + TimeStamp length (if it is ACF CAN/CAN FD frame) + CAN ID length + CAN DLC) If the process ends normally, E_OK is returned. |  |  |  |  |
| Preconditions | N/A |  |  |  |  |
| Remarks | The frame extraction position check is divided into two times to prevent access outside the buffer area. |  |  |  |  |
| DET/DEM Errors Handled | PROTOCONV_E_NOTFRAME |  |  | No CAN frame |  |
|  | PROTOCONV_E_PACKEDCAN |  |  | Packed CAN Frame Error |  |

[Covers: req-8552172-00905318]

[Covers: req-8552172-00905321]

[Covers: req-8552172-00903613]

[Covers: req-8552172-00904663]

[Covers: req-8552172-00904662]

[Covers: req-8552172-00904664]

[Covers: req-8552172-00904665]

[Covers: req-8552172-00904666]

[Covers: req-8552172-00909476]

[Covers: req-8552172-00909478]

[Covers: req-8552172-00528003]

[Covers: req-8552172-00528002]

[Covers: req-8552172-00528006]

[Covers: req-8552172-00527993]

[Covers: req-8552172-00527980]

#### ProtoConv_UnpackCanIPDUM

[Gen5GW_ProtoConv_AD_C_13_010]

Table 3-29: ProtoConv_UnpackCanIPDUM

| Function Name | UnpackCanIPDUM |  |  |  |
| --- | --- | --- | --- | --- |
| ASIL Level | ASIL D | Status | New |  |
| Syntax | Std_ReturnType ProtoConv_UnpackCanIPDUM(const uint8 *pPayload, const uint32 ulPayloadLen, uint32 *pDataPosition, R_ProtoConv_CanFrameType *pCanFrame, uint8 *pErrorStatus) |  |  |  |
| Sync/Async | Synchronous |  |  |  |
| Reentrancy | Non-Reentrant |  |  |  |
| Interrupt State | Runs in the interrupt enable state |  |  |  |
| Parameters (In) | const uint8 *pPayLoad |  | Address of buffer where payload is stored |  |
|  |  |  | Range | Not NULL_PTR |
| Parameters (In) | const uint32 ulPayloadLen |  | Payload length |  |
|  |  |  | Range | 8 - 1488 (maximum Ethernet payload minus maximum NTSCF header length) |
| Parameters (In-Out) | uint32 *pDataPosition |  | (IN) Unpacking start position (OUT)Next unpacking start position |  |
|  |  |  | Range | Not NULL_PTR |
| Parameters (Out) | R_ProtoConv_CanFrameType *pCanFrame |  | CAN frame taken out |  |
|  |  |  | Range | Not NULL_PTR |
| Parameters (Out) | uint8 *pErrorStatus |  | Error status taken out |  |
|  |  |  | Range | Not NULL_PTR |
| Return Value | Std_ReturnType |  | E_OK E_NOT_OK |  |
| Description | Extract one CAN frame in IPDU-M format from the payload. The value of pDataPosition shall be the frame extraction position. If adding the total value of PDU_LEN_ID and PDU_LEN_DLC to the frame extraction position value make the data length exceeded, set PROTOCONV_E_NOTFRAME to *pErrorStatus and the process shall be terminated. Extract the DLC from the frame fetch position. If the DLC is greater than CANFRAME_LEN_MAX (64U) or adding the DLC to the updated frame retrieval position exceeds the data length, the function returns E_NOT_OK, reports DET error PROTOCONV_E_PACKEDCAN (if DET is enabled), sets *pErrorStatus= PROTOCONV_E_PACKEDCAN and terminates the process. Copy the CAN frame to the buffer. Store the retrieved CAN DLC and CAN data to the argument CAN frame structure (CAN ID is already set by caller). Update the frame retrieval position after extracting CAN data (*pDataPosition = frame extraction position + PDU ID length + PDU DLC length + CAN DLC) If the process ends normally, E_OK is returned. |  |  |  |
| Preconditions | N/A |  |  |  |
| Remarks | The frame extraction position check is divided into two times to prevent access outside the buffer area. |  |  |  |
| DET/DEM Errors Handled | PROTOCONV_E_NOTFRAME |  |  | No CAN frame |
|  | PROTOCONV_E_PACKEDCAN |  |  | Packed CAN Frame Error |

[Covers: req-8552172-00905318]

[Covers: req-8552172-00905321]

[Covers: req-8552172-00905221]

[Covers: req-8552172-00903613]

[Covers: req-8552172-00904663]

[Covers: req-8552172-00904662]

[Covers: req-8552172-00904664]

[Covers: req-8552172-00904665]

[Covers: req-8552172-00904666]

[Covers: req-8552172-00909476]

[Covers: req-8552172-00909478]

[Covers: req-8552172-00528003]

[Covers: req-8552172-00528002]

[Covers: req-8552172-00528006]

[Covers: req-8552172-00527993]

[Covers: req-8552172-00527980]

#### ProtoConv_SearchLinRoutTbl

[Gen5GW_ProtoConv_AD_C_13_012]

Table 3-30: ProtoConv_SearchLinRoutTbl

| Function Name | SearchLinRoutTbl |  |  |  |
| --- | --- | --- | --- | --- |
| ASIL Level | ASIL D | Status | New |  |
| Syntax | Std_ReturnType ProtoConv_SearchLinRoutTbl(const R_ProtoConv_LinFrameType *pLinFrame, uint8 *pDestBusIdx, uint32 *pAddInfo) |  |  |  |
| Sync/Async | Synchronous |  |  |  |
| Reentrancy | Non-Reentrant |  |  |  |
| Interrupt State | Runs in the interrupt enable state |  |  |  |
| Parameters (In) | const R_ProtoConv_LinFrameType *pLinFrame |  | LIN frame |  |
|  |  |  | Range | Not NULL_PTR |
| Parameters (In-Out) | N/A |  | - |  |
| Parameters (Out) | uint8 *pDestBusIdx |  | Address where destination Bus ID is stored |  |
|  |  |  | Range | Not NULL_PTR |
| Parameters (Out) | uint8 *pAddInfo |  | Address where additional information is stored |  |
|  |  |  | Range | Not NULL_PTR |
| Return Value | Std_ReturnType |  | E_OK E_NOT_OK |  |
| Description | Matching process between LIN frames and Ethernet to LIN Main Routing Table is performed. Perform the binary searching for the number of Ethernet to LIN Main Routing Table (ProtoConv_GulEthtoCanMainTblNum). If the entry LIN ID is found, perform the following process. - Save the Ethernet to LIN Sub Routing Table at the found index - Calculate the index to Ethernet to LIN Sub Routing Table - Calculate the index to Ethernet to LIN Destination Routing table - Save the destination Bus ID stored in Ethernet to LIN Destination Routing table at the calculated index to *pDestBusIdx - Save the Additional Information stored in Ethernet to LIN Destination Routing table at the calculated index to *pAddInfo Otherwise, do nothing. If the process ends normally, E_OK is returned. |  |  |  |
| Preconditions | N/A |  |  |  |
| Remarks | N/A |  |  |  |
| DET/DEM Errors Handled | N/A |  |  |  |

[Covers: req-8552172-00905319]

[Covers: req-8552172-00905321]

[Covers: req-8552172-00903613]

[Covers: req-8552172-00904663]

[Covers: req-8552172-00904662]

[Covers: req-8552172-00904664]

[Covers: req-8552172-00904665]

[Covers: req-8552172-00904666]

[Covers: req-8552172-00909476]

[Covers: req-8552172-00909478]

[Covers: req-8552172-00528003]

[Covers: req-8552172-00528002]

[Covers: req-8552172-00528006]

[Covers: req-8552172-00527993]

[Covers: req-8552172-00527980]

#### ProtoConv_UnpackLin

[Gen5GW_ProtoConv_AD_C_13_013]

Table 3-31: ProtoConv_UnpackLin

| Function Name | UnpackLin |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| ASIL Level | ASIL D | Status | New |  |  |
| Syntax | Std_ReturnType ProtoConv_UnpackLin(const uint8 *pPayload, const uint32 ulPayloadLen, uint32 *pDataPosition, R_ProtoConv_LinFrameType *pLinFrame, ProtoConv_LinMsgInfoType *pLinMsgInfo, uint8 *pErrorStatus) |  |  |  |  |
| Sync/Async | Synchronous |  |  |  |  |
| Reentrancy | Non-Reentrant |  |  |  |  |
| Interrupt State | Runs in the interrupt enable state |  |  |  |  |
| Parameters (In) | const uint8 *pPayLoad |  | Address of buffer where payload is stored |  |  |
|  |  |  | Range |  | Not NULL_PTR |
| Parameters (In) | const uint32 ulPayloadLen |  | Payload length |  |  |
|  |  |  | Range |  | 8 - 1488 (maximum Ethernet payload minus maximum NTSCF header length) |
| Parameters (In-Out) | uint32 *pDataPosition |  | (IN) Unpacking start position (OUT)Next unpacking start position |  |  |
|  |  |  | Range |  | Not NULL_PTR |
| Parameters (Out) | R_ProtoConv_LinFrameType *pLinFrame |  | LIN frame taken out |  |  |
|  |  |  | Range |  | Not NULL_PTR |
| Parameters (Out) | ProtoConv_LinMsgInfoType *pLinMsgInfo |  | LIN message information taken out |  |  |
|  |  |  | Range |  | Not NULL_PTR |
| Parameters (Out) | uint8 *pErrorStatus |  | Error status taken out |  |  |
|  |  |  | Range |  | Not NULL_PTR |
| Return Value | Std_ReturnType |  | E_OK E_NOT_OK |  |  |
| Description | Extract one LIN frame in ACF format from the payload. The value of pDataPosition shall be the frame extraction position. Retrieve the LIN message information from the frame retrieval position. If adding a minimum ACF LIN frame (DLC = 0) to the frame extraction position value makes the data length exceeded, set PROTOCONV_E_NOTFRAME to *pErrorStatus and the process shall be terminated. Retrieve the Nanosecond and Second member values of the pseudo TimeStamp from the frame fetch position. Retrieve the LIN ID from the frame retrieval position. Extract the ACF fields from LIN message information and store them in the correspoding members of argument LIN message information structure pLinMsgInfo. Calculate LIN DLC based on ACF frame length and padding value and store it in the local variable. If the DLC is greater than LINFRAME_LEN_MAX (8U) or adding the DLC to the updated frame retrieval position exceeds the data length, the function returns E_NOT_OK, reports DET error PROTOCONV_E_PACKEDLIN (if DET is enabled), sets *pErrorStatus= PROTOCONV_E_PACKEDLIN and terminates the process. Copy the LIN frame to the buffer. Store the retrieved LIN ID, LIN DLC and LIN data to the argument LIN frame structure. Update the frame retrieval position after extracting LIN data (*pDataPosition = frame extraction position + length of LIN message information + LIN ID length + LIN DLC) If the process ends normally, E_OK is returned. |  |  |  |  |
| Preconditions | N/A |  |  |  |  |
| Remarks | The frame extraction position check is divided into two times to prevent access outside the buffer area. |  |  |  |  |
| DET/DEM Errors Handled | PROTOCONV_E_NOTFRAME |  |  | No LIN frame |  |
|  | PROTOCONV_E_PACKEDLIN |  |  | Packed LIN Frame Error |  |

[Covers: req-8552172-00905319]

[Covers: req-8552172-00905321]

[Covers: req-8552172-00903613]

[Covers: req-8552172-00904663]

[Covers: req-8552172-00904662]

[Covers: req-8552172-00904664]

[Covers: req-8552172-00904665]

[Covers: req-8552172-00904666]

[Covers: req-8552172-00909476]

[Covers: req-8552172-00909478]

[Covers: req-8552172-00528003]

[Covers: req-8552172-00528002]

[Covers: req-8552172-00528006]

[Covers: req-8552172-00527993]

[Covers: req-8552172-00527980]

#### ProtoConv_UnpackLinIPDUM

[Gen5GW_ProtoConv_AD_C_13_014]

Table 3-32: ProtoConv_UnpackLinIPDUM

| Function Name | UnpackLinIPDUM |  |  |  |
| --- | --- | --- | --- | --- |
| ASIL Level | ASIL D | Status | New |  |
| Syntax | Std_ReturnType ProtoConv_UnpackLinIPDUM(const uint8 *pPayload, const uint32 ulPayloadLen, const uint32 ulEntryNum, uint32 *pDataPosition, R_ProtoConv_LinFrameType *pLinFrame, uint8 *pErrorStatus) |  |  |  |
| Sync/Async | Synchronous |  |  |  |
| Reentrancy | Non-Reentrant |  |  |  |
| Interrupt State | Runs in the interrupt enable state |  |  |  |
| Parameters (In) | const uint8 *pPayLoad |  | Address of buffer where payload is stored |  |
|  |  |  | Range | Not NULL_PTR |
| Parameters (In) | const uint32 ulPayloadLen |  | Payload length |  |
|  |  |  | Range | 8 - 1488 (maximum Ethernet payload minus maximum NTSCF header length) |
| Parameters (In) | const uint32 *pEntryNum |  | Address of routing table index |  |
|  |  |  | Range | Not NULL_PTR |
| Parameters (In-Out) | uint32 *pDataPosition |  | (IN) Unpacking start position (OUT)Next unpacking start position |  |
|  |  |  | Range | Not NULL_PTR |
| Parameters (Out) | R_ProtoConv_LinFrameType *pLinFrame |  | LIN frame taken out |  |
|  |  |  | Range | Not NULL_PTR |
| Parameters (Out) | uint8 *pErrorStatus |  | Error status taken out |  |
|  |  |  | Range | Not NULL_PTR |
| Return Value | Std_ReturnType |  | E_OK E_NOT_OK |  |
| Description | Extract one LIN frame in IPDU-M format from the payload. The value of pDataPosition shall be the frame extraction position. If adding the total value of PDU_LEN_ID and PDU_LEN_DLC to the frame extraction position value make the data length exceeded, set PROTOCONV_E_NOTFRAME to *pErrorStatus and the process shall be terminated. Extract the DLC from the frame fetch position. If the DLC is greater than LINFRAME_LEN_MAX (8U) or adding the DLC to the updated frame retrieval position exceeds the data length, the function returns E_NOT_OK, reports DET error PROTOCONV_E_PACKEDLIN (if DET is enabled), sets *pErrorStatus= PROTOCONV_E_PACKEDLIN and terminates the process. Copy the LIN frame to the buffer. Store the retrieved LIN DLC and LIN data to the argument LIN frame structure (LIN ID is already set by caller). Update the frame retrieval position after extracting LIN data (*pDataPosition = frame extraction position + PDU ID length + PDU DLC length + LIN DLC) If the process ends normally, E_OK is returned. |  |  |  |
| Preconditions | N/A |  |  |  |
| Remarks | The frame extraction position check is divided into two times to prevent access outside the buffer area. |  |  |  |
| DET/DEM Errors Handled | PROTOCONV_E_NOTFRAME |  |  | No LIN frame |
|  | PROTOCONV_E_PACKEDLIN |  |  | Packed LIN Frame Error |

[Covers: req-8552172-00905319]

[Covers: req-8552172-00905321]

[Covers: req-8552172-00905221]

[Covers: req-8552172-00903613]

[Covers: req-8552172-00904663]

[Covers: req-8552172-00904662]

[Covers: req-8552172-00904664]

[Covers: req-8552172-00904665]

[Covers: req-8552172-00904666]

[Covers: req-8552172-00909476]

[Covers: req-8552172-00909478]

[Covers: req-8552172-00528003]

[Covers: req-8552172-00528002]

[Covers: req-8552172-00528006]

[Covers: req-8552172-00527993]

[Covers: req-8552172-00527980]

#### ProtoConv_UnpackPort

[Gen5GW_ProtoConv_AD_C_13_015]

Table 3-33: ProtoConv_UnpackPort

| Function Name | UnpackLin |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| ASIL Level | ASIL D | Status | New |  |  |
| Syntax | Std_ReturnType ProtoConv_UnpackPort(const uint8 *pPayload, const uint32 ulPayloadLen, uint32 *pDataPosition, uint16 *pLength, uint8 *pErrorStatus) |  |  |  |  |
| Sync/Async | Synchronous |  |  |  |  |
| Reentrancy | Non-Reentrant |  |  |  |  |
| Interrupt State | Runs in the interrupt enable state |  |  |  |  |
| Parameters (In) | const uint8 *pPayLoad |  | Address of buffer where payload is stored |  |  |
|  |  |  | Range |  | Not NULL_PTR |
| Parameters (In) | const uint32 ulPayloadLen |  | Payload length |  |  |
|  |  |  | Range |  | 8 - 1488 (maximum Ethernet payload minus maximum NTSCF header length) |
| Parameters (In-Out) | uint32 *pDataPosition |  | (IN) Unpacking start position (OUT)Next unpacking start position |  |  |
|  |  |  | Range |  | Not NULL_PTR |
| Parameters (In-Out) | uint16 *pLength |  | Value of Port Data length |  |  |
|  |  |  | Range |  | Not NULL_PTR |
| Parameters (Out) | uint8 *pErrorStatus |  | Error status taken out |  |  |
|  |  |  | Range |  | Not NULL_PTR |
| Return Value | Std_ReturnType |  | E_OK E_NOT_OK |  |  |
| Description | Extract data length and Port data from a ACF Parallel Port message in the payload. Copy value of Total Port data length argument *pLength to the local variable LusLength. The value of pDataPosition shall be the frame extraction position. If adding the total value of ACF_PORT_MSG_INFO_LEN to the frame extraction position value makes the data length exceeded, set PROTOCONV_E_NOTFRAME to *pErrorStatus and the process shall be terminated. Retrieve the Port message information from the frame retrieval position. Extract the ACF fields from Port message information. If the extracted value of bit_width field is PORT_BIT_WIDTH_MIN (0U), set the local variable storing the Data length of the current extracting frame LusPortDataLen and the local variable storing the total payload length of ACF Parallel message to MAX_PORT_UNIT_LEN (32U). If the extracted value of bit_width field is not PORT_BIT_WIDTH_MIN (0U), calculate the data length of the current extracting frame and the total payload length corresponding with the bit_width value, then store them to the local variables. Check if the ACF length does not match with the bit_width value or the bit_width value is out of range, the function returns E_NOT_OK, reports DET error PROTOCONV_E_PACKEDPORT (if DET is enabled), sets *pErrorStatus= PROTOCONV_E_PACKEDPORT and terminates the process. Copy the Port Data to the buffer. Add the current extracted data length to the the total value of retrieved Port data length (LusLength = LusLength + LusPortDataLen) Update the frame retrieval position after extracting LIN data (*pDataPosition = LulFramePos + (uint32) LusRequiredLen) Update the total value of Port data length after extracting Port data (*pLength = LusLength) If the process ends normally, E_OK is returned. |  |  |  |  |
| Preconditions | N/A |  |  |  |  |
| Remarks | The frame extraction position check is divided into two times to prevent access outside the buffer area. |  |  |  |  |
| DET/DEM Errors Handled | PROTOCONV_E_NOTFRAME |  |  | No Port frame |  |
|  | PROTOCONV_E_PACKEDPORT |  |  | Packed Port Frame Error |  |

[Covers: req-8552172-00905320]

[Covers: req-8552172-00905321]

[Covers: req-8552172-00903613]

[Covers: req-8552172-00904663]

[Covers: req-8552172-00904662]

[Covers: req-8552172-00904664]

[Covers: req-8552172-00904665]

[Covers: req-8552172-00904666]

[Covers: req-8552172-00909476]

[Covers: req-8552172-00909478]

[Covers: req-8552172-00528003]

[Covers: req-8552172-00528002]

[Covers: req-8552172-00528006]

[Covers: req-8552172-00527993]

[Covers: req-8552172-00527980]

#### ProtoConv_UnpackPortIPDUM

[Gen5GW_ProtoConv_AD_C_13_016]

Table 3-34: ProtoConv_UnpackPortIPDUM

| Function Name | UnpackPortIPDUM |  |  |  |
| --- | --- | --- | --- | --- |
| ASIL Level | ASIL D | Status | New |  |
| Syntax | Std_ReturnType ProtoConv_UnpackPortIPDUM(const uint8 *pPayload, const uint32 ulPayloadLen, uint32 *pDataPosition, uint16 *pLength, uint8 *pErrorStatus) |  |  |  |
| Sync/Async | Synchronous |  |  |  |
| Reentrancy | Non-Reentrant |  |  |  |
| Interrupt State | Runs in the interrupt enable state |  |  |  |
| Parameters (In) | const uint8 *pPayLoad |  | Address of buffer where payload is stored |  |
|  |  |  | Range | Not NULL_PTR |
| Parameters (In) | const uint32 ulPayloadLen |  | Payload length |  |
|  |  |  | Range | 8 - 1488 (maximum Ethernet payload minus maximum NTSCF header length) |
| Parameters (In) | const uint32 *pEntryNum |  | Address of routing table index |  |
|  |  |  | Range | Not NULL_PTR |
| Parameters (In-Out) | uint32 *pDataPosition |  | (IN) Unpacking start position (OUT)Next unpacking start position |  |
|  |  |  | Range | Not NULL_PTR |
| Parameters (In-Out) | uint16 *pLength |  | Value of taken-out Port Data length |  |
|  |  |  | Range | Not NULL_PTR |
| Parameters (Out) | uint8 *pErrorStatus |  | Error status taken out |  |
|  |  |  | Range | Not NULL_PTR |
| Return Value | Std_ReturnType |  | E_OK E_NOT_OK |  |
| Description | Extract data length and Port data from a Port message in the IPDU-M Ethernet payload. Copy value of Total Port data length argument *pLength to the local variable LusLength. The value of pDataPosition shall be the frame extraction position. If adding the total value of PDU_LEN_ID and PDU_LEN_DLC to the frame extraction position value make the data length exceeded, set PROTOCONV_E_NOTFRAME to *pErrorStatus and the process shall be terminated. Extract the data length of Port from the frame fetch position. If the data length of Port is greater than MAX_PORT_TOTAL_SIZE (256U) or adding the DLC to the updated frame retrieval position exceeds the data length, the function returns E_NOT_OK, reports DET error PROTOCONV_E_PACKEDPORT (if DET is enabled), setS *pErrorStatus= PROTOCONV_E_PACKEDPORT and terminates the process. Copy the Port frame to the buffer. Add the current extracted data length to the total value of retrieved Port data length (LusLength = LusLength + (uint16)(DLC & MASK_16BIT)) Update the frame retrieval position after extracting LIN data (*pDataPosition = LulFramePos + DLC) Update the total value of Port data length after extracting Port data (*pLength = LusLength) If the process ends normally, E_OK is returned. |  |  |  |
| Preconditions | N/A |  |  |  |
| Remarks | The frame extraction position check is divided into two times to prevent access outside the buffer area. |  |  |  |
| DET/DEM Errors Handled | PROTOCONV_E_NOTFRAME |  |  | No Port frame |
|  | PROTOCONV_E_PACKEDPORT |  |  | Packed Port Frame Error |

[Covers: req-8552172-00905320]

[Covers: req-8552172-00905321]

[Covers: req-8552172-00903613]

[Covers: req-8552172-00904663]

[Covers: req-8552172-00904662]

[Covers: req-8552172-00904664]

[Covers: req-8552172-00904665]

[Covers: req-8552172-00904666]

[Covers: req-8552172-00909476]

[Covers: req-8552172-00909478]

[Covers: req-8552172-00528003]

[Covers: req-8552172-00528002]

[Covers: req-8552172-00528006]

[Covers: req-8552172-00527993]

[Covers: req-8552172-00527980]

### Error handling

Table 3-35: Error handling List

| No. | Error Name | Error Factor | Error Handling |
| --- | --- | --- | --- |
| (1) | E_OK | Successful Completion | - |
| (2) | E_NOT_OK | Unsuccessful Completion |  |
| (3) | PROTOCONV_E_INV_PARAM | Invalid Parameter Error | Returns the error to the user application that calls this API interfaces. Error handling is performed by the user application. |
| (4) | PROTOCONV_E_TRANS | Transmit Error |  |
| (5) | PROTOCONV_E_PARAM_DATA_LENGTH | Format Error |  |
| (6) | PROTOCONV_E_PACKEDCAN | Packed CAN Frame Error |  |
| (7) | PROTOCONV_E_PACKEDLIN | Packed LIN Frame Error |  |
| (8) | PROTOCONV_E_PACKEDPORT | Packed Parallel Port Frame Error |  |
| (9) | PROTOCONV_E_PACKEDMSG | Packed CAN/LIN/Port Frame Error |  |
| (11) | PROTOCONV_E_FIFO_FULL | Unexpectedly full FIFO memory |  |
| (12) | PROTOCONV_E_FIFO_EMPTY | Unexpectedly empty FIFO memory |  |
| (13) | PROTOCONV_E_IPCKSUM | IP Header Checksum Error |  |
| (14) | PROTOCONV_E_UDPCKSUM | UDP Header Checksum Error |  |
| (15) | PROTOCONV_E_BSWNOTIFY | BSW Notification Error |  |
| (16) | PROTOCONV_E_FILTER | Filtered and discarded |  |
| (17) | PROTOCONV_E_CFG_ACFTYPE | Incorrect or non-supported ACF type |  |

## Utility Function

### Features

Table 3-36: Function List

| No. | Function Name | Access | ASIL | Source File Name |
| --- | --- | --- | --- | --- |
|  | ProtoConv_UtilMemCpy | Public | ASIL D | CDD_ProtoConv_Util.c |
|  | ProtoConv_UtilMemSet | Public | ASIL D | CDD_ProtoConv_Util.c |
|  | ProtoConv_UtilCalcSum | Public | ASIL D | CDD_ProtoConv_Util.c |
|  | ProtoConv_UtilConvMacAddr8Bit | Public | ASIL D | CDD_ProtoConv_Util.c |
|  | ProtoConv_UtilEndianConv16 | Public | ASIL D | CDD_ProtoConv_Util.c |
|  | ProtoConv_UtilEndianConv32 | Public | ASIL D | CDD_ProtoConv_Util.c |
|  | ProtoConv_UtilCanDlcCheck | Public | ASIL D | CDD_ProtoConv_Util.c |
|  | ProtoConv_UtilCanIdCheck | Public | ASIL D | CDD_ProtoConv_Util.c |

### Interfaces

#### ProtoConv_UtilMemCpy

[Gen5GW_ProtoConv_AD_C_15_001]

Table 3-37: ProtoConv_UtilMemCpy

| Function Name | ProtoConv_UtilMemCpy |  |  |  |
| --- | --- | --- | --- | --- |
| ASIL Level | ASIL D | Status | New |  |
| Syntax | void ProtoConv_UtilMemCpy(void *pDest, const void *pSrc, const uint32 ulSize) |  |  |  |
| Sync/Async | Synchronous |  |  |  |
| Reentrancy | Non-Reentrant |  |  |  |
| Interrupt State | Runs in the interrupt enable state |  |  |  |
| Parameters (In) | const void *pSrc |  | Source address |  |
|  |  |  | Range | Not NULL_PTR |
| Parameters (In) | const uint32 ulSize |  | Data size |  |
|  |  |  | Range | 0x0 - 0xFFFFFFFF |
| Parameters (In-Out) | N/A |  | - |  |
| Parameters (Out) | uint8 *pDest |  | Destination address |  |
|  |  |  | Range | Not NULL_PTR |
| Return Value | N/A |  | - |  |
| Description | Copy the ulSize bytes of data from pSrc to the memory specified by pDest. ※The same operation as the standard function memcpy() is performed. |  |  |  |
| Preconditions | N/A |  |  |  |
| Remarks | This function does not check whether the specified address is valid or not, so the user should take this into account. |  |  |  |
| DET/DEM Errors Handled | N/A |  |  |  |

[Covers: req-8552172-00903610]

[Covers: req-8552172-00903613]

[Covers: req-8552172-00904663]

[Covers: req-8552172-00904662]

[Covers: req-8552172-00905224]

[Covers: req-8552172-00905235]

[Covers: req-8552172-00905234]

[Covers: req-8552172-00905238]

[Covers: req-8552172-00905225]

[Covers: req-8552172-00905237]

[Covers: req-8552172-00905226]

[Covers: req-8552172-00904667]

[Covers: req-8552172-00905227]

[Covers: req-8552172-00905218]

[Covers: req-8552172-00905219]

[Covers: req-8552172-00904664]

[Covers: req-8552172-00904665]

[Covers: req-8552172-00904666]

#### ProtoConv_UtilMemSet

[Gen5GW_ProtoConv_AD_C_15_002]

Table 3-38: ProtoConv_UtilMemSet

| Function Name | ProtoConv_UtilMemSet |  |  |  |
| --- | --- | --- | --- | --- |
| ASIL Level | ASIL D | Status | New |  |
| Syntax | void ProtoConv_UtilMemSet(void *pDest, const uint8 ucMyChar, const uint32 ulSize) |  |  |  |
| Sync/Async | Synchronous |  |  |  |
| Reentrancy | Non-Reentrant |  |  |  |
| Interrupt State | Runs in the interrupt enable state |  |  |  |
| Parameters (In) | const uint8 ucMyChar |  | Set this data to pDest |  |
|  |  |  | Range | 0 – 0xFF |
| Parameters (In) | const uint32 ulSize |  | Data size |  |
|  |  |  | Range | 0 - 0xFFFFFFFF |
| Parameters (In-Out) | N/A |  | - |  |
| Parameters (Out) | void *pDest |  | Destination address |  |
|  |  |  | Range | Not NULL_PTR |
| Return Value | N/A |  | - |  |
| Description | ucMyChar value is repeatedly stored from pDest for ulSize. The same operation as the standard function memset() is performed. |  |  |  |
| Preconditions | N/A |  |  |  |
| Remarks | This function does not check whether the specified address is valid or not, so the user should take this into account. |  |  |  |
| DET/DEM Errors Handled | N/A |  |  |  |

[Covers: req-8552172-00903610]

[Covers: req-8552172-00903613]

[Covers: req-8552172-00904663]

[Covers: req-8552172-00904662]

[Covers: req-8552172-00905224]

[Covers: req-8552172-00905235]

[Covers: req-8552172-00905234]

[Covers: req-8552172-00905238]

[Covers: req-8552172-00905225]

[Covers: req-8552172-00905237]

[Covers: req-8552172-00905226]

[Covers: req-8552172-00904667]

[Covers: req-8552172-00905227]

[Covers: req-8552172-00905218]

[Covers: req-8552172-00905219]

[Covers: req-8552172-00904664]

[Covers: req-8552172-00904665]

[Covers: req-8552172-00904666]

#### ProtoConv_UtilCalcSum

[Gen5GW_ProtoConv_AD_C_15_003]

Table 3-39: ProtoConv_UtilCalcSum

| Function Name | ProtoConv_UtilCalcSum |  |  |  |
| --- | --- | --- | --- | --- |
| ASIL Level | ASIL D | Status | New |  |
| Syntax | void ProtoConv_UtilCalcSum(const void *pBuf, const uint16 usBytes, uint32 *pSum) |  |  |  |
| Sync/Async | Synchronous |  |  |  |
| Reentrancy | Non-Reentrant |  |  |  |
| Interrupt State | Runs in the interrupt enable state |  |  |  |
| Parameters (In) | const void *pBuf |  | Address of buffer containing data for checksum calculation |  |
|  |  |  | Range | Not NULL_PTR |
| Parameters (In) | const uint16 usBytes |  | Data length for checksum calculation |  |
|  |  |  | Range | 0 – 0xFFFF |
| Parameters (In-Out) | uint32 *pSum |  | In: Checksum value already partially calculated Out: Checksum calculation result |  |
| Parameters (Out) | N/A |  | - |  |
| Return Value | N/A |  | - |  |
| Description | The checksum is calculated based on the value stored in memory for usBytes from pBuf. |  |  |  |
| Preconditions | The address of pBuff must be in 2-byte alignment. For new operations, the input value of pSum must be cleared with NUM0. |  |  |  |
| Remarks | The checksum is a one's complement sum operation in units of 16-bit words. When storing the result in the checksum field of the IP header or UDP header, it shall be converted to one's complement. (See [6] for details.) |  |  |  |
| DET/DEM Errors Handled | N/A |  |  |  |

[Covers: req-8552172-00903610]

[Covers: req-8552172-00903613]

[Covers: req-8552172-00904663]

[Covers: req-8552172-00904662]

[Covers: req-8552172-00905224]

[Covers: req-8552172-00905235]

[Covers: req-8552172-00905234]

[Covers: req-8552172-00905238]

[Covers: req-8552172-00905225]

[Covers: req-8552172-00905237]

[Covers: req-8552172-00905226]

[Covers: req-8552172-00904667]

[Covers: req-8552172-00905227]

[Covers: req-8552172-00905218]

[Covers: req-8552172-00905219]

[Covers: req-8552172-00904664]

[Covers: req-8552172-00904665]

[Covers: req-8552172-00904666]

#### ProtoConv_UtilConvMacAddr8Bit

[Gen5GW_ProtoConv_AD_C_15_004]

Table 3-40: ProtoConv_UtilConvMacAddr8Bit

| Function Name | ProtoConv_UtilConvMacAddr8Bit |  |  |  |
| --- | --- | --- | --- | --- |
| ASIL Level | ASIL D | Status | New |  |
| Syntax | void ProtoConv_UtilConvMacAddr8Bit(const uint32 *aaInputMacAddr, uint8 *aaOutputMacAddr) |  |  |  |
| Sync/Async | Synchronous |  |  |  |
| Reentrancy | Non-Reentrant |  |  |  |
| Interrupt State | Runs in the interrupt enable state |  |  |  |
| Parameters (In) | const uint32 *aaInputMacAddr |  | MAC address to be converted |  |
|  |  |  | Range | 32-bit value |
| Parameters (In-Out) | uint8 * aaOutputMacAddr |  | Array to store converted MAC addresses |  |
| Parameters (Out) | N/A |  | - |  |
| Return Value | N/A |  | - |  |
| Description | Convert a 32-bit, 2-entry MAC address to an 8-bit, 6-entry MAC address. Store the following values of aaInputMacAddr in each element of aaOutputMacAddr. - aaOutputMacAddr[IDX_0]: (aaInputMacAddr[IDX_0] >> SHIFT_8BIT) & MASK_8BIT - aaOutputMacAddr[IDX_1]: aaInputMacAddr[IDX_0] & MASK_8BIT - aaOutputMacAddr[IDX_2]: (aaInputMacAddr[IDX_1] >> SHIFT_24BIT) - aaOutputMacAddr[IDX_3]: (aaInputMacAddr[IDX_1] >> SHIFT_16BIT) & MASK_8BIT - aaOutputMacAddr[IDX_4]: (aaInputMacAddr[IDX_1] >> SHIFT_8BIT) & MASK_8BIT - aaOutputMacAddr[IDX_5]: aaInputMacAddr[IDX_1] & MASK_8BIT |  |  |  |
| Preconditions | The aaInputMacAddr array must have 32 bits and 2 entries. The aaOutputMacAddr array must have 8 bits and 6 entries. |  |  |  |
| Remarks | N/A |  |  |  |
| DET/DEM Errors Handled | N/A |  |  |  |

[Covers: req-8552172-00903610]

[Covers: req-8552172-00903613]

[Covers: req-8552172-00904663]

[Covers: req-8552172-00904662]

[Covers: req-8552172-00905224]

[Covers: req-8552172-00905235]

[Covers: req-8552172-00905234]

[Covers: req-8552172-00905238]

[Covers: req-8552172-00905225]

[Covers: req-8552172-00905237]

[Covers: req-8552172-00905226]

[Covers: req-8552172-00904667]

[Covers: req-8552172-00905227]

[Covers: req-8552172-00905218]

[Covers: req-8552172-00905219]

[Covers: req-8552172-00904664]

[Covers: req-8552172-00904665]

[Covers: req-8552172-00904666]

#### ProtoConv_UtilEndianConv16

[Gen5GW_ProtoConv_AD_C_15_006]

Table 3-41: ProtoConv_UtilEndianConv16

| Function Name | ProtoConv_UtilEndianConv16 |  |  |
| --- | --- | --- | --- |
| ASIL Level | ASIL D | Status | New |
| Syntax | void ProtoConv_UtilEndianConv16(uint16 *pValue) |  |  |
| Sync/Async | Synchronous |  |  |
| Reentrancy | Non-Reentrant |  |  |
| Interrupt State | Runs in the interrupt enable state |  |  |
| Parameters (In) | N/A |  | - |
| Parameters (In-Out) | uint16 *pValue |  | In: value before conversion Out: Value after conversion |
| Parameters (Out) | N/A |  | - |
| Return Value | N/A |  | - |
| Description | The 16-bit value received in *pValue is endian-converted and stored in *pValue. |  |  |
| Preconditions | N/A |  |  |
| Remarks | N/A |  |  |
| DET/DEM Errors Handled | N/A |  |  |

[Covers: req-8552172-00903610]

[Covers: req-8552172-00903613]

[Covers: req-8552172-00904663]

[Covers: req-8552172-00904662]

[Covers: req-8552172-00905224]

[Covers: req-8552172-00905235]

[Covers: req-8552172-00905234]

[Covers: req-8552172-00905238]

[Covers: req-8552172-00905225]

[Covers: req-8552172-00905237]

[Covers: req-8552172-00905226]

[Covers: req-8552172-00904667]

[Covers: req-8552172-00905227]

[Covers: req-8552172-00905218]

[Covers: req-8552172-00905219]

[Covers: req-8552172-00904664]

[Covers: req-8552172-00904665]

[Covers: req-8552172-00904666]

#### ProtoConv_UtilEndianConv32

[Gen5GW_ProtoConv_AD_C_15_007]

Table 3-42: ProtoConv_UtilEndianConv32

| Function Name | ProtoConv_UtilEndianConv32 |  |  |
| --- | --- | --- | --- |
| ASIL Level | ASIL D | Status | New |
| Syntax | void ProtoConv_UtilEndianConv32(uint32 *pValue) |  |  |
| Sync/Async | Synchronous |  |  |
| Reentrancy | Non-Reentrant |  |  |
| Interrupt State | Runs in the interrupt enable state |  |  |
| Parameters (In) | N/A |  | - |
| Parameters (In-Out) | uint32 *pValue |  | In: value before conversion Out: Value after conversion |
| Parameters (Out) | N/A |  | - |
| Return Value | N/A |  | - |
| Description | The 32-bit value received in *pValue is endian-converted and stored in *pValue. |  |  |
| Preconditions | N/A |  |  |
| Remarks | N/A |  |  |
| DET/DEM Errors Handled | N/A |  |  |

[Covers: req-8552172-00903610]

[Covers: req-8552172-00903613]

[Covers: req-8552172-00904663]

[Covers: req-8552172-00904662]

[Covers: req-8552172-00905224]

[Covers: req-8552172-00905235]

[Covers: req-8552172-00905234]

[Covers: req-8552172-00905238]

[Covers: req-8552172-00905225]

[Covers: req-8552172-00905237]

[Covers: req-8552172-00905226]

[Covers: req-8552172-00904667]

[Covers: req-8552172-00905227]

[Covers: req-8552172-00905218]

[Covers: req-8552172-00905219]

[Covers: req-8552172-00904664]

[Covers: req-8552172-00904665]

[Covers: req-8552172-00904666]

#### ProtoConv_UtilCanDlcCheck

[Gen5GW_ProtoConv_AD_C_15_008]

Table 3-43: ProtoConv_UtilCanDlcCheck

| Function Name | ProtoConv_UtilCanDlcCheck |  |  |  |
| --- | --- | --- | --- | --- |
| ASIL Level | ASIL D | Status | New |  |
| Syntax | Std_ReturnType ProtoConv_UtilCanDlcCheck(const uint32 ulCanDlc) |  |  |  |
| Sync/Async | Synchronous |  |  |  |
| Reentrancy | Non-Reentrant |  |  |  |
| Interrupt State | Runs in the interrupt enable state |  |  |  |
| Parameters (In) | const uint32 ulCanDlc |  | DLC of received CAN frames |  |
|  |  |  | Range | NUM0 ~ PROTOCONV_DLC_8, PROTOCONV_DLC_12, PROTOCONV_DLC_16, PROTOCONV_DLC_20, PROTOCONV_DLC_24, PROTOCONV_DLC_32, PROTOCONV_DLC_48, PROTOCONV_DLC_64 |
| Parameters (In-Out) | N/A |  | - |  |
| Parameters (Out) | N/A |  | - |  |
| Return Value | Std_ReturnType |  | E_OK E_NOT_OK |  |
| Description | Check if the argument ulCanDlc matches one of the following PROTOCONV_DLC_8 or lower, PROTOCONV_DLC_12, PROTOCONV_DLC_16, PROTOCONV_DLC_20, PROTOCONV_DLC_24, PROTOCONV_DLC_32, PROTOCONV_DLC_48, PROTOCONV_DLC_64 If the value of the argument ulCanDlc is out of range, return E_NOT_OK If it is within the range, return E_OK. |  |  |  |
| Preconditions | N/A |  |  |  |
| Remarks | N/A |  |  |  |
| DET/DEM Errors Handled | N/A |  |  |  |

[Covers: req-8552172-00903610]

[Covers: req-8552172-00904662]

[Covers: req-8552172-00904667]

[Covers: req-8552172-00905227]

#### ProtoConv_UtilCanIdCheck

[Gen5GW_ProtoConv_AD_C_15_009]

Table 3-44: ProtoConv_UtilCanIdCheck

| Function Name | ProtoConv_UtilCanIdCheck |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| ASIL Level | ASIL D |  | Status |  | New |
| Syntax | Std_ReturnType ProtoConv_UtilCanIdCheck(const uint32 ulCanId) |  |  |  |  |
| Sync/Async | Synchronous |  |  |  |  |
| Reentrancy | Non-Reentrant |  |  |  |  |
| Interrupt State | Runs in the interrupt enable state |  |  |  |  |
| Parameters (In) | const uint32 ulCanId | DLC of received CAN frames |  |  |  |
|  |  | Range |  | 0x0 – CANID_BASE_MAX(0x7FFU): CAN base format CANID_EXTENDED_MIN(0x80000000U) - CANID_EXTENDED_MAX(0x9FFFFFFFU): CAN extended format CANFD_ID_BASE_MIN(0x40000000U) - CANFD_ID_BASE_MAX(0x400007FFU): CANFD base format CANFD_ID_EXTENDED_MIN(0xC0000000U) - CANFD_ID_EXTENDED_MAX(0xDFFFFFFFU): CANFD extended format |  |
| Parameters (In-Out) | N/A | - |  |  |  |
| Parameters (Out) | N/A | - |  |  |  |
| Return Value | Std_ReturnType | E_OK E_NOT_OK |  |  |  |
| Description | Check if the argument ulCanId matches one of the following CANID_BASE_MAX_8 or less, CANID_EXTENDED_MIN to CANID_EXTENDED_MAX, CANFD_ID_BASE_MIN to CANFD_ID_BASE_MAX, CANFD_ID_EXTENDED_MIN to CANFD_ID_EXTENDED_MAX If the value of the argument ulCanId is out of range, return E_NOT_OK If it is within the range, E_OK is returned. |  |  |  |  |
| Preconditions | N/A |  |  |  |  |
| Remarks | N/A |  |  |  |  |
| DET/DEM Errors Handled | N/A |  |  |  |  |

[Covers: req-8552172-00903610]

[Covers: req-8552172-00904662]

[Covers: req-8552172-00904667]

[Covers: req-8552172-00905227]

### Error handling

Table 3-45: Error handling List

| No. | Error Name | Error Factor | Error Handling |
| --- | --- | --- | --- |
| (1) | E_OK | Successful Completion | - |
| (2) | E_NOT_OK | Unsuccessful Completion | - |

# File Structure

[Gen5GW_ProtoConv_AD_C_04_001]

[iso26-18_p6_7.4.5] a

Protocol Conversion consists of files with the following names (brackets are not included in file names).

CDD_ProtoConv_<module>[_<suffix>].<extension>

Where <module> is an abbreviation for modules that constitute Protocol Conversion that are arbitrarily divided.

<module> must not contain underline.

<extension> is “h” for C header, “c” for C source.

The part in square brackets [_<suffix>] can be omitted or can be used when further division of the module is necessary.

The combination of <module>, <suffix> and <extension> must be unique in Protocol Conversion files.

Refer to [3] for the rules that are the basis of above naming convention.

Conventions:

Inclusion

init.c

init.h

init c source includes init C header

e.g.) in init.c #include “init.h”

Non-Safety related

Safety related

Sub-component

Component

Figure 4-1: File Structure

[Covers: req-8552172-00527992]

[Covers: req-8552172-00527985]

[Covers: req-8552172-00527986]

The source of Protocol Conversion consists of the following modules.

Modules are an abbreviation for Functions that constitute Protocol Conversion that are arbitrarily divided.

Table 4-1: Module List

| No. | Module | Function | Sub-component | Description |
| --- | --- | --- | --- | --- |
| (1) | - (interface) | Protocol Conversion Interface | - | It provides DET code error, service IDs of functions, the Protocol Conversion Interface, type definitions, and included header files from the upper layer. |
| (2) | - (ProtoConv common) | Protocol Conversion common | - | It provides common definitions used in this module. |
| (3) | api | Protocol Conversion API | - | It provides the Protocol Conversion API for using Protocol Conversion from the outside. |
| (4) | init | Initialization Function | Initialization | It processes the Initialization function when the related API is called. |
| (5) | CantoEth | CAN to Ethernet Conversion Function | CAN to Ethernet Conversion Function | It processes the CAN to Ethernet Conversion function when the related interface is called. |
| (6) | LintoEth | LIN to Ethernet Conversion Function | LIN to Ethernet Conversion Function | It processes the LIN to Ethernet Conversion function when the related interface is called. |
| (7) | PorttoEth | Port to Ethernet Conversion Function | Port to Ethernet Conversion Function | It processes the Port to Ethernet Conversion function when the related interface is called. |
| (8) | MixtoEth | Mix to Ethernet Conversion Function | Mix to Ethernet Conversion Function | It processes the CAN/LIN/Parallel Port to Ethernet Conversion function when the related interface is called. |
| (9) | EthtoCan | Ethernet to CAN Conversion Function | Ethernet to CAN Conversion Function | It processes the Ethernet to CAN Conversion function when the related interface is called. |
| (10) | EthtoLin | Ethernet to LIN Conversion Function | Ethernet to LIN Conversion Function | It processes the Ethernet to LIN Conversion function when the related interface is called. |
| (11) | EthtoPort | Ethernet to Port Conversion Function | Ethernet to Port Conversion Function | It processes the Ethernet to Port Conversion function when the related interface is called. |
| (12) | interface (User Defined) | Interface Function | - | User can implement interface function in this module. |
| (13) | Util | Utilities Function | Utilities Function | Modules of common functions such as memory manipulation and checksum calculation. |

## Source File List

[Gen5GW_ProtoConv_AD_C_04_002]

[iso26-18_p6_7.4.5] a

The source files included in each module are following. The numbers correspond to in Table 4-1.

Table 4-2: Source File List

| No. | Source File Name | Description |
| --- | --- | --- |
|  | CDD_ProtoConv_api.c | It contains the process of APIs of Protocol Conversion. |
|  | CDD_ProtoConv_init.c | It contains the process of Initialization functions. |
|  | CDD_ProtoConv_CantoEth.c | It contains the process of CAN to Ethernet Conversion. |
|  | CDD_ProtoConv_LintoEth.c | It contains the process of LIN to Ethernet Conversion. |
|  | CDD_ProtoConv_PorttoEth.c | It contains the process of Port to Ethernet Conversion. |
|  | CDD_ProtoConv_Fifo.c | It contains the process of buffering CAN/LIN/Parallel Port messages to mixed-type Ethernet frame. |
|  | CDD_ProtoConv_XtoEth.c | It contains the process of creating and sending the converted Ethernet frame. |
|  | CDD_ProtoConv_EthtoCan.c | It contains the detailed mutual process used in the general Conversion of Ethernet to CAN/LIN/Port or only for converting Ethernet to CAN. |
|  | CDD_ProtoConv_EthtoLin.c | It contains the detailed mutual process used in the general Conversion of Ethernet to CAN/LIN/Port or only for converting Ethernet to LIN. |
|  | CDD_ProtoConv_EthtoPort.c | It contains the detailed mutual process used in the general Conversion of Ethernet to CAN/LIN/Port or only for converting Ethernet to Port. |
|  | CDD_ProtoConv_EthtoX.c | It contains the detailed process of Ethernet to CAN/LIN/Port Conversion. |
|  | CDD_ProtoConv_Util.c | It contains the common functions of Protocol Conversion. |
|  | CDD_ProtoConv_PBcfg.c | It contains the post-build configuration data. |

[Covers: req-8552172-00528014]

[Covers: req-8552172-00527982]

## Header File List

[Gen5GW_ProtoConv_AD_C_04_003]

[iso26-18_p6_7.4.5] a

The header files included in each module are shown below. The numbers correspond to in Table 4-1.

Table 4-3: Header File List

| No. | Header File Name | Description |
| --- | --- | --- |
|  | CDD_ProtoConv.h | It defines DET code error, service IDs of functions, the Protocol Conversion Interface and type definitions. |
|  | CDD_ProtoConv_common.h | It defines the common definitions in this module. |
|  | CDD_ProtoConv_api.h | It provides the interface of “CDD_ProtoConv_api.c”. |
|  | CDD_ProtoConv_init.h | It provides the interface of “CDD_ProtoConv_init.c”. |
|  | CDD_ProtoConv_CantoEth.h | It provides the interface of “CDD_ProtoConv_CantoEth.c”. |
|  | CDD_ProtoConv_LintoEth.h | It provides the interface of “CDD_ProtoConv_LintoEth.c”. |
|  | CDD_ProtoConv_PorttoEth.h | It provides the interface of “CDD_ProtoConv_PorttoEth.c”. |
|  | CDD_ProtoConv_Fifo.h | It provides the interface of “CDD_ProtoConv_MixtoEth.c”. |
|  | CDD_ProtoConv_XtoEth.h | It provides the interface of “CDD_ProtoConv_XtoEth.c”. |
|  | CDD_ProtoConv_EthtoCan.h | It provides the interface of “CDD_ProtoConv_EthtoCan.c”. |
|  | CDD_ProtoConv_EthtoLin.h | It provides the interface of “CDD_ProtoConv_EthtoLin.c”. |
|  | CDD_ProtoConv_EthtoPort.h | It provides the interface of “CDD_ProtoConv_EthtoPort.c”. |
|  | CDD_ProtoConv_EthtoX.h | It provides the interface of “CDD_ProtoConv_EthtoX.c”. |
|  | CDD_ProtoConv_interface.h | It provides the interface of “CDD_ProtoConv_interface.c” |
|  | CDD_ProtoConv_Util.h | It provides the interface of “CDD_ProtoConv_Util.c”. |
|  | CDD_ProtoConv_Cfg.h | It provides various driver Pre-compile time parameters. |

[Covers: req-8552172-00527983]

# Configuration and Calibration Design

[iso26-18_p6_C.4.1]; [iso26-18_p6_C.4.6]; [iso26-18_p6_C.4.10]

Protocol Conversion Component does not have configuration and calibration design.

# State Transition

[iso26-18_p6_7.4.5] b

Legend:

Trigger to change state

State

Initial State

<Function call / Action / Event>

<State name>

## Summary of State

Table 6-1: State List Table

| No. | State Name | Description |
| --- | --- | --- |
| (1) | Uninitialized state | This state is the default software state immediately after booting the system. |
| (2) | Initialized state | Protocol Conversion initialized. |

State Transition Diagram:

Figure 6-1: State Transition Diagram

State Transition Table:

Table 6-2: State Transition Table

| Current State Event | Uninitialized state | Initialized state |
| --- | --- | --- |
| Initialization API call | [Success API call] Initialized State | - |

## Send State

Table 6-3: State List Table

| No. | State Name | Description |
| --- | --- | --- |
| (1) | Send Enable state | This state is the default state after CAN Routing initialization. And this state allows CAN packet sending. |
| (2) | Send Disable state | This state disallows CAN packet sending. |

State Transition Diagram:

At the time after initialization, all channels are ready for transmission.

Transmittable/non-transmittable status has a status for each channel independently.

Figure 6-2: State Transition Diagram

State Transition Table:

Table 6-4: State Transition Table

| Current State Event | Send Enable state | Send Disable state |
| --- | --- | --- |
| UpdateSendStatus API call | - | [Success API call] Send Enable State |
| UpdateSendStatus API call | [Success API call] Send Disable State | - |

# Control Flow Design

[iso26-18_p6_7.4.5] b

This section shows sequence diagrams for the APIs of software components.

Conventions:

Message

Reply message

<Function name>

<Message name>

<Element>

<Element type>

Hardware Element

Lifeline

## Sequence diagram

### R_ProtoConv_Init

Figure 7-1: Sequence diagram of R_ProtoConv_Init

### R_ProtoConv_CantoEthTS

Figure 7-2: Sequence diagram of R_ProtoConv_CantoEthTS

There is no restriction on the calling order of R_ProtoConv_MainFunction() and R_ProtoConv_CantoEthTS() due to the exclusive control using the interrupt disable process (ProtoConv_EnterCriticalUserDef()) and interrupt enable process ProtoConv_ExitCriticalUserDef()).

### R_ProtoConv_CantoEth

Figure 7-3: Sequence diagram of R_ProtoConv_CantoEth

There is no restriction on the calling order of R_ProtoConv_MainFunction() and R_ProtoConv_CantoEth() due to the exclusive control using the interrupt disable and allow processes (ProtoConv_EnterCriticalUserDef(), ProtoConv_ExitCriticalUserDef()).

### R_ProtoConv_LintoEth

Figure 7-4: Sequence diagram of R_ProtoConv_LintoEth

There is no restriction on the calling order of R_ProtoConv_MainFunction() and R_ProtoConv_LintoEth() due to the exclusive control using the interrupt disable and allow processes (ProtoConv_EnterCriticalUserDef(), ProtoConv_ExitCriticalUserDef()).

### R_ProtoConv_PorttoEth

Figure 7-5: Sequence diagram of R_ProtoConv_PorttoEth

There is no restriction on the calling order of R_ProtoConv_MainFunction() and R_ProtoConv_PorttoEth() due to the exclusive control using the interrupt disable process (ProtoConv_EnterCriticalUserDef()) and interrupt enable process ProtoConv_ExitCriticalUserDef()).

### R_ProtoConv_TriggerXtoEthTx

Figure 7-6: Sequence diagram of R_ProtoConv_TriggerXtoEthTx

There is no restriction on the calling order of R_ProtoConv_MainFunction() and R_ProtoConv_TriggerXtoEthTx() due to the exclusive control using the interrupt disable process (ProtoConv_EnterCriticalUserDef()) and interrupt enable process ProtoConv_ExitCriticalUserDef()).

### R_ProtoConv_MainFunction

Figure 7-7: Sequence diagram of R_ProtoConv_MainFunction

### R_ProtoConv_EthtoX

Figure 7-8: Sequence diagram of R_ProtoConv_EthtoX

### R_ProtoConv_GetSendStatus

Figure 7-9: Sequence diagram of R_ProtoConv_GetSendStatus

### R_ProtoConv_UpdateSendStatus

Figure 7-10: Sequence diagram of R_ProtoConv_UpdateSendStatus

### R_ProtoConv_EnqueueCan

Figure 7-11: Sequence diagram of R_ProtoConv_EnqueueCan

There is no restriction on the calling order of R_ProtoConv_MainFunction() and R_ProtoConv_EnqueueCan() due to the exclusive control using the interrupt disable process (ProtoConv_EnterCriticalUserDef()) and interrupt enable process ProtoConv_ExitCriticalUserDef()).

### R_ProtoConv_EnqueueLin

Figure 7-12: Sequence diagram of R_ProtoConv_EnqueueLin

There is no restriction on the calling order of R_ProtoConv_MainFunction() and R_ProtoConv_EnqueueLin() due to the exclusive control using the interrupt disable process (ProtoConv_EnterCriticalUserDef()) and interrupt enable process ProtoConv_ExitCriticalUserDef()).

### R_ProtoConv_EnqueuePort

Figure 7-13: Sequence diagram of R_ProtoConv_EnqueuePort

There is no restriction on the calling order of R_ProtoConv_MainFunction() and R_ProtoConv_EnqueuePort() due to the exclusive control using the interrupt disable process (ProtoConv_EnterCriticalUserDef()) and interrupt enable process ProtoConv_ExitCriticalUserDef()).

### R_ProtoConv_EnqueueEth

Figure 7-14: Sequence diagram of R_ProtoConv_EnqueueEth

### R_ProtoConv_DequeueEth

Figure 7-15: Sequence diagram of R_ProtoConv_DequeueEth

### R_ProtoConv_TriggerQueueDataXtoEth

Figure 7-16: Sequence diagram of R_ProtoConv_TriggerQueueDataXtoEth

There is no restriction on the calling order of R_ProtoConv_MainFunction() and R_ProtoConv_TriggerQueueDataXtoEth() due to the exclusive control using the interrupt disable process (ProtoConv_EnterCriticalUserDef()) and interrupt enable process ProtoConv_ExitCriticalUserDef()).

## Error Handling Sequence

This section shows Error handling sequence diagrams.

### API error on Protocol Conversion

The component interface sequence of the API error in Protocol Conversion is shown below.

Figure 7-17: Interface Sequence of API error in Protocol Conversion

# Data Flow Design

[iso26-18_p6_7.4.5] b

- [x-y] in the figure shows Functionality (x) and Function Overview (y) of Figure 8-1.

- The relationship between Functionality used in Figure 8-1 and sub-components of Protocol Conversion is shown in Table 8-1: Conventions.

Table 8-1: Conventions

## R_ProtoConv_Init

Figure 8-1: Data Flow diagram (R_ProtoConv_Init)

Table 8-2: R_ProtoConv_Init

| API Function |  | Functionally |  | Software Sub-component Name |
| --- | --- | --- | --- | --- |
| 1 | R_ProtoConv_Init | 1 | The User Application initializes Protocol Conversion. Therefore, specify the following data and call API. - Pointer to the initialization configuration. | API Layer |
|  |  | 2 | Reads the following data. - Initialization configuration. |  |
|  |  | 3 | After the Initialization API “R_ProtoConv_Init” processes, it returns the following to the User Application. - Result as Std_ReturnType (normally be used with value E_OK or E_NOT_OK) |  |
| 2 | ProtoConv_Init | 1 | “R_ProtoConv_Init” specifies the following data and calls function. - Pointer to the initialization configuration. | Initialization Function |
|  |  | 2 | After the Initialization Function “ProtoConv_Init” processes, it returns the following to the upper sub-component that calls this sub-component's interfaces. - Result as Std_ReturnType (normally be used with value E_OK or E_NOT_OK) |  |
| 3 | ProtoConv_XtoEthInit | 1 | “ProtoConv_Init” specifies the following data and calls function. - Pointer to initialization configuration | CAN/LIN/Port to Ethernet Conversion Function |
|  |  | 2 | As Protocol Conversion initialization, stores the following data to RAM. - Ethernet Conversion Function Configuration |  |
|  |  | 3 | As Protocol Conversion initialization, stores the following data to RAM. - CAN/LIN/Port to Ethernet Routing Tables configuration |  |
|  |  | 4 | After the Initialization Function “ProtoConv_XtoEthInit” processes, it returns the following to the upper sub-component that calls this sub-component's interfaces. - Result as Std_ReturnType (normally be used with value E_OK or E_NOT_OK) |  |
| 4 | ProtoConv_EthtoXInit | 1 | “ProtoConv_Init” specifies the following data and calls function. - Pointer to initialization configuration | Ethernet to CAN/LIN/Port Conversion Function |
|  |  | 2 | As Protocol Conversion initialization, stores the following data to RAM. - Ethernet to CAN/LIN/Port Routing Tables configuration |  |
|  |  | 3 | After the Initialization Function “ProtoConv_EthtoXInit” processes, it returns the following to the upper sub-component that calls this sub-component's interfaces. - Result as Std_ReturnType (normally be used with value E_OK or E_NOT_OK) |  |
|  |  | 4 | As Protocol Conversion initialization, stores the following data to RAM. - Ethernet to CAN/LIN/Port Filtering Table configuration |  |

## R_ProtoConv_CantoEthTS

Figure 8-2: Data Flow diagram (R_ProtoConv_CantoEthTS)

Table 8-3: R_ProtoConv_CantoEthTS

| API Function |  | Functionally |  | Software Sub-component Name |
| --- | --- | --- | --- | --- |
| 1 | R_ProtoConv_CantoEthTS | 1 | The User Application specifies the following data and call API. - Bus ID. - Pointer to the CAN frame structure. - Pointer to the TimeStamp. | API Layer |
|  |  | 2 | After the API “R_ProtoConv_CantoEthTS” processes, it returns the following to the User Application. - Result as Std_ReturnType (be used with value E_OK or E_NOT_OK) |  |
| 2 | ProtoConv_CantoEthTS | 1 | “R_ProtoConv_CantoEthTS” specifies the following data and calls function. - Bus ID. - Pointer to the CAN frame structure. - Pointer to the TimeStamp. | CAN frame with TimeStamp to Ethernet Conversion Function |
|  |  | 2 | Reads the following data. - CAN frame structure. |  |
|  |  | 3 | Search CAN to Ethernet Routing Table and Ethernet Destination Routing Table and get results. |  |
|  |  | 4 | Store CAN frame to CAN Rx Buffer. |  |
|  |  | 5 | Copy CAN frame from CAN Rx Buffer to Ethernet Tx Buffer. |  |
|  |  | 6 | After the Function “ProtoConv_CantoEthTS” processes, it returns the following to the upper sub-component that calls this sub-component's interfaces. - Result as Std_ReturnType (be used with value E_OK or E_NOT_OK) |  |
| 3 | ProtoConv_EthTransmitUserDef | 1 | “ProtoConv_CantoEthTS” specifies the following data and calls function. - Pointer to Ethernet frame stored in Ethernet Tx Buffer - Destination MAC Address - Ethernet frame size - EtherType | Interface Function |
|  |  | 2 | After the Function “ProtoConv_EthTransmitUserDef” processes, it returns the following to the upper sub-component that calls this sub-component's interfaces. - Result as Std_ReturnType (be used with value E_OK or E_NOT_OK) |  |

## R_ProtoConv_CantoEth

Figure 8-3: Data Flow diagram (R_ProtoConv_CantoEth)

Table 8-4: R_ProtoConv_CantoEth

| API Function |  | Functionally |  | Software Sub-component Name |
| --- | --- | --- | --- | --- |
| 1 | R_ProtoConv_CantoEth | 1 | The User Application specifies the following data and call API. - Bus ID. - Pointer to the CAN frame structure. | API Layer |
|  |  | 2 | After the API “R_ProtoConv_CantoEth” processes, it returns the following to the User Application. - Result as Std_ReturnType (be used with value E_OK or E_NOT_OK) |  |
| 2 | ProtoConv_CantoEth | 1 | “R_ProtoConv_CantoEth” specifies the following data and calls function. - Bus ID. - Pointer to the CAN frame structure. | CAN to Ethernet Conversion Function |
|  |  | 2 | Reads the following data. - CAN frame structure. |  |
|  |  | 3 | Search CAN to Ethernet Routing Table and Ethernet Destination Routing Table and get results. |  |
|  |  | 4 | Store CAN frame to CAN Rx Buffer. |  |
|  |  | 5 | Copy CAN frame from CAN Rx Buffer to Ethernet Tx Buffer. |  |
|  |  | 6 | After the Function “ProtoConv_CantoEth” processes, it returns the following to the upper sub-component that calls this sub-component's interfaces. - Result as Std_ReturnType (be used with value E_OK or E_NOT_OK) |  |
| 3 | ProtoConv_EthTransmitUserDef | 1 | “ProtoConv_CantoEth” specifies the following data and calls function. - Pointer to Ethernet frame stored in Ethernet Tx Buffer - Destination MAC Address - Ethernet frame size - EtherType | Interface Function |
|  |  | 2 | After the Function “ProtoConv_EthTransmitUserDef” processes, it returns the following to the upper sub-component that calls this sub-component's interfaces. - Result as Std_ReturnType (be used with value E_OK or E_NOT_OK) |  |

## R_ProtoConv_LintoEth

Figure 8-4: Data Flow diagram (R_ProtoConv_LintoEth)

Table 8-5: R_ProtoConv_LintoEth

| API Function |  | Functionally |  | Software Sub-component Name |
| --- | --- | --- | --- | --- |
| 1 | R_ProtoConv_LintoEth | 1 | The User Application specifies the following data and call API. - Bus ID. - Pointer to the LIN frame structure. | API Layer |
|  |  | 2 | After the API “R_ProtoConv_LintoEth” processes, it returns the following to the User Application. - Result as Std_ReturnType (be used with value E_OK or E_NOT_OK) |  |
| 2 | ProtoConv_LintoEth | 1 | “R_ProtoConv_LintoEth” specifies the following data and calls function. - Bus ID. - Pointer to the LIN frame structure. | LIN to Ethernet Conversion Function |
|  |  | 2 | Reads the following data. - LIN frame structure. |  |
|  |  | 3 | Search LIN to Ethernet Routing Table and Ethernet Destination Routing Table and get results. |  |
|  |  | 4 | Store LIN frame to LIN Rx Buffer. |  |
|  |  | 5 | Copy LIN frame from LIN Rx Buffer to Ethernet Tx Buffer. |  |
|  |  | 6 | After the Function “ ProtoConv_LintoEth” processes, it returns the following to the upper sub-component that calls this sub-component's interfaces. - Result as Std_ReturnType (be used with value E_OK or E_NOT_OK) |  |
| 3 | ProtoConv_EthTransmitUserDef | 1 | “ ProtoConv_LintoEth” specifies the following data and calls function. - Pointer to Ethernet frame stored in Ethernet Tx Buffer - Destination MAC Address - Ethernet frame size - EtherType | Interface Function |
|  |  | 2 | After the Function “ProtoConv_EthTransmitUserDef” processes, it returns the following to the upper sub-component that calls this sub-component's interfaces. - Result as Std_ReturnType (be used with value E_OK or E_NOT_OK) |  |

## R_ProtoConv_PorttoEth

Figure 8-5: Data Flow diagram (R_ProtoConv_PorttoEth)

Table 8-6: R_ProtoConv_PorttoEth

| API Function |  | Functionally |  | Software Sub-component Name |
| --- | --- | --- | --- | --- |
| 1 | R_ProtoConv_PorttoEth | 1 | The User Application specifies the following data and call API. - Data Length. - Pointer to the Port Data. - Index to Ethernet Destination Routing Table. | API Layer |
|  |  | 2 | After the API “ R_ProtoConv_PorttoEth” processes, it returns the following to the User Application. - Result as Std_ReturnType (be used with value E_OK or E_NOT_OK) |  |
| 2 | ProtoConv_PorttoEth | 1 | “R_ProtoConv_PorttoEth” specifies the following data and calls function. - Data Length. - Pointer to the Port Data. - Index to Ethernet Destination Routing Table. | Port to Ethernet Conversion Function |
|  |  | 2 | Reads the following data. - Parallel Port frame structure. |  |
|  |  | 3 | Use Index to Ethernet Destination Routing Table and get results. |  |
|  |  | 4 | Store Parallel Port frame to Port Rx Buffer. |  |
|  |  | 5 | Copy Parallel Port frame from Port Rx Buffer to Ethernet Tx Buffer. |  |
|  |  | 6 | After the Function “ ProtoConv_PorttoEth” processes, it returns the following to the upper sub-component that calls this sub-component's interfaces. - Result as Std_ReturnType (be used with value E_OK or E_NOT_OK) |  |
| 3 | ProtoConv_EthTransmitUserDef | 1 | “ ProtoConv_PorttoEth” specifies the following data and calls function. - Pointer to Ethernet frame stored in Ethernet Tx Buffer - Destination MAC Address - Ethernet frame size - EtherType | Interface Function |
|  |  | 2 | After the Function “ProtoConv_EthTransmitUserDef” processes, it returns the following to the upper sub-component that calls this sub-component's interfaces. - Result as Std_ReturnType (be used with value E_OK or E_NOT_OK) |  |

## R_ProtoConv_TriggerXtoEthTx

Figure 8-6: Data Flow diagram (R_ProtoConv_TriggerXtoEthTx)

Table 8-7: R_ProtoConv_TriggerXtoEthTx

| API Function |  | Functionally |  | Software Sub-component Name |
| --- | --- | --- | --- | --- |
| 1 | R_ProtoConv_TriggerXtoEthTx | 1 | The User Application specifies the following data and call API. - Target Buffer Index | API Layer |
|  |  | 2 | After the API “R_ProtoConv_TriggerXtoEthTx” processes, it returns the following to the User Application. - Result as Std_ReturnType (be used with value E_OK or E_NOT_OK) |  |
| 2 | ProtoConv_TriggerXtoEthTx | 1 | Copy CAN/LIN/Port frame from Mix Rx Buffer. | CAN/LIN/Port to Ethernet Conversion Function |
|  |  | 2 | Copy Ethernet frame to Ethernet Tx Buffer |  |
| 3 | ProtoConv_EthTransmitUserDef | 1 | “ R_ProtoConv_TriggerXtoEthTx” specifies the data and calls function. - Pointer to Ethernet frame stored in Ethernet Tx Buffer - Destination MAC Address - Ethernet frame size - EtherType | Interface Function |
|  |  | 2 | After the Function “ProtoConv_EthTransmitUserDef” processes, it returns the following to the upper sub-component that calls this sub-component's interfaces. - Result as Std_ReturnType (be used with value E_OK or E_NOT_OK) |  |

## R_ProtoConv_Mainfunction

Figure 8-7: Data Flow diagram (R_ProtoConv_MainFunction)

Table 8-8: R_ProtoConv_MainFunction

| API Function |  | Functionally |  | Software Sub-component Name |
| --- | --- | --- | --- | --- |
| 1 | R_ProtoConv_MainFunction | - | This function have no input and output because input and output are void type. | API Layer |
| 2 | ProtoConv_MixtoEthMain | 1 | Copy CAN/LIN/Parallel Port frame from Mix Rx Buffer. | CAN/LIN/Port to Ethernet Conversion Function |
|  |  | 2 | Copy Ethernet frame to Ethernet Tx Buffer |  |
| 3 | ProtoConv_EthTransmitUserDef | 1 | “ProtoConv_MixtoEthMain” specifies the following data and calls function. - Pointer to Ethernet frame stored in Ethernet Tx Buffer - Destination MAC Address - Ethernet frame size - EtherType | Interface Function |
|  |  | 2 | After the Function “ProtoConv_EthTransmitUserDef” processes, it returns the following to the upper sub-component that calls this sub-component's interfaces. - Result as error code |  |

## R_ProtoConv_EthtoX

Figure 8-8: Data Flow diagram (R_ProtoConv_EthtoX)

Table 8-9: R_ProtoConv_EthtoX

| API Function |  | Functionally |  | Software Sub-component Name |
| --- | --- | --- | --- | --- |
| 1 | R_ProtoConv_EthtoX | 1 | The User Application specifies the following data and call API. - Pointer to the Ethernet frame structure. | API Layer |
|  |  | 2 | After the API “R_ProtoConv_EthtoX” processes, it returns the following to the User Application. - Result as Std_ReturnType (normally be used with value E_OK or E_NOT_OK) and packet transmission result |  |
| 2 | ProtoConv_EthtoX | 1 | “ R_ProtoConv_EthtoX” specifies the following data and calls function. - Pointer to the Ethernet frame structure. - Pointer to the Buffer Information structure. | Ethernet to CAN/LIN/Port Conversion Function |
|  |  | 2 | Store Ethernet frame to Ethernet Rx Buffer. |  |
|  |  | 3 | Read CAN/LIN/Parallel Port frame from Ethernet frame in Ethernet Rx Buffer and get results. |  |
|  |  | 4 | For CAN/LIN, search Ethernet to CAN/LIN Routing Table and get results. For Port, use the fixed value in configuration. |  |
|  |  | 5 | After the Function “ProtoConv_EthtoX” processes, it returns the following to the upper sub-component that calls this sub-component's interfaces. - Result as Std_ReturnType (normally be used with value E_OK or E_NOT_OK) and packet transmission result |  |
|  |  | 6 | Search Ethernet to CAN/LIN/Port Filtering Table and get results. |  |
| 3 | ProtoConv_CanTransmitTSUserDef | 1 | “ProtoConv_EthtoX” specifies the following data and calls function. - Pointer to CAN frame stored in Ethernet Rx Buffer - CAN frame size (DLC) - CAN ID - Destination Bus ID - Pointer to TimeStamp - Additional Information | Interface Function |
|  |  | 2 | After the Function “ProtoConv_CanTransmitTSUserDef” processes, it returns the following to the upper sub-component that calls this sub-component's interfaces. - Result as error code |  |
| 4 | ProtoConv_CanTransmitUserDef | 1 | “ProtoConv_EthtoX” specifies the following data and calls function. - Pointer to CAN frame stored in Ethernet Rx Buffer - CAN frame size (DLC) - CAN ID - Destination Bus ID - Additional Information |  |
|  |  | 2 | After the Function “ProtoConv_CanTransmitUserDef” processes, it returns the following to the upper sub-component that calls this sub-component's interfaces. - Result as error code |  |
| 5 | ProtoConv_CanNotifyTSUserDef | 1 | “ProtoConv_EthtoX” specifies the following data and calls function. - Pointer to CAN frame stored in Ethernet Rx Buffer - CAN frame size (DLC) - CAN ID - Source Bus ID - Pointer to TimeStamp |  |
|  |  | 2 | After the Function “ProtoConv_CanNotifyTSUserDef” processes, it returns the following to the upper sub-component that calls this sub-component's interfaces. - Result as error code |  |
| 6 | ProtoConv_CanNotifyUserDef | 1 | “ProtoConv_EthtoX” specifies the following data and calls function. - Pointer to CAN frame stored in Ethernet Rx Buffer - CAN frame size (DLC) - CAN ID - Source Bus ID |  |
|  |  | 2 | After the Function “ProtoConv_CanNotifyUserDef” processes, it returns the following to the upper sub-component that calls this sub-component's interfaces. - Result as error code |  |
| 7 | ProtoConv_LinTransmitUserDef | 1 | “ProtoConv_EthtoX” specifies the following data and calls function. - Pointer to LIN frame stored in Ethernet Rx Buffer - LIN frame size (DLC) - LIN ID - Destination Bus ID - Additional Information |  |
|  |  | 2 | After the Function “ProtoConv_LinTransmitUserDef” processes, it returns the following to the upper sub-component that calls this sub-component's interfaces. - Result as error code |  |
| 8 | ProtoConv_LinNotifyUserDef | 1 | “ProtoConv_EthtoX” specifies the following data and calls function. - Pointer to LIN frame stored in Ethernet Rx Buffer - LIN frame size (DLC) - LIN ID - Source Bus ID |  |
|  |  | 2 | After the Function “ProtoConv_LinNotifyUserDef” processes, it returns the following to the upper sub-component that calls this sub-component's interfaces. - Result as error code |  |
| 9 | ProtoConv_PortTransmitUserDef | 1 | “ProtoConv_EthtoX” specifies the following data and calls function. - Pointer to Port Data - Data Length |  |
|  |  | 2 | After the Function “ProtoConv_PortTransmitUserDef” processes, it returns the following to the upper sub-component that calls this sub-component's interfaces. - Result as error code |  |

## R_ProtoConv_GetSendStatus

Figure 8-9: Data Flow diagram (R_ProtoConv_GetSendStatus)

Table 8-10: R_ProtoConv_GetSendStatus

| API Function |  | Functionally |  | Software Sub-component Name |
| --- | --- | --- | --- | --- |
| 1 | R_ProtoConv_GetSendStatus | 1 | The User Application specifies the following data and call API. - Pointer to send status | API Layer |
|  |  | 2 | After the API “R_ProtoConv_GetSendStatus” processes, it returns the following to the User Application. - Result as Std_ReturnType (normally be used with value E_OK or E_NOT_OK) - CAN send status |  |
| 2 | ProtoConv_GetSendStatus | 1 | After the API “ ProtoConv_GetSendStatus” processes, it returns the following to the User Application. - Get CAN send status | Ethernet to CAN Conversion Function |

## R_ProtoConv_UpdateSendStatus

Figure 8-10: Data Flow diagram (R_ProtoConv_UpdateSendStatus)

Table 8-11: R_ProtoConv_UpdateSendStatus

| API Function |  | Functionally |  | Software Sub-component Name |
| --- | --- | --- | --- | --- |
| 1 | R_ProtoConv_UpdateSendStatus | 1 | The User Application specifies the following data and call API. - Send Status | API Layer |
|  |  | 2 | After the API “R_ProtoConv_UpdateSendStatus” processes, it returns the following to the User Application. - Result as Std_ReturnType (normally be used with value E_OK or E_NOT_OK) |  |
| 2 | ProtoConv_UpdateSendStatus | 1 | After the API “ProtoConv_UpdateSendStatus” processes, it returns the following to the User Application. - Send Status | Ethernet to CAN Conversion Function |

## R_ProtoConv_EnqueueCan

Figure 8-11: Data Flow diagram (R_ProtoConv_EnqueueCan)

Table 8-12: R_ProtoConv_EnqueueCan

| API Function |  | Functionally |  | Software Sub-component Name |
| --- | --- | --- | --- | --- |
| 1 | R_ProtoConv_EnqueueCan | 1 | The User Application specifies the following data and call API. - Bus ID. - Pointer to the CAN frame structure. - Pointer to the Timestamp | API Layer |
|  |  | 2 | After the API “ R_ProtoConv_EnqueueCan” processes, it returns the following to the User Application. - Result as Std_ReturnType (be used with value E_OK or E_NOT_OK) |  |
| 2 | ProtoConv_EnqueueCan | 1 | “R_ProtoConv_EnqueueCan” specifies the following data and calls function. - Bus ID. - Pointer to the CAN frame structure. - Pointer to the Timestamp | CAN to Ethernet Conversion Function |
|  |  | 2 | After the Function “ProtoConv_EnqueueCan” processes, it returns the following to the upper sub-component that calls this sub-component's interfaces. - Result as Std_ReturnType (be used with value E_OK or E_NOT_OK) |  |

## R_ProtoConv_EnqueueLin

Figure 8-12: Data Flow diagram (R_ProtoConv_EnqueueLin)

Table 8-13: R_ProtoConv_EnqueueLin

| API Function |  | Functionally |  | Software Sub-component Name |
| --- | --- | --- | --- | --- |
| 1 | R_ProtoConv_EnqueueLin | 1 | The User Application specifies the following data and call API. - Bus ID. - Pointer to the LIN frame structure. | API Layer |
|  |  | 2 | After the API “ R_ProtoConv_EnqueueLin” processes, it returns the following to the User Application. - Result as Std_ReturnType (be used with value E_OK or E_NOT_OK) |  |
| 2 | ProtoConv_EnqueueLin | 1 | “R_ProtoConv_EnqueueLin” specifies the following data and calls function. - Bus ID. - Pointer to the LIN frame structure. | LIN to Ethernet Conversion Function |
|  |  | 2 | After the Function “ProtoConv_EnqueueLin” processes, it returns the following to the upper sub-component that calls this sub-component's interfaces. - Result as Std_ReturnType (be used with value E_OK or E_NOT_OK) |  |

## R_ProtoConv_EnqueuePort

Figure 8-13: Data Flow diagram (R_ProtoConv_EnqueuePort)

Table 8-14: R_ProtoConv_EnqueuePort

| API Function |  | Functionally |  | Software Sub-component Name |
| --- | --- | --- | --- | --- |
| 1 | R_ProtoConv_EnqueuePort | 1 | The User Application specifies the following data and call API. - Data Length. - Pointer to the Port Data. - Index to Ethernet Destination Routing Table. | API Layer |
|  |  | 2 | After the API “ R_ProtoConv_EnqueuePort” processes, it returns the following to the User Application. - Result as Std_ReturnType (be used with value E_OK or E_NOT_OK) |  |
| 2 | ProtoConv_EnqueuePort | 1 | “R_ProtoConv_EnqueuePort” specifies the following data and calls function. - Data Length. - Pointer to the Port Data. - Index to Ethernet Destination Routing Table. | Port to Ethernet Conversion Function |
|  |  | 2 | After the Function “ ProtoConv_EnqueuePort” processes, it returns the following to the upper sub-component that calls this sub-component's interfaces. - Result as Std_ReturnType (be used with value E_OK or E_NOT_OK) |  |

## R_ProtoConv_EnqueueEth

Figure 8-14: Data Flow diagram (R_ProtoConv_EnqueueEth)

Table 8-15: R_ProtoConv_EnqueueEth

| API Function |  | Functionally |  | Software Sub-component Name |
| --- | --- | --- | --- | --- |
| 1 | R_ProtoConv_EnqueueEth | 1 | The User Application specifies the following data and call API. - Frame Type - Source MAC Address - Pointer to Ethernet frame - Frame Length | API Layer |
|  |  | 2 | After the API “ R_ProtoConv_EnqueueEth” processes, it returns the following to the User Application. - Result as Std_ReturnType (be used with value E_OK or E_NOT_OK) |  |
| 2 | ProtoConv_EnqueueEth | 1 | “R_ProtoConv_EnqueueEth” specifies the following data and calls function. - Frame Type - Source MAC Address - Pointer to Ethernet frame - Frame Length | Ethernet to X Conversion Function |
|  |  | 2 | After the Function “ ProtoConv_EnqueueEth” processes, it returns the following to the upper sub-component that calls this sub-component's interfaces. - Result as Std_ReturnType (be used with value E_OK or E_NOT_OK) |  |

## R_ProtoConv_DequeueEth

Figure 8-15: Data Flow diagram (R_ProtoConv_DequeueEth)

Table 8-16: R_ProtoConv_EnqueueEth

| API Function |  | Functionally |  | Software Sub-component Name |
| --- | --- | --- | --- | --- |
| 1 | R_ProtoConv_DequeueEth | 1 | The User Application specifies the following data and call API. - Pointer to Frame Type - Pointer to Source MAC Address - Pointer to Pointer to Ethernet frame - Pointer to Frame Length | API Layer |
|  |  | 2 | After the API “ R_ProtoConv_DequeueEth” processes, it returns the following to the User Application. - Result as Std_ReturnType (be used with value E_OK or E_NOT_OK) |  |
| 2 | ProtoConv_DequeueEth | 1 | “R_ProtoConv_DequeueEth” specifies the following data and calls function. - Pointer to Frame Type - Pointer to Source MAC Address - Pointer to Pointer to Ethernet frame - Pointer to Frame Length | Ethernet to X Conversion Function |
|  |  | 2 | After the Function “ ProtoConv_DequeueEth” processes, it returns the following to the upper sub-component that calls this sub-component's interfaces. - Result as Std_ReturnType (be used with value E_OK or E_NOT_OK) |  |

## R_ProtoConv_TriggerQueueDataXtoEth

Figure 8-16: Data Flow diagram (R_ProtoConv_TriggerQueueDataXtoEth)

Table 8-17: R_ProtoConv_TriggerQueueDataXtoEth

| API Function |  | Functionally |  | Software Sub-component Name |
| --- | --- | --- | --- | --- |
| 1 | R_ProtoConv_TriggerQueueDataXtoEth | 1 | This function have no input because input is void type. After the API “R_ProtoConv_TriggerQueueDataXtoEth” processes, it returns the following to the User Application. - Result as Std_ReturnType (be used with value E_OK or E_NOT_OK) | API Layer |
| 2 | ProtoConv_TriggerQueueDataXtoEth | 1 | Copy CAN frame from CAN FIFO Memory. | CAN/LIN/Port to Ethernet Conversion Function |
|  |  | 2 | Copy LIN frame from LIN FIFO Memory. |  |
|  |  | 3 | Copy Port frame from Port FIFO Memory. |  |
|  |  | 4 | Copy Ethernet frame to Ethernet Tx Buffer |  |
| 3 | ProtoConv_EthTransmitUserDef | 1 | “ R_ProtoConv_TriggerQueueDataXtoEth specifies the data and calls function. - Pointer to Ethernet frame stored in Ethernet Tx Buffer - Destination MAC Address - Ethernet frame size - EtherType | Interface Function |
|  |  | 2 | After the Function “ProtoConv_EthTransmitUserDef” processes, it returns the following to the upper sub-component that calls this sub-component's interfaces. - Result as Std_ReturnType (be used with value E_OK or E_NOT_OK) |  |

# Resource Usage

## Hardware Function List

This software does not depend on HW.

## Code Size

Code: 13KB (Gen5 Target)

[iso26-18_p6_7.4.13] b

## Stack Size

[Gen5GW_ProtoConv_AD_C_09_001]

[iso26-18_p6_7.4.13] b

Stack Size: 2324 Bytes (Gen5 Target)

[Covers: req-8552172-00905239]

## Execution Time

[Gen5GW_ProtoConv_AD_C_09_002]

CAN/LIN/Port -> Ethernet conversion and Ethernet -> CAN/LIN/Port conversion are completed by 100us.

Target Performance (Throughput) of Protocol Conversion:

40Mbps (5Mbps x 16ch x 50%)

[Covers: req-8552172-00902552]

## Memory Map

[iso26-18_p6_7.4.13] b

Assumed to be placed in the RT-VRAM area

### RAM

[Gen5GW_ProtoConv_AD_C_09_003]

RAM: 53.3 KB (Gen5 Target)

[Covers: req-8552172-00905239]

[Covers: req-8552172-00527979]

### ROM

[Gen5GW_ProtoConv_AD_C_09_004]

ROM: 17.3 KB (Gen5 Target)

[Covers: req-8552172-00905239]

[Covers: req-8552172-00527979]

# Uncovered List

[Gen5GW_ProtoConv_AD_C_00_000]

| Document name | Uncovered IDs | Reason |
| --- | --- | --- |
| ProtoConv Conversion Requirement Specification | [Covers: req-8552172-00909477] | ProtoConv shall support the GHS compiler, similar with U2Ax, U2Bx-E, U2Cx, U5x MCAL modules. |
|  | [Covers: req-8552172-00909475] | ProtoConv shall comply with AUTOSAR R22-11, similar with U2Ax, U2Bx-E, U2Cx and AUTOSAR R23-11 U5x MCAL modules. |
|  | [Covers: req-8552172-00905240] | Generation Tool of ProtoConv uses ARXML file to generate C Source code. ProtoConv provides BSWMDT file and PDF file. |
|  | [Covers: req-8552172-00905253] | ProtoConv occupies one core and performs the conversion process. |
|  | [Covers: req-8552172-00945698] | ProtoConv operates with 4 RH850/R-Car MCAL drivers: CAN, ETH, LIN, PORT. |
| Common Requirement Specification | [Covers: req-8552172-00527969] | This is information of module ID only. |
|  | [Covers: req-8552172-00527981] | ProtoConv does not have interrupt functions. |
|  | [Covers: req-8552172-00527991] | ProtoConv does not have Link time configuration. |
|  | [Covers: req-8552172-00527984] | ProtoConv does not have callback functions. |
|  | [Covers: req-8552172-00528000] | The requirement is for general design guideline. |
|  | [Covers: req-8552172-00528001] | ProtoConv can directly access to microcontroller resources (e.g. a hardware timer) or use the MCAL(e.g. CAN and ETH). |

Version History

| Date | Version | Change | Approver | Checker | Author |
| --- | --- | --- | --- | --- | --- |
| February 28, 2025 | 1.00 | Initial Version | H.Shigehisa | T.Toyoshima | Huy Huynh |
| June 15, 2025 | 1.01 | - Table 2-79: Update PROTOCONV_MIXBUF_MAX value to XTOETH_DEST_TABLE_ENTRY_NUM and add definition of PROTOCONV_VENDOR_ID, PROTOCONV_MODULE_ID, PROTOCONV_INSTANCE_ID, PROTOCONV_AR_RELEASE_MAJOR_VERSION, PROTOCONV_AR_RELEASE_MINOR_VERSION, PROTOCONV_AR_RELEASE_REVISION_VERSION, PROTOCONV_SW_MAJOR_VERSION, PROTOCONV_SW_MINOR_VERSION, PROTOCONV_SW_PATCH_VERSION, PROTOCONV_PBTYPES_AR_RELEASE_MAJOR_VERSION, PROTOCONV_PBTYPES_AR_RELEASE_MINOR_VERSION, PROTOCONV_PBTYPES_AR_RELEASE_REVISION_VERSION, PROTOCONV_PBTYPES_SW_MAJOR_VERSION, PROTOCONV_PBTYPES_SW_MINOR_VERSION - Figure 4-1, Table 4-2, and Table 4-3: remove the "common" suffix from the file names. - Table 2-35: Update description for the return value of ProtoConv_TriggerQueueDataXtoEth() - Table 3-2: Update description of ProtoConv_Init to add execution of ProtoConv_FifoInit() and update requirements mapping - Add the new section 3.2.2.14 for new function ProtoConv_FifoInit() - Table 3-1: Remove ProtoConv_UtilCheckBufRangeOverlap from Function List - Remove 3.4.2.11.ProtoConv_UtilCheckBufRangeOverlap - Table 2-74 to 2-78: Update location of R_ProtoConv_CanFifoType, R_ProtoConv_LinFifoType, R_ProtoConv_PortFifoType, R_ProtoConv_EthFifoType to CDD_ProtoConv_Fifo.h - Table 3-35: Add range check of LusBitWidth in ProtoConv_UnpackPort | Luan Le | Toan Pham | Huy Huynh |
| June 20, 2025 | 1.02 | - Table 2-33: Add range check for frame length in R_ProtoConv_EnqueueEth - Table 3-35: Update description of ProtoConv_UnpackPort for payload position after unpacking ACF Parallel message | Luan Le | Toan Pham | Huy Huynh |
| June 26, 2025 | 1.03 | - Table 2-20, 2-21, 2-22, 2-23, 2-24, 2-27, 2-47, 2-80, 3-3, 3-15, 3-19, 3-37: Remove redundant DET error code - Table 2-29: Add new DET error in R_ProtoConv_UpdateSendStatus - Table 2-50: Change aaCantoEthMainTblGeneral[PROTOCONV_CAN_CH_NUM] to pCantoEthMainTblGeneral and aaLintoEthMainTblGeneral[PROTOCONV_LIN_CH_NUM] to pLintoEthMainTblGeneral - Table 2-66: Add 9th element R_ProtoConv_EthFormatType enFormatType into R_ProtoConv_HeaderType struct type - Table 2-79: Update PROTOCONV_SW_MINOR_VERSION from 2U to 3U; remove PROTOCONV_CAN_CH_NUM and PROTOCONV_LIN_CH_NUM - Table 2-80: Change value of PROTOCONV_E_PARAM_CONFIG from 0x17 to 0x12, PROTOCONV_E_FIFO_FULL from 0x1E to 0x13, PROTOCONV_E_FIFO_EMPTY from 0x1F to 0x14 - Table 3-2, 3-5, 3-6, 3-7, 3-8, 3-9, 3-17, 3-21, 3-22: Update description and remove redundant DET error code of ProtoConv_Init, ProtoConv_CantoEthTS, ProtoConv_LintoEth, ProtoConv_PorttoEth, ProtoConv_StorePdutoEthBufData, ProtoConv_EthtoXInit, ProtoConv_EthtoX - Table 3-20, 3-36: Remove unused functions - Remove ProtoConv_SetCanRoutTbl, ProtoConv_SetLinRoutTbl, ProtoConv_UtilConvMacAddr32Bit, ProtoConv_UtilCheckBufAddrRange | Luan Le | Toan Pham | Huy Huynh |
| October 7, 2025 | 1.04 | Update Uncovered list to add attribution for U2Ax, U2Bx-E devices | Luan Le | Toan Pham | Huy Huynh |
| Nov 22, 2025 | 1.05 | Table 2-79: Update PROTOCONV_SW_MAJOR_VERSION from 0 to 1 and PROTOCONV_SW_MINOR_VERSION from 3 to 0 for U2Bx-E/U2Ax Release | Luan Le | Toan Pham | Huy Huynh |
| Feb 04, 2026 | 1.06 | - Add figure 1.2 - Add reference [8] - Section 2.7.1.1: Remove table Buffer address range and its description | Luan Le | Toan Pham | Huy Huynh |
| Aug 06 | 1.07 | Update Reference [1] to MCAL Software AD Specification Add Environment and Limitations for development in section 1.5 Table 3-16: Update description ProtoConv_CreateAndSendEthFrame to change the sequence from zero-padding then copying data to copying data then zero-padding |  |  | Huy Huynh |

Template History

| Date | Version | Change | Approver | Checker | Author |
| --- | --- | --- | --- | --- | --- |
| January 31, 2020 | 1.00 | Newly created | Y.Inoue | Y.Inoue | T.Sakaguchi |
| December 24, 2020 | 1.01 | Re-embedded all ID of ISO requirement by adding their information into corresponding sections (set as non-printing characters in Word). | Y.Inoue | H.Nano/Wei Liu | Duy Do |